| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to MCS Table for CQI (15.3.13.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Sung-Eun Park, David Mazzarese, Jerry Pi, Hokyu Choi Samsung Electronics Co., Ltd. | se.park@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D1 Section: 15.3.13 Link Adaptation |  |
| Abstract | The contribution proposes the text changes to the MCS table for CQI of the 802.16m draft amendment, IEEE P802.16m/D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to MCS Table for CQI (15.3.13.1)

Sung-Eun Park, David Mazzarese, Jerry Pi, Hokyu Choi
Samsung Electronics Co., Ltd.

- Introduction

The MCS level used for CQI reporting depends on the burst size or resource allocation. There is a quite large difference in the SNR required to decode a burst when the range of the burst size is large. Therefore it is preferra to specify the burst size (or resource allocation size) and the target error rate as reference parameters that were considered when designing the MCS table for CQI adopted in [1].

- References

[1] IEEE P802.16m/D1, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” July 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify section 15.3.13.1 DL Link Adaptation as follows >

15.3.13.1 DL Link Adaptation

The serving ABS may adapt the modulation and coding scheme (MCS) level based on the channel quality indicator (CQI) and/or HARQ ACK/NACK reported by the AMS.

The serving ABS shall adapt the MIMO mode, according to CQI reports from the AMS and considering system parameters, such as: traffic load, number of users, ACK/NACK, CQI variation, preferred MFM etc.

AMS shall measure the DL channel quality and report back to ABS. The exact measurement method used to derive the CQI feedback is implementation specific.

The nominal MCS for CQI feedback shall be selected from Table 772. MCS index should be selected at the MS assuming 4 LRUs as a resource allocation, and 10% as a target error rate for the first HARQ transmission.

Table 772─MCS table for CQI

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| 0000 | QPSK | 31/256 |
| 0001 | QPSK | 48/256 |
| 0010 | QPSK | 71/256 |
| 0011 | QPSK | 101/256 |
| 0100 | QPSK | 135/256 |
| 0101 | QPSK | 171/256 |
| 0110 | 16QAM | 102/256 |
| 0111 | 16QAM | 128/256 |
| 1000 | 16QAM | 155/256 |
| 1001 | 16QAM | 184/256 |
| 1010 | 64QAM | 135/256 |
| 1011 | 64QAM | 157/256 |
| 1100 | 64QAM | 181/256 |
| 1101 | 64QAM | 205/256 |
| 1110 | 64QAM | 225/256 |
| 1111 | 64QAM | 237/256 |

---------------------------------------------------  Text End  ---------------------------------------------------