| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Assignment A-MAP (15.3.6.2.2.2) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Lei.Huang@sg.panasonic.com Yoshii.Isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30: P902.16m/D1 |  |
| Abstract | This document contains the suggested clean-up modification on Section 15.3.6.2.2.2 Assignment A-MAP |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Changes to Assignment A-MAP (15.3.6.2.2.2)

- Lei Huang

- Panasonic Singapore Laboratories

- Isamu Yoshii

- Panasonic Corporation

- Introduction

This contribution proposed some modifications on Section 15.3.6.2.2.2 Assignment A-MAP in [1]. The suggested modifications are mainly regarding the grouping principle of assignment A-MAPs, since grouping of assignment A-MAPs in case of FFR is not clearly described in [1].

- Reference

- IEEE P802.16m/D1, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, July 2009

- Suggested text modification

- Editorial notes

- Current text in IEEE P802.16m/D1

Remove text from IEEE P802.16m/D1

- Add new text to IEEE P802.16m/D1

------------------------------------------------------------Start of the Text--------------------------------------------------------

- 15.3.6.2.2.2 Assignment A-MAP

Assignment A-MAP contains resource assignment information which is categorized into multiple types of resource assignment IEs (assignment A-MAP IE). Each assignment A-MAP IE is coded separately and carries information for one or a group of users.

The size of the assignment A-MAP is indicated by non-user-specific A-MAP.

The minimum logical resource unit in the assignment A-MAP is called MLRU, consisting of NMLRU = 56 data tones.

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region. The number of logically contiguous MLRUs is determined based on the assignment A-MAP IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

Assignment A-MAPs are grouped together based on frequency partition, channel coding rate and A-MAP IE sizes. Assignment A-MAPs in the same group are transmitted in the same frequency partition with the same channel coding rate and contain the same A-MAP IE size. Each assignment A-MAP group contains several logically contiguous MLRUs. The number of assignment A-MAPs in each assignment A-MAP group is signaled through non-user specific A-MAP.

All the multicast A-AMAP IEs, i.e., all the A-AMAP IEs intended for more than a single AMS, present in any A-AMAP group, shall occupy a contiguous set of MLRUs starting from the beginning of the region in which the A-AMAP group is transmitted. The DL/UL Group Resource Allocation A-MAP IE, the DL/UL Group Configuration A-MAP IE and the DL/UL Composite Persistent A-MAP IE are examples of multicast A-AMAP IEs.

-------------------------------------------------------------End of the Text--------------------------------------------------------