| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Long TTI operation for TDD frame structure related to HARQ timing (15.3.9.2.2.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Kenichi KURI, Isamu YOSHII, Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kuri.kenichi@jp.pansonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30: P802.16m/D1 |  |
| Abstract | This contribution proposed clean-up modification on P802.16m/D1 section 15.3.9.2.2.1. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Long TTI operation for TDD frame structure related to HARQ timing
(15.3.9.2.2.1)

Kenichi KURI, Isamu YOSHII,

Panasonic Corporation

# Introduction

This contribution includes some proposed modifications on section 15.3.9.2.2.1 HARQ timing on TDD frame structure and related A-MAP IE description/notes in [1]. These are related to Long TTI operation for TDD frame structure.

# Current status and proposed long TTI operation for TDD

This contribution includes two proposed modifications and one issue which need to study further as follows.

(1) Clarification on description/notes of Long TTI indicator (or Long TTI length) in A-MAP IE

(2) Modification on Table 725 in P802.16m/D1 in case of DL/UL ratio = 6:2

(3) HARQ timing issue in case of DL/UL ratio = 8:0

## Clarification on description/notes of Long TTI indicator

According to P802.16m/D1, 8 kinds of A-MAP IE include syntax of ‘Long TTI indicator (or Long TTI length)’. Current description is that “0b1 : 4DL subframes for FDD or all DL(or UL) subframes for TDD”. It’s unclear expression related to TDD. If Long TTI indication is mapped to 2nd DL subframe, all DL subframes can be read remaining 4 DL subframes in i-th frame as Figure 1 (i.e. 4=D - 1, in case of l=1). In this example, Tproc is 3.

Figure 1 Example of TDD DL HARQ timing related to long TTI transmission (l=1)

However the constraint condition of long TTI transmission is defined that the length of allocated DL subframe is D. Therefore example in Figure 2 does not satisfy its condition. We proposed that the description/notes of Long TTI are defined as follows. Corresponding HARQ timing relations of Long TTI indication at non-1st DL subframe are described in Figure 2.

0b1: 4DL subframes for FDD or D (or U) subframes for TDD [ D (or U) depends on TDD subframe configuration. ]

Figure 2 TDD DL HARQ timing related to long TTI transmission (l=1~4)

## Modification on Table 725 in case of DL/UL ratio = 6:2

In case of DL/UL ratio = 6:2 is assumed, HARQ timing of Long TTI transmission corresponding to DL basic assignment A-MAP in 5-th subframe is described like Figure 3.

While processing time (Tproc) supports 2 or 3. Tproc=2 can satisfy HARQ timing relation based on Table 725 in P802.16m/D1. However Tproc=3 can’t satisfy HARQ timing relation. We proposed modification that frame index of HARQ feedback in UL is j=i+zmod4 or j=i+z+1mod4 for l=D-1, D:U=6:2, Tproc=3 as Figure 4 and Table 1.

Figure 3 TDD DL HARQ timing related to long TTI transmission (l=5) under D:U=6:2

Figure 4 TDD HARQ timing related to long TTI transmission (l=5) under D:U=6:2 and Tproc=3

Table 1 TDD DL HARQ timing

| Content | Subframe Index | Frame Index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m = l | i |
| HARQ feedback in UL | For D > U n=0,m−K,U−1,forforfor0≤m<KK≤m<U+KU+K≤m<D | j=i+zmod4 or j=(i+z+1)mod4 (for l=D – 1, D:U=6:2 and Tproc=3) |
|  | For D≤U, n=m−K |  |

## HARQ timing issue in case of DL/UL ratio = 8:0

In section 15.3.3.2.2 TDD frame structure, TDD subframe configuration supports DL/UL ratio = 8:0. The relationship of HARQ timing on TDD frame structure is defined in Table 725 and 726. In terms of DL/UL ratio = 8:0, corresponding relationship has not been defined in these table. Additionally HARQ feedback in UL is not possible to be transmitted because of no UL subframe under DL/UL ratio = 8:0.

# References

- IEEE P802.16m/D1, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, July 2009

# Proposed text modification on P802.16m/D1

We propose to modify the description on Long TTI indicator as follows for P802.16m/D1 Section 15.3.9.2.2.2.1 Downlink.

============================== Start of Proposed Text =================================

- Proposal#1

- 8 kinds of A-MAP IE include syntax of ‘Long TTI indicator (or Long TTI length)’.
- DL (or UL) basic assignment A-MAP IE
- DL (or UL) individual persistent A-MAP IE
- DL (or UL) composite persistent A-MAP IE
- DL (or UL) group resource allocation A-MAP IE

Description/Notes of ‘Long TTI indicator (or Long TTI length)’ is modified as follows.

- 0b1: 4DL subframes for FDD or all DL (or UL) subframes for TDD D (or U) subframes for TDD [ D (or U) depends on TDD subframe configuration. ]

- 15.3.6.5.4.2 DL basic assignment A-MAP IE

Table 679 DL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP(){ |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| } |  |  |

- 15.3.6.5.4.3 UL basic assignment A-MAP IE

Table 670 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP(){ |  |  |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| } |  |  |

- 15.3.6.5.4.4 Group resource allocation A-MAP IE

Table 671 DL group resource allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL/UL subframes for TDD |
| … | … | … |
| } |  |  |

Table 672 UL group resource allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL/UL subframes for TDD |
| … | … | … |
| } |  |  |

- 15.3.6.5.4.6 DL PA A-MAP IE

Table 676 DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| If (Allocation Period==0b00){ |  |  |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| } else if (Allocation Period != 0b00){ |  |  |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| } |  |  |

Table 677 DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or D subframes for TDD [ D depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| } |  |  |

- 15.3.6.5.4.7 UL PA A-MAP IE

Table 679 UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| If (Allocation Period==0b00){ |  |  |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| } else if (Allocation Period != 0b00){ |  |  |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| … | … | … |
| } |  |  |

Table 680 UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4DL subframes for FDD or U subframes for TDD [ U depends on TDD subframe configuration. ] 0b1: 4DL subframes for FDD or all DL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| … | … | … |
| } |  |  |

- Proposal#2

- 15.3.9.2.2 A-MAP relevance and HARQ timing

- …

- 15.3.9.2.2.2 TDD

- 15.3.9.2.2.2.1 Downlink

In DL HARQ transmission, DL Basic Assignment A-MAP IE, the HARQ subpacket, and the corresponding feedback shall follow the timing defined in Table 725—.

Table 725 FDD UL TDD DL HARQ timing

| Content | Subframe Index | Frame Index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m = l or m = l+NA-MAP – 1 | i |
| HARQ feedback in UL | For D > U n=0,m−K,U−1,forforfor0≤m<KK≤m<U+KU+K≤m<D | j=i+zmod4 or j=(i+z+1)mod4 (for l=D – 1, D:U=6:2 and Tproc=3) |
|  | For D≤U, n=m−K |  |

DL HARQ subpacket transmission corresponding to a DL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 725—. In the table, if the sum of D and U is an odd number and D is less than U /NA-MAP, K=ceilD−U2 for D≥U, and K=−ceilD−U2 forD<U. Otherwise, K=floorD−U2 for D≥U, and K=−floorD−U2 for D<U.

Note that the subframe index l shall range from 0 to NA-MAP・(ceil(D/ NA-MAP) - 1) D-1with an increment of  NA-MAP. For NA-MAP =2, m shall be selected between l and l+1. The selection information of m shall be provided in DL Assignment A-MAP IE.

DL HARQ feedback offset z shall be set to 1, only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with z = 0 is shorter than the data burst processing time Tproc. Otherwise, z shall be set to 0. This rule shall be also applied to the long TTI transmission:

z=

where NTTI is the number of subframes which a HARQ subpacket spans; i.e. 1 for the default TTI and D for the long TTI in TDD DL. The index m in Table 725— indicates the 1st subframe which a long TTI subpacket spans.

============================ End of Proposed Text ===================================