| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for a set of messages for location based service in connect state(LBS) |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Chuan Zhong, Fangmin Xu Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Yih-Shen Chen, Paul Cheng yihshen.chen@mediatek.com MediaTek | l.zhou@samsung.com +86-10-59253333 |
| Re: | P802.16m/D1 LBS: IEEE 802.16m-09/0037, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes a scheme to transmit concurrently multiple broadcast management messages in a subframe; especially to process paging message and traffic indication message in the subframe. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for a set of messages for location based service             in connect state

Lei Zhou, Fangmin Xu, Xufeng Zheng, Chuan Zhong

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

In the current SDD, description on LBS report and connect state is as follows:

12.3 Reporting Methods for LBS

For E911 services, the AMS location can be reported to ABS through UL inband signaling. Other reporting methods are FFS.

12.3.1	Reporting Types

According to the measurement methods of LBS, some location information or some LBS measurement parameters such as CINR/RSSI/RD/RTD/Angle are transmitted to the ABS to measure the location.

12.3.2	Reporting Mode

An AMS supported LBS reports location information if any of following location information reporting condition is met.

-Timer based location information reporting

-Threshold based location information reporting

An LBS-capable AMS should support the following reporting modes: per-request, periodic, and event-triggered reporting modes. The event-triggered reporting mode is a variation of the periodic reporting mode with reporting criteria, such as a moving distance threshold and updated timer expiration. For example, the AMS will report the location when the distance between the current location and the last reported location beyond the “moving distance threshold”.

12.4.1 Connected State

The system should be able to locate the mobile when in connected state.

For connected state, LBS can be initiated by the ABS or the AMS. LBS message contains some LBS information, which may include identifier of the AMS, and indicator of LBS measurement method. Other associated parameters for LBS measurement are FFS. Indicator of LBS measurement is used to instruct the ABS and/or the AMS to perform LBS measurement and report location information.

Based on above-mentioned description of SDD, we can know that for connected state, LBS can be initiated by the ABS and AMS need report its measurement result to ABS. So in connect state, we need a set of control messages for LBS including Broadcast (AAI_LBS-ADV), Request (AAI_LBS-REQ) and Response (AAI_LBS-RSP) messages.

In the current AWD, There is neither general definition nor description about above-mentioned LBS control messages in connect state. So we propose four control messages to be satisfied with requirement of location based service in connect state.

1.	MS managed location.

(1)	In this scenario, the MS measures and calculates its location. The measurement could be based on Relative Delay (RD for TDOA). The procedure for this is shown in Figure 1.

Figure 1 the procedure of MS managed location

Description of procedure of MS managed location is as follows:

Step 1 : The serving ABS periodically sends broadcast messages (AAI-NBR-ADV) and AAI_LBS-ADV;

Step 2 : The serving ABS sends DL reference signals to AMS and then AMS measures and scans

Step 3 : neighbor ABS2 sends DL reference signals to AMS and then AMS measures and scans

Step 4 : neighbor ABS3 sends DL reference signals to AMS and then AMS measures and calculation AMS location;

2. Network managed location.

Network managed location has three scenarios.

a)	AMS measurement and the network calculation (For example, DL-TDOA, RSSI, etc). The procedure for this is shown in Figure 2

Figure 2 MS measurement and the network calculation

Description of procedure of MS measurement and the network calculation is as follows:

Step 1: LBS trigger (Location request from location server (LS));

Step 2 : After the Serving ABS gets location request (from Location Server), the Serving ABS sends AAI_LBS_RSP signaling message to AMS;

Step 3 : The serving ABS periodically sends broadcast messages (AAI-NBR-ADV) and AAI_LBS-ADV;

Step 4 : The serving ABS sends DL reference signals to AMS and then AMS measures and scans

Step 5 : neighbor ABS2 sends DL reference signals to AMS and then AMS measures and scans

Step 6 : neighbor ABS3 sends DL reference signals to AMS and then AMS measures;

Step 7 : AMS sends measurement report AAI_LBS_RSP to the serving ABS and then ABS feedbacks measurement results to Location Server;

Step 8 : Location Server calculation AMS location based on AMS measurement report

b)	Network measurement and network calculation.(For example, UTDOA, etc). The procedure for this is shown in figure 3(a) (b).

Figure 3(a) Network measurement and network calculation.

Description of procedure of Network measurement and network calculation (a) is as follows:

Step 1: LBS trigger (Location request from location server (LS));

Step 2: After the Serving ABS gets location request (from Location Server), the Serving ABS sends AAI_LBS_REQ signaling message to AMS;

Step 3 : The AMS sends UL ranging signal to the serving ABS

Step 4 : The AMS sends UL ranging signal to neighbor ABS2

Step 5 : The AMS sends UL ranging signal to neighbor ABS3

Step 6 : the serving ABS, neighbor ABS2 and ABS3 measure respectively delay time t

Step 7 : neighbor ABS2 and ABS3 feedback measuring time t2 and t3 to the serving ABS and then ABS feedback measurement time to Location server;

Step 8 : The Location Server calculations AMS location based on feedback measurement time

Figure 3(b) Network measurement and network calculation

Description of procedure of Network measurement and network calculation (b) is as follows:

Step 1: LBS trigger (Location request from location server (LS));

Step 2: After the Serving ABS gets location request (from Location Server), the Serving ABS respectively sends three AAI_LBS_REQ signaling messages to AMS;

Step 3 : The AMS sends UL ranging signal to the serving ABS;

Step 4 : The AMS sends UL ranging signal to neighbor ABS2;

Step 5 : The AMS sends UL ranging signal to neighbor ABS3;

Step 6 : The serving ABS, neighbor ABS2 and ABS3 measure respectively time t;

Step 7 : Neighbor ABS2 and ABS3 feedback measuring time t2 and t3 to the serving ABS and then ABS feedback measurement time to the Location Server;

Step 8 : The Location Server calculates AMS location based on feedback measurement time

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

15.2.6.X  AAI_LBS-ADV

A ABS may use the AAI_LBS-ADV message to broadcast the LBS information. The message may be broadcast periodically without solicitation. This message is sent from the ABS to all AMSs on 12-bits STID and 4bits FID.

The following parameters shall be included in the AAI_LBS-ADV message:

- Number_of_BS

Number of neighbor BSs included in this message that are identified using the BSID

- BSID

he Base Station ID parameter in the A-MAP message of the Serving BS or Neighbor BS

- Number_of_BS_Index

Number of neighbor BSs included in this message that are identified using an index to their position in the NBR-ADV message

- Configuration change count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

- Neighbor_BS_Index

Index that corresponds to the position of the BS in the NBR-ADV message

- Absolute Position (Long Format)

The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and if the Absolute Position (Short Format) is not used.

- Absolute Position (Short Format)

The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and if the Absolute Position (Long Format) is not used.

- Relative Position

This Relative Position is used to provide the absolute position of an ABS. A Neighbor BS’s coordinates. When this Relative Position is included it provides the position of a neighbor BS relatively to the transmitting BS.

- GPS Time

Information about GPS time and time accuracy.

- Frequency Accuracy

Information about the frequency accuracy.

15.2.6.X+1 AAI_LBS-REQ

An AAI_LBS-REQ message shall be transmitted by the ABS. An ABS may transmit AAI_LBS-REQ to start AMS LBS measurement response with or without measurement allocation. The message shall be transmitted on the Basic STID+FID.

An ABS index bitmap (Req_Bitmap_Index) in a AAI_LBS-Rsp message may be used to identify AAI_LBS-Req ABS indexes such that each bit position corresponds to an ABS Index of the corresponding AAI_LBS_REQ message, the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last recommended ABS, and ABSs with ABS Index greater than the last recommended ABS are not recommended and do not have a corresponding bit position in the bitmap. Bitmap position bit value of 1 indicates that the ABS is recommended and a bit value of 0 indicates that the ABS is not recommended.

The following parameters shall be included in the AAI_LBS-REQ message:

- LBS_method_type (CellID, D-TDOA, U-TDOA Hybrid method(such as D-TDOA+TOA))

- Response mode

0b00: per-request report

0b01: Periodic report

0b10: No report (For DL-TDOA, stop report, for UL-TDOA, no report)

0b11: TBD

- Response period

The period of AMS’s report of scanning result when the AMS is required to response the value periodically or one-time. The period is calculated from the start of the first scan duration. If the ABS sends an AAI_LBS-REQ message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_LBS-REQ message. Response period is the number of frames that BS suggests to AMS before receiving next LBS-REQ.

- Response metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be:

Bit 0:ABS CINR mean

Bit 1:ABS RSSI mean

Bit 2: Relative delay

Bit 3: ABS RTD; this metric shall be only measured on serving BS/anchor BS

- ABS type (macro, micro, femto, relay, TBD)

- Use_Nbr_Bitmap_Index

Indicates if the bitmap of ABS indexes for AAI_NBR-ADV is used.

- Req_Seq_Num

One-bit sequence number for the AAI_LBS-REQ message that is toggled for each new message and may be included in a AAI_LBS-REP message to identify the AAI_LBS-REQ message associated with a AAI_LBS-REQ ABS Index bitmap (Rsp_Bitmap_Index), where a bit position in this bitmap corresponds to a BS index of the AAI_LBS-REQ message and where an ABS index corresponds to the position of a BS in the AAI_LBS-REQ message. When an ABS receives a AAI_LBS-RSP message from an AMS that includes Req_Seq_Num, the ABS shall compare its stored value of Rsp_Seq_Num with the one included in the AAI_LBS-RSP message and discard the AAI_LBS-RSP message if there is a mismatch.

- Configuration change count for  NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

- Nbr_Bitmap_Size

This may be less than or equal to the number of BSs in AAI_NBR-ADV.

- Nbr_Bitmap_Index

Bitmap of ABS indexes for the corresponding AAI_NBR-ADV message where each bit position corresponds to an ABS Index of the corresponding AAI_NBR-ADV message, the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last recommended ABS, and ABSs with ABS Index greater than the last recommended BS are not reported and do not have a corresponding bit position in the bitmap. Bitmap position bit value of 1 indicates that the ABS is recommended and a bit value of 0 indicates that the ABS is not recommended.When Use_Req_Bitmap_Index equals 1, Nbr_Bitmap_Index only includes ABSs included in the AAI_NBR-ADV message but not included in the corresponding AAI_LBS-REQ message.

- Scanning_Signal_type (bitmap indicating signal type)

Bit 0: A-preamble (SA-Preamble)

Bit 1: Ranging

- Neighbor_ABS_Index

ABS index corresponds to position of ABS in AAI_NBR-ADV message.

- N_Recommended_BS_Index

Number of neighbor BSs included in this message that are identified using an index to their position in the AAI_NBR-ADV message

- Preamble Index

This parameter defines the PHY specific.

- BS ERIP

This field is present only if the BS EIRP indicator is set in PHY Profile ID. Otherwise, the ABS has the same EIRP as the serving ABS.

- Ranging Opportunity Information

- This field includes ranging code and ranging resource allocation.

- Periodicity of Ranging:

The parameter defines the time that AMS performs periodic ranging with its serving and neighboring ABSs.

- Report update value: next preferred update timer due to crossing the boundary of the cell or threshold to measure for moving distance threshold

Updated Timer (t)

- Threshold

15.2.6.X+2 AAI_LBS-RSP

When the response mode is event triggered in the most recently received AAI_LBS-REQ, the MS shall transmit an AAI_LBS-RSP message to report the LBS measurement results to its serving ABS after each measurement period if the trigger condition is met. For per-request report mode, the AMS reports the LBS measurement results to its serving ABS at the time indicated in the AAI_LBS-REQ message. For a periodic report, the AMS stops reporting after all measurement intervals in the AAI_LBS-REQ message. The AMS shall include all available scanning results for the requested ABSs specified in the AAI_LBS-REQ message. The MS may transmit an AAI_LBS-RSP message to report the LBS measurement results to its serving ABS at anytime. The message shall be transmitted on the Primary Management STID+FID.

The following parameters shall be included in the AAI_LBS-RSP message:

- Response mode

Action code for an AMS’s scan report of its measurement:

- 0b00: per-request report

- 0b01: Periodic report

- 0b10: stop report (no report)

- 0b11: TBD (event triggered)

- Response metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be:

Bit 0: BS CINR Mean

Bit 1: BS RSSI mean

Bit 2: Relative delay

Bit 3: BS RTD; this metric shall be only measured on serving BS/anchor BS

- Use_Nbr_Bitmap_Index

Indicates if the bitmap of ABS indexes for AAI_NBR-ADV is used.

- Use_Req_Bitmap_Index

Indicates if the bitmap of BS indexes for AAI_LBS-REQ is used.

- N_Neighbor_BS_Index

BS index corresponds to position of BS in NBR-ADV message

- Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor ABSID.

- Nbr_Bitmap_Size

This may be less than or equal to the number of ABSs in AAI_NBR-ADV.

- Nbr_Bitmap_Index

Bitmap of ABS indexes for the corresponding AAI_NBR-ADV message where each bit position corresponds to an ABS Index of the corresponding AAI_NBR-ADV message, the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last reported ABS, and ABSs with ABS Index greater than the last reported ABS are not reported and don’t have a corresponding bit position in the bitmap. Bitmap position bit value of 1 indicates that the ABS is reported and a bit value of 0 indicates that the ABS is not reported. When Use_Req_Bitmap_Index equals 1, Nbr_Bitmap_Index only includes BSs included in the AAI_NBR-ADV message but not included in the corresponding AAI_LBS-REQ message.

- ABS type (macro, micro, femto, relay, TBD)

- BS CINR mean

The BS CINR Mean parameter indicates the CINR measured by the AMS from the particular ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the particular ABS’s segment and averaged over the measurement period.

- ABS RSSI mean

The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the particular ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as –103.75 dBm. An AMS shall be able to report values in the range –103.75 dBm to –40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period.

- Relative delay

This parameter indicates the delay of neighbor DL signals relative to the serving ABS, as measured by the AMS for the particular ABS. The value shall be interpreted as a signed integer in units of samples.

- ABS RTD

The ABS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the serving ABS. RTD can be given by the latest time advance taken by AMS. The value shall be interpreted as an unsigned byte with units of 1/Fs. This parameter shall be only measured on serving ABS/anchor ABS.

- Configuration change count for NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

- Req_Seq_Num

Sequence number for the AAI_LBS-REQ message associated with a AAI_LBS-REQ ABS Index bitmap (Req_Bitmap_Index), where a bit position in this bitmap corresponds to an ABS index of the AAI_LBS-REQ message and where an ABS index corresponds to the position of an ABS in the AAI_LBS-REQ message.

- Req_Bitmap_Index

Bitmap of indexes for the corresponding AAI_LBS-REQ message, where each bit position corresponds to an ABS Index of the corresponding AAI_LBS-REQ message, the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last reported ABS, and ABSs with ABS Index greater than the last reported ABS are not reported and do not have a corresponding bit position in the bitmap. Bitmap position bit value of 1 indicates that the BS is reported and a bit value of 0 indicates that the BS is not reported.

- N_Neighbor_BS_Index

Number of neighboring BS to be scanned or associated, which are included in AAI_NBR-ADV message.

- Neighbor_ABS_Index

ABS index corresponds to position of ABS in AAI_NBR-ADV message.

- Report Timer

-------------------------------  Text End  ---------------------------------------------------