| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto AWD Text Proposal for 15.4.7 Network Entry |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Feng Xie，Zhaohua Lu，Yang Liu， Kaiying Lv，Ying Liu，Hongyun Qu，Mary Chion ZTE Corporation | {xie.feng2, lu.zhaohua, liu.yang8, lv.kaiying, liu.ying, qu.hongyun}@zte.com.cn mchion@zteusa.com |
| Re: | Call for LB 30 |  |
| Abstract | This contribution provides the Femtocell AWD Text. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Femto AWD Text Proposal for 15.4.7 Network Entry

Feng Xie，Zhaohua Lu，Yang Liu，Kaiying Lv，Ying Liu，Hongyun Qu, Mary Chion

ZTE Corporation

- Introduction

For network-initiated update of CSG whitelist, any change of CSG members or subscribers is initiated by the operator or the owner of CSG femto ABSs. However, compared with the AMS-initiated (or user-initiated) update of CSG whitelist, the real use of network-initiated update of CSG whitelist has some limitations:

- For network-initiated update of CSG whitelist, a user is required to find out the MAC ID of his/her AMS and tell it to the operator or the owner of CSG femto ABSs. The procedure of obtaining MAC ID is hardly known to ordinary users. Then such use case is quite user-unfriendly.

- The network-initiated update of CSG whitelist is not straightforward for use if a user is requesting to join a CSG. It requires intensive interactions between the user and the owner, especially if the owner has to instruct the user to find out the MAC ID.

- The network-initiated update of CSG whitelist requires the owner to have specific expertise in managing the CSG whitelist. For instance, the owner must be familiar with the CSG management application as well as the way of finding out the MAC ID of AMS. Then it will be embarrassing when a user asking to join the CSG while the “expert” owner is absent. For example, Bob and his brother own a CSG femto ABS in their home. Bob has extensive knowledge on managing the CSG whitelist but his brother doesn’t. One day Bob’s uncle comes to his house while Bob is out. Then it becomes difficult for Bob’s brother to add his uncle to the CSG.

In comparison, AMS-initiated (or user-initiated) CSG whitelist update has the following advantages:

- The MAC ID of an AMS is present at the femto ABS when it receives a request from the AMS for joining the CSG..

- The femto owner/manager can easily add users to CSG by simply click on the “accept the request” or “reject the request” on the interface of adding new CSG members.

- The user with his/her AMS can simply send out the CSG-joining request by manually selecting the CSG femto ABS on user interface.

So we can see that AMS-initiated request for joining CSG can be used as a second option of CSG whitelist management.

AMS-initiated update of CSG whitelist can be realized in manual femto ABS selection, which is familiar to WiFi users. Manual femto ABS selection enables a human user to select femto ABSs according to the user’s knowledge and preference. With this feature, a human user is able to initiate a request for joining the CSG when he/she comes to the range of a visible femto ABS. Upon receiving the AMS’s request, the network will initiate pre-defined authorization process. Based on the decision of the femto ABS owner (accept/reject) and/or the operator’s policy, the access of AMS is accepted or rejected. Manual femto ABS selection also enables a human user to select the femto ABS with the most appealing subscription fees, the femto ABS with the application (such as a game or video) that a user likes, etc.

- Proposed Scheme for Manual Femto ABS Selection

Manual femto ABS selection enables a human user to select a femto ABS. In manual femto ABS selection the AMS may scan all frequencies in the supported frequency bands, with SA-preamble partitioning information, for all or specific subscription types of femto ABSs. Then AMS displays a list of found femto ABS identifiers, the femto ABS subscription type, and indications as to whether the found femto ABSs are contained in the AMS’s CSG white list.

If the user selects an entry in the list, an AMS is allowed to attempt to access a CSG-Closed/CSG-Open femto ABS not contained in the CSG white list. Based on the access response (e.g. access accepted or rejected) from the CSG-Closed/CSG-Open femto ABS, the AMS’s CSG white list is updated.

The AMS is allowed to not support manual femto ABS selection in connected mode.

Reference:

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface, P802.16m/D1 July 2009.

[2] Requirements for WiMAX Femtocell Systems, Version 1.0.0, Apr. 2009.

[3] IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document (SDD)”, July 2009.

- AWD Text Proposal for 15.4.7 Network Entry

Black text: existing text in AWD (C802.16m-09/D1)

Red Strikethrough text: deleted text

Blue text: new text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15.4.7 Network Entry

15.4.7.1 Femto ABS detection, identification and selection

The AMS detects the Femto ABS using the PHY identifier (see section x).

The AMS identifies the Femto ABS using the MAC identifier (ex. BSID, CSGID).

The AMS shall use scanning procedures specified in 15.2.15.2. When a CSG white list is maintained at an AMS, it should select the Femto ABSs according to the white list.

The AMS selects the target Femto ABS for network entry based on the subscription types of the Femto ABS and the membership to the relevant subscribers groups or accessibility to the service providers network (ex: Cell bar).

15.4.7.2 Manual Femto ABS Selection

Manual femto ABS selection enables a human user to select a femto ABS. In manual femto ABS selection the AMS may blind scan all frequencies in the supported frequency bands, with SA-preamble partitioning information, for all or specific subscription types of femto ABSs. Then AMS displays a list of found femto ABSs for user’s selection.

If the user selects an entry in the list, an AMS is allowed to attempt to access a CSG-Closed femto ABS not contained in the CSG white list. Based on the access response (e.g. AAI_RNG-RSP with indication of access accepted or rejected) from the CSG-Closed femto ABS, the AMS’s CSG white list is updated.

15.4.7.23 Femto ABS Access Restrictions

AMS shall not attempt to access or initiate handover to a CSG-closed femto ABS, if such an AMS is not included in the Femto ABS CSG, except in case of emergency call or manual Femto BS selection (see 15.4.7.2).

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

- AWD Text Proposal for 15.2.6.1 AAI_RNG-REQ

Black text: existing text in AWD (C802.16m-09/D1)

Red Strikethrough text: deleted text

Blue text: new text

| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#5 is set to 1, it indicates that the AMS is initiating the idle mode location update process with manual CSG selection. Bit#56-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| --- | --- | --- |

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

- AWD Text Proposal for 15.2.6.2 AAI_RNG-RSP

Black text: existing text in AWD (C802.16m-09/D1)

Red Strikethrough text: deleted text

Blue text: new text

| Location Update Response | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved Success of location update and manual CSG selection 0x03=Success of location update and DL traffic pending 0x04~0xFF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update |
| --- | --- | --- |

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++