| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Handover Scheme With SA-preamble Partitioning Information for 15.4.8 Handover(HO) |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Feng Xie，Zhaohua Lu，Yang Liu， Kaiying Lv，Ying Liu，Hongyun Qu，Mary Chion ZTE Corporation | {xie.feng2, lu.zhaohua, liu.yang8, lv.kaiying, liu.ying, qu.hongyun}@zte.com.cn mchion@zteusa.com |
| Re: | Call for LB 30 |  |
| Abstract | This contribution provides the Femtocell AWD Text. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Handover Scheme With SA-preamble Partitioning Information for 15.4.8 Handover(HO)

Feng Xie，Zhaohua Lu，Yang Liu，Kaiying Lv，Ying Liu，Hongyun Qu, Mary Chion

ZTE Corporation

- Introduction

Design Criteria:

- Typical handover procedures supported in Legacy System should also be supported by Advanced System.

- Advanced System can have features different from Legacy System if the feature is proved absolutely necessary.

- There might be a large number of femto BSs (e.g. > 500) inside or outside the coverage area of a macro BS.

- An MS may not have access to a large number of neighboring femto BSs (e.g. > 500).

- The number of femto BSs accessible to an MS within the coverage area of a macro BS is limited (e.g. 10).

- During emergency call, all neighboring femto BSs are considered accessible to the MS; and thus there might be a large number of femto BSs accessible to the MS during emergency call (e.g. >500).

- The MS scan latency and power consumption shall be reduced, esp. when a large number of femto BSs need to be scanned.

- The MS belonging to an operator’s network may uniquely operate in a different FA.

- Handover between a macro BS and femto BSs or between two femto BSs may be disabled by the serving NAP/NSP (e.g. if the target BS belongs to different NAP/NSPs).

- An MS may or may not be femto-aware.

- LBS may or may not be supported, and thus handover SHALL be able to work without LBS.

- Proposed Scheme for Handover to CSG-Open femto ABS with SA-preamble Partitioning Information in Critical Conditions

A macro/femto ABS can provide an AMS with a list of accessible femto ABSs. Based on the list, AMS can scan for candidate femto ABSs for HO. However, AMS has no knowledge on other CSG-Open femto ABSs which are accessible only in critical conditions for maintaining their connections. In SDD [3], “MS’s that are not members of CSG for a CSG-Open Femtocell BS’s shall not attempt network entry or handover to the BS’s unless it is critical to maintain their connection or in case of emergency call.” For example, if an MS couldn’t maintain a list of candidate BSes, it should search for such CSG-Open femto BSs that may accept its access request under critical conditions. In that case, the MS can use SA-preamble partitioning information for femtocell BS detection.

Reference:

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface, P802.16m/D1 July 2009.

[2] Requirements for WiMAX Femtocell Systems, Version 1.0.0, Apr. 2009.

[3] IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document (SDD)”, July 2009.

- AWD Text Proposal

Black text: existing text in AWD (C802.16m-09/D1)

Red Strikethrough text: deleted text

Blue text: new text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15.4.8 Handover (HO)

The handover process of an AMS between a Femto ABS and a Macro ABS or between two Femto ABSs will follow the same procedure as described in section 15.2.615.2.7 with the exception of procedures described in this section. For Femto ABSs that support the Legacy MSs they shall follow the same procedure as described in section 6.3.21 for WirelessMAN-OFDMA Reference System.

15.4.8.1 HO from Macro ABS to Femto ABS

15.4.8.1.1 Network Topology Acquisition

15.4.8.1.1.1 Network Topology Advertisement

15.4.8.1.1.2 AMS scanning of neighbor femto ABSs

AMS may access unsubscribed CSG-Open femto ABSs if no candidate ABSs are available at the AMS after scanning accessible femto ABSs and macro ABSs. For this purpose, AMS detects CSG-Open femto ABS which may accept its access request under critical conditions. AMS scans such femto ABS using SA-preamble partitioning information (See 15.4.4).

15.4.8.2 HO from Femto ABS to Macro ABS or other Femto ABS

15.4.8.2.1 Network Topology Acquisition

15.4.8.2.1.1 Network Topology Advertisement

15.4.8.2.1.2 AMS scanning of neighbor femto ABSs

AMS may access unsubscribed CSG-Open femto ABSs if no candidate ABSs are available at the AMS after scanning accessible femto ABSs and macro ABSs. For this purpose, AMS detects CSG-Open femto ABS which may accept its access request under critical conditions. AMS scans such femto ABS with SA-preamble partitioning information (See 15.4.4).

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++