| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femtocell BS Text Proposal: Idle Mode Support for Femtocell |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | Yang Liu ，Zhaohua Lu，Feng Xie， Kaiying Lv，Ying Liu，Hongyun Qu, Mary Chion, Tricci So ZTE Corporation | liu.yang8@zte.com.cn lu.zhaohua@zte.com.cn lv.kaiying@zte.com.cn mchion@zteusa.com |
| Re: | Call for Letter Ballot 30. |  |
| Abstract | This contribution provides the Text for idle mode for Femtocell operation. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Femtocell BS Text Proposal: Idle Mode Support for Femtocell

Yang Liu，Zhaohua Lu，Feng Xie，Kaiying Lv, Ying Liu, Hongyun Qu, Mary Chion, Tricci So

ZTE Corporation

- Introduction

A natural thought for idle in femto is: when the MS is under a CSG-Closed Femto, only this Femto BS should page it and no macros BS (in the same PG) should page it. We propose a method for this goal without changing any current paging mechanism defined in IEEE P802.16m D1.

In the figure example below, on the left side, the femto cells are assigned with paging group ID 2, the Macro cells are assigned with paging group ID 128 and 2; on the right side, the femto cells are assigned with paging group ID 1, the Macro cells are assigned with paging group ID 128 and 1. In the middle, the Macro cell is assigned with paging group 128, 1, and 2.

128, 1

128, 1

128, 1, 2

128, 2

128, 2

2

2

2

1

1

1

1

1

1

1

2

2

2

n

CSG Femtocell assigned to CSG Paging Group n

M, n

Macro-cell assigned to Public Paging Group m & CSG Paging Group n

Assignment Area for

CSG Paging Group 2

Assignment Area for

CSG Paging Group 1

The right side is used as our starting point for discussion.

If AMS enter sleep mode under one femto cell, it is in the paging group with ID 1. Therefore, the system knows it is in femto cell.

If AMS enter sleep mode under Macro and  moves from Macro to femto, it performs location update due to the paging group ID change. (from paging group 128 to paging group 1). the system knows it is in femto. When the AMS moves out of the femto, it finds the paging group ID 1 in the Macro ABS's paging group set (ABS has 1 and 128), nothing will happen to the AMS. If the AMS leaves the femto, when paging happens in femtocell, the AMS will not be found. Since no location update is received, the network knows that AMS is still in paging group 1, therefore, a new paging in paging group 1 with Macro ABS included will be performed.

If AMS enter sleep mode in femto and moves from femto to Macro, it will not perform location update. If the paging in femto does not find the AMS, the system knows it is in Macro and paging the AMS again with paging group ID 1 in all the macro BSs which in the same paging group.

- Text Proposal

Black text: existing text in SDD (C802.16m-08/003r9a)

Red Strikethrough text: deleted text

Blue text: new text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15.4.9 Idle Mode

Femtocell BS shall support idle mode.

The OSG Femtocell BSs operate like macro BSs in Idle mode.

Dependent on topology design to support both Femtocell BS and macro BS, one or more paging groups may be assigned to a Femtocell or a macro BS. The overlay macro BS and the Femtocell BS may share the same paging group ID.

A CSG Femtocell BS may broadcast the paging messages that are related to their subscribed AMSs only.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++