| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on Interference Mitigation |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Kun Liu，Zhaohua Lu ZTE Corporation | liu.kun12@zte.com.cn lu.zhaohua@zte.com.cn |
| Re: | Call for LB 30 |  |
| Abstract | The contribution proposed the text relevant to Interference Mitigation section included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on Interference Mitigation

Kun Liu, Zhaohua Lu

ZTE Corporation

# Introduction

In current AWD text (P80216m_D1), AAI_FFR-CMD/REP messages can request and report the interference statistics for one or all frequency partitions. But it's more favorable to make these messages flexible that can support not only for a specific frequency partition and all frequency partitions, but also for more than one frequency partition. So we suggest to add a bitmap in the AAI_FFR-CMD/REP messages to indicate more than one frequency partition that shall request and report the interference statistics.

# Proposed text

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.2.6.17 AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message shall be transmitted by the ABS to instruct AMS perform measurement over specific frequency partition. The following are the parameters shall be included into the AAI_FFR-CMD message:

Table 694 AAI_FFR-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ FFR-CMD _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Full Report Flag | 1 | 0: Report the measurement result for specified frequency partition 1: Report the measurement result for full frequency partition |
| if (Full Report Flag == 0) { |  |  |
| Frequency Partition Index | 2 | - |
| Frequency Partition Bitmap | N_FP | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0:no report interference statistics 0b1: report interference statistics N_FP: the number of Frequency Partition. |
| Reserved | 5 | Shall be set to zero |
| } |  |  |
| else { |  |  |
| Reserved | 7 | Shall be set to zero |
| } |  |  |
| Measurement Duration | 8 | AMS shall reply AAI_FFR-REP message to report the measurement after this indicated time |
| Frame Offset | 8 | The offset (in units of frames) from the current frame in which the AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |
| Padding | variable | if needed for alignment to byte boundary |
| } |  |  |

15.2.6.18 AAI_FFR-REP (FFR Report) Message

An AAI_FFR-REP message is sent by an AMS to report the interference statistics of frequency partition. The following are the parameters shall be included into the AAI_FFR-REP message:

Table 695 AAI_FFR-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ FFR-REP _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Full Report Flag | 1 | 0: Report the measurement result for specified frequency partition 1: Report the measurement result for full frequency partition |
| if (Full Report Flag == 0) { |  |  |
| Frequency Partition Index | 2 | - |
| Frequency Partition Bitmap | N_FP | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0:no report interference statistics 0b1: report interference statistics N_FP: the number of Frequency Partitions. |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| If (Reporting Type == 0) { |  |  |
| for (n=0; n<N_Bitmap_1; n++) { |  | N_Bitmap_1: number of bit ‘1’ in Frequency Partition bitmap |
| Interference Measurement Report - Mean | 8 |  |
| Interference Measurement Report - Variance | 8 |  |
| Reserved | 4 |  |
| } |  |  |
| } else { |  |  |
| for (n=0; n<N_Bitmap_1; n++) { |  | N_Bitmap_1: number of bit ‘1’ in Frequency Partition bitmap |
| SINR Measurement Report – Mean | 8 |  |
| SINR Measurement Report – Variance | 8 |  |
| Reserved | 4 |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| for (n=0; n<N_FP; n++) { |  | N_FP: number of total frequency partition |
| If (Reporting Type == 0) { |  |  |
| Interference Measurement Report for FPn – Mean | 8 |  |
| Interference Measurement Report for FPn- Variance | 8 |  |
| } else { |  |  |
| SINR Measurement Report for FPn - Mean | 8 |  |
| SINR Measurement Report for FPn - Variance | 8 |  |
| } |  |  |
| } |  |  |
| Reserved | 6 |  |
| } |  |  |
| Padding | variable | if needed for alignment to byte boundary |
| } |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------