| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Resolving the FPEH, RFPEH, MEH, RMEH co-existence issue |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes fixes to MEH, MEHB, and RMEHB. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Resolving the FPEH, RFPEH, MEH, RMEH co-existence issue

Joey Chou, Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty

Intel

- Introduction

D1 draft includes FPEH, RFPEH, MEH, 1st MEHB, MEHB, 1st RMEHB and RMEHB to support packing / fragmentation of SDUs/SDU fragments for single or multiple flows that is very complicated and has flaws. For example:

- MEH is an extended header. FPEH is not (there is not EH type). According to the header design, it can’t stop the sender form packing GMH + MEH EH + FPEH into a MPDU.

- When a few information bits are added, new control bits are added, new EH are created (i.e. RFPEH, 1st RMEHB and RMEHB) that complicates the header design, and is prone to errors (e.g. RI bit definitions in Table 669 and Table 672 are wrong.

- The GMH length will indicate the length of the 1st SDU fragment in the 1st connection. There are N-1 length fields in MEHB/RMEHB that indicate the length of SDU fragment 2 to N, respectively. This is very complicated. The receiver needs to parse not only all extended headers, but all length fields in MEHB/RMEHB to figure out the size of a MPDU. The saving of 11 bit length field is so small in a 1000 byte MPDU when compares with the complexity it creates

- The reason of having Table 668 First MEHB format and Table 671 First RMEHB format is to save 4 bits FlowId. The reserved bits for byte alignment can be larger than 4 bits. So, the saving is immaterial when comparing with the complexity of Table 671 and 672.

This contribution proposes using a single FPEH header to support packing / fragmentation of SDUs/SDU fragments for single or multiple flows.

- Proposed text

15.2.2.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single or multiple transport connection. The FPEH exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in GMH set to '1' or after the GMH if 'EH' in GMH set to '0'. The FPEH format is defined in Table 659 .

[Editor Note: Replace Table 659 with the following.]

Table 659—FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH () { | - |  |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPH |
| N_Flows | 4 | Number of flows (1 to 15) |
| For (i = 1 ; i <= N_Flows ; i++) { |  |  |
| FlowID | 4 | Flow Identifier. The FlowID of the 1st flow will be the same as the FlowID in GMH. |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control (see Table 661) |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| AIC | 1 | ARQ Information Control bit 0 = No additional ARQ information 1 = Additional ARQ information included |
| If (AIC == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indication 0 = Indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = Indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | 1 | SUB-SN of the first ARQ sub-block |
| } |  |  |
| N_SDUF | 4 | Number of SDUs or SDU fragments (1 to 15) |
| For (j = 1 ; j <= N_SDUF ; j++) { | 1 |  |
| Length | 11 | Length of SDU or SDU fragment |
| } | - |  |
| } | - |  |
| Padding | variable | For byte alignment |
| } | - |  |

Table 661— Encoding of FC field

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | - One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | - MPDU with only First fragment of an SDU - MPDU with one or more unfragmented - SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | - MPDU with only Last fragment of an SDU - MPDU with Last fragment of an SDU, - followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | - MPDU with only middle fragment of an SDU - MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

[Editor Note: Delete 15.2.2.2.2 Rearrangement Fragmentation and Packing Extended Header (RFPEH).]

[Editor Note: Delete 15.2.2.2.4 Multiplexing extended header (MEH).]