| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Emergency alert indicator in AAI_PAG-ADV (15.3.12.2.1) |  |
| Date Submitted | 2009-08-27 |  |
| Source(s) | Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) Whai-En Chen National Ilan University (NIU) Shiann-Tsong Sheu National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16-09/0037 |  |
| Abstract | This contribution proposes text for emergency alert indicator in AAI_PAG-ADV |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Emergency alert indicator in AAI_PAG-ADV (15.3.12.2.1)

Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Whai-En Chen
National Ilan University (NIU)

Shiann-Tsong Sheu
National Central University (NCU)

Introduction

In current IEEE P802.16m/D1, for alerting the imminent disaster, the AMS in idle state will receive the emergency information in addition to paging message. Emergency alert indicator shall be included in the paging message to notify the idle AMSs reception of the emergency information. In this contribution, we propose the AAI_PAG-ADV with emergency alert indicator to clarify the procedure as shown in Table 1.

Table 1   Parameters for AAI_PAG-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Emergency alert indicator | 0 = No action. 1 = Decode the A-A-MAP for obtaining the emergency information. | Emergency alert indicator is used to notify AMSs in idle state the transmissions of emergency information. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |

Proposed text for the P802.16m/D1:

----------------------------------------------------Start of the Text---------------------------------------------------------

[Modify the text in Line 56, Page 207 as indicated]

#### 15.3.12.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement. In addition, an emergency alert indicator is included in the paging message to notify the idle AMSs receoption of the emergency information.

[Modify Table 792 as indicated]

Table 792   Parameters for AAI_PAG-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Emergency alert indicator | 0 = No action. 1 = Decode the A-A-MAP for obtaining the emergency information. | Emergency alert indicator is used to notify AMSs in idle state the transmissions of emergency information. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |

----------------------------------------------------End of the Text---------------------------------------------------------

[1] IEEE P802.16m/D1, “P802.16m/D1: Advanced Air Interface”