| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Neighboring Macro BS Self-configuration |  |
| Date Submitted | 2009-09-14 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: SON |  |
| Abstract | This contribution proposes text for Self-Organizing networks. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Neighboring Macro BS Self-configuration

Joey Chou

Intel

- Introduction

This contribution proposes a self-optimizing FFR text for AWD.

- Proposed text

15.5 Support for Self-organization

15.5.1 		Self-Configuration

Self-configuration is the process of initializing and configuring BSs automatically with minimum human intervention.

15.5.1.1 	Neighboring Macro BS Self-Configuration

Cellular networks today still require much manual configuration of neighboring macro BS to ensure neighboring cell sites work properly and hand off connections successfully that have greatly burdened the operators. Neighboring macro BS self-configuration provides a mechanism to enable a BS to automatically update its neighboring BS list and their associated attributes, as neighboring BSs are going online / offline dynamically. Since the deployment of macro BS is planned, and their locations are known by operators, its neighbors can be automatically identified by cell site and Sector Bearing. The details of the neighboring macro BS self-configuration procedure are described in Annex A. Neighboring macro BS self-configuration is to support the following 3 scenarios.

- A BS is going online

- A BS is going offline

- A BS has attribute changes

Table 1 shows the parameters the BS should send to the SON server to request neighbor list update.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Neighbor list update request () { | - |  |
| Request_type | 2 | 1 – a BS is going online 2 – a BS is going offline 3 – a BS has attribute changes |
| If (Request_type == 1) { |  |  |
| BSID | TBD | BSID of the BS going online |
| Longitude | 9 | Location of BS (0 – 360˚) |
| Latitude | 9 | Location of BS (0 – 360˚) |
| Sector bearing | 11 | Indicating the direction where the BS sector is pointing from the north ( 0– 360˚) |
| AAI_NBR-ADV attributes TLV | variable | Attributes of the BS going online to be sent in the AAI_NBR-ADV message by its neighbors |
| } |  |  |
| else If (Request_type == 2) { |  |  |
| BSID | TBD | BSID of the BS going offline |
| } |  |  |
| else If (Request_type == 3) { |  |  |
| BSID | TBD | BSID of the BS that request attribute changes |
| Attributes change TLV | variable | Indicate the attributes that have been changed. These attributes are defined in AAI_NBR-ADV message. |
| } |  |  |
| } | - |  |

Table 1: Neighbor List Update Request

Table 2 shows the parameters the SON server should provide to the BS to update its neighbor list.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Neighbor list update response () { | - |  |
| Response_type | 2 | 1 – a BS is going online 2 – a BS is going offline 3 – a BS has attribute changes |
| If (Request_type == 1) { |  |  |
| BSID | TBD | BSID of the neighbor BS just going online |
| AAI_NBR-ADV attributes TLV | variable | Attributes of the neighbor BS be sent in the AAI_NBR-ADV message |
| } |  |  |
| else If (Request_type == 2) { |  |  |
| BSID | TBD | BSID of the neighbor BS just going offline |
| } |  |  |
| else If (Request_type == 3) { |  |  |
| BSID | TBD | BSID of the BS that has attribute changes |
| Attributes change TLV | variable | Indicate the attributes that have been changed. These attributes are defined in AAI_NBR-ADV message. |
| } | - |  |
| } | - |  |

Table 2: Neighbor List Update Response

Annex A   Neighboring Macro BS Self-configuration (Informative)

BS self-configuration is currently supported by network management systems that can download MAC/PHY and other system attributes automatically, as soon as such BS becomes online. However, existing cellular networks still require much manual configuration and optimization to ensure neighboring cell sites work properly and hand off connections successfully. This Annex describes neighboring macro BS self-configuration that is intended to remove such manual tasks. It provides a mechanism to enable a BS to automatically update its neighboring BS list and their associated attributes, as neighboring BSs are going online / offline dynamically.

Figure 1: Example of Neighboring Macro BS Self-configuration

Figure 1 shows an example of Neighboring BS Self-Configuration procedure to be implemented in the SON_Server to supports the following scenarios:

- A BS going online

- A BS going offline

- Attribute changes in any BS

The key to the Neighboring BS Self-Configuration is to automatically find its neighbors. Figure 2 is a cell planning example that describes how SON_Server can identify its neighbors from cell site and Sector Bearing. The blue dot in each cell indicates the Cell Center in longitude and latitude. Haversine formula (1) can be used to compute the distance d between two Cell Centers.

Haversine formula (1):

R = earth’s radius (mean radius = 6,371km)
Δlat = lat2− lat1
Δlong = long2− long1
a = sin²(Δlat/2) + cos(lat1).cos(lat2).sin²(Δlong/2)
c = 2.atan2(√a, √(1−a))
d = R.c

It is assumed that Online_BS is in Cell_A with Cell Center = A. By computing the distances between A and all other cells in the serving area, SON_APP_Server can identify its neighboring BSs that have the shortest distance to Cell_A. For single sector cellular networks, Cell_B, Cell_C, Cell_D, Cell_E, Cell_F, and Cell_G will be the neighboring cells of Online_BS.

Cell_A

BSID_A2

BSID_A3

BSID_A1

Cell_D

BSID_D2

BSID_D3

BSID_D1

Cell_E

BSID_E2

BSID_E3

BSID_E1

Cell_F

BSID_F2

BSID_F3

BSID_F1

Cell_G

BSID_G2

BSID_G3

BSID_G1

Cell_B

BSID_B2

BSID_B3

BSID_B1

Cell_C

BSID_C2

BSID_C3

BSID_C1

θ

N

A1

A2

A3

B1

B2

B3

C1

C2

C3

D1

D2

D3

E1

E2

E3

F1

F2

F3

G1

G2

G3

A

B

C

D

E

F

G

Figure 2: Example of Cell Planning

For multi-sector cellular networks, the Sector Bearing parameter, indicating the direction where the sector is pointing, will be needed to locate its neighboring sectors. By providing the Cell Center (lat1 and long1), Sector Distance (d), and Sector Bearing (θ) to Haversine formula (2) below, the Sector Edge, as shown in the red dot in Figure 2, can be computed. Sector Distance should be chosen in a way that makes Sector Edge close to the edge of the sector.

Haversine formula (2):

lat2 = asin(sin(lat1)*cos(d/R) + cos(lat1)*sin(d/R)*cos(θ))

lon2 = lon1 + atan2(sin(θ)*sin(d/R)*cos(lat1), cos(d/R)−sin(lat1)*sin(lat2))

It is assumed that Online_BS is Sector Edge A1. SON_APP_Server will compute the Sector Edge for all sectors in the neighboring cells of Online_BS, based on Haversine formula (2). Then, it will compute the distance between A1 and all other Sector Edge. SON_APP_Server can identify its neighboring sectors that have the shortest distance to Sector Edge A1.

Table 5 shows an example of SON_Server database that contains information for SON_Server to identify its neighbors.

| BSID | Cell ID | Cell center locations | Sector bearing | Sector edge locations | Neighboring BS List | Status | Neighbor BS attributes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BSID_A1 | Cell_A | A | 15° | A1 | BSID C2, C3, D3, D2, E1, E3, A2, A3, B2, B1 | Online | Attributes as defined in AAI_NBR-ADV message. E.g. Channel Bandwidth, FFT Size, Cyclic Prefix, …. |
| BSID_A2 | Cell_A | A | 135° | A2 | BSID E1, E3, F1, F3, G1, G2, A1, A3, B2 | Online |  |
| BSID_A3 | Cell_A | A | 255° | A3 | BSID F1, F3, G2, G1, B2, B1, C3, C2, A1, A3 | Online |  |
| BSID_B1 | Cell_B | B | 15° | B1 | BSID C3, C2, A1, A3, B2 | Online |  |
| BSID_B2 | Cell_B | B | 135° | B2 | BSID C3, C2, A1, A3, G1, G3, B1 | Online |  |
| BSID_B3 | Cell_B | B | 255° | B3 | BSID B1, B2, G1, G3 | Offline |  |
| BSID_C1 | Cell_C | C | 15° | C1 | BSID D1, D3, C2, C3 | Offline |  |
| BSID_C2 | Cell_C | C | 135° | C2 | BSID B1, A1, A3, B2, B1, C3 | Online |  |
| BSID_C3 | Cell_C | C | 255° | C3 | BSID C2, A1, A3, B2, B1 | Online |  |

Table 5: Example of SON_App_Server Database