| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes to the supported permutations for each DL MIMO mode inside and outside the OL region in IEEE P802.16m/D1 (15.3.7.2.5) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | Corrections to the tables that specify the permutations that are supported for each DL MIMO mode inside and outside the OL region with MaxMt streams. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Editorial changes to the supported permutations for each DL MIMO mode inside and outside the OL region in IEEE P802.16m/D1 (15.3.7.2.5)

David Mazzarese et Al.

Samsung Electronics

# Introduction

Table 689 in [1] specifies the supported permutations for each DL MIMO mode, but this table is only applicable outside the OL region. A second table is required to specify which permutations are supported for each DL MIMO mode inside the OL region with MaxMt streams.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2.5 Usage of MIMO modes” on line 35 page 365 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.5 Usage of MIMO modes

Table 690 shows permutations supported for each MIMO mode outside the OL region. The definitions of DRU, mini-band based CRU, and subband based CRU are in subclause [TBD]15.3.5.

Table 690 – Supported permutation for each DL MIMO mode outside the OL region

|  | DRU | Mini-band based CRU (diversity allocation) | Mini-band and Subband based CRU (localized allocation) |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes | Yes | No |
| MIMO mode 1 | Yes, with Mt=2 | Yes, with Mt≤4 | Yes |
| MIMO mode 2 | No | Yes, with Mt=1 | Yes |
| MIMO mode 3 | No | No | Yes |
| MIMO mode 4 | No | Yes | Yes |
| MIMO mode 5 | No | No | No |

Table 691 shows permutations supported for each MIMO mode inside the OL region with MaxMt streams.

Table 691 – Supported permutation for each DL MIMO mode in the OL region with MaxMt streams

|  | DRU | Mini-band based CRU (diversity allocation) | Subband based CRU (localized allocation) |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes, with MaxMt =2 | No | No |
| MIMO mode 1 | Yes, with MaxMt =2 | No | Yes, with MaxMt =2 |
| MIMO mode 2 | No | No | No |
| MIMO mode 3 | No | No | Yes, with MaxMt =2 |
| MIMO mode 4 | No | No | No |
| MIMO mode 5 | No | Yes, with MaxMt =1 | Yes, with MaxMt =1 |

Miniband based CRU diversity allocation represents a resource allocation composed of non-contiguous minibands.

All pilots are precoded regardless of number of transmit antennas and allocation type.

-------------------------------  Text End  ---------------------------------------------------