| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes to encoding and precoding sections of uplink SU MIMO and MU MIMO modes in IEEE P802.16m/D1 (15.3.10.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | Clean-up of encoding and precoding sections of uplink SU MIMO and MU MIMO modes |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Editorial changes to encoding and precoding sections of uplink SU MIMO and MU MIMO modes in IEEE P802.16m/D1 (15.3.10.2)

David Mazzarese et Al.

Samsung Electronics

# Introduction

The encoding and precoding sections for the uplink SU-MIMO and MU-MIMO have been copied and pasted from the DL section, which is incorrect. The maximum number of streams with spatial multiplexing on the uplink is 4, not 8. Horizontal encoding is not used on the uplink, even for MU-MIMO, where vertical encoding shall be used. On the uplink, precoding is done at the AMS, not at the ABS. The text below proposes changes to correct these mistakes.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.10.2 Transmission schemes for data channels” on line 31 page 314 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.10.2 Transmission schemes for data channels

15.3.10.2.1 Encoding and precoding of SU-MIMO

15.3.10.2.1.1 Encoding of SU-MIMO modes

MIMO mode 0: SFBC encoding of section 15.3.10.1.1.1 shall be used with MIMO mode 0.

MIMO mode 1: Vertical encoding of section 15.3.10.1.1.2 shall be used with MIMO mode 1. The number of streams is Mt≤min(Nt,Nr), where Mt is no more than 8 4.

MIMO mode 2: Vertical encoding of section 15.3.10.1.1.2 shall be used with MIMO mode 2. The number of streams is Mt≤min(Nt,Nr), where Mt is no more than 8 4.

15.3.10.2.1.2 Precoding of SU-MIMO modes

MIMO mode 0: Non-adaptive precoding with Mt =2 streams shall be used with MIMO mode 0.

MIMO mode 1: Non-adaptive precoding with Mt streams shall be used with MIMO mode 1.

MIMO mode 2: Adaptive precoding shall be used with MIMO mode 2.

15.3.10.2.2 Encoding and precoding of collaborative spatial multiplexing (MU-MIMO) MU-MIMO modes

MSs can perform collaborative spatial multiplexing onto the same RU. In this case, the BS assigns different pilot patterns for each MS

15.3.10.2.2.1 Encoding of MU-MIMO modes

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more MSs. Multi-user transmission with one stream per user is supported for MU-MIMO.

MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 MSs, and 4Tx or 8Tx antennas to support up to 4 MSs, with 1 stream per MS.

Both OL MU-MIMO (mode 3) and CL MU-MIMO (mode 4) are supported with the following encoding:

MIMO mode 3: Horizontal Vertical encoding shall be used with MIMO mode 3.

MIMO mode 4: Horizontal Vertical encoding shall be used with MIMO mode 4.

15.3.10.2.2.2 Precoding of MU-MIMO modes

MIMO mode 3: Non-adaptive precoding shall be used with MIMO mode 3.

MIMO mode 4: Adaptive precoding shall be used with MIMO mode 4.

MIMO mode 3:

Non-adaptive precoding of section <<15.3.7.1.2.1>> shall be used with MIMO mode 3.

Inside the OL region with OL MU MIMO, the precoder W with 2 streams is predefined and fixed over time.

Outside the OL region with OL MU MIMO, the precoder W is an Nt×Mt sub-matrix of a predefined Nt×MaxMt matrix.

The precoding matrix W used by the ABS is represented in Equation (270).

W(k)=[v1(k)v2(k)⋯vM(k)]								(270)

Where vi(k) is the precoding vector for the i-th AMS on the k-th subcarrier.

vi(k) shall be used for precoding the pilot symbols on the i-th pilot stream on the k-th subcarrier.

MIMO mode 4

Adaptive precoding of section <<15.3.7.1.2.2>> shall be used with MIMO mode 4.

In CL MU MIMO, the precoder W is an Nt×M matrix for each subcarrier. It is used to communicate to M AMSs simultaneously. The form and derivation of the precoding matrix does not need to be known at the AMS. The ABS determines the precoding matrix based on the feedback received from the AMS.

The ABS shall construct the precoding matrix W as represented in Equation (271).

W(k)=[v1(k)v2(k)⋯vM(k)]								(271)

Where, vi(k) is the precoding vector for the i-th AMS on the k-th subcarrier.

-------------------------------  Text End  ---------------------------------------------------