| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Physical Structure to support multiplexing of E-MBS and Unicast traffic |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Kaiying Lv, Yanfeng Guan, Zhaohua Lu, Bo Sun | lv.kaiying@zte.com.cn |
| Re: | IEEE 802.16m-09/0037, “Call for Contributions on Project 802.16m Amendment Content”. Target topic: “E-MBS”. |  |
| Abstract | This contribution proposes downlink physical structure to support the multiplexing of E-MBS and Unicast traffic |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink Physical Structure to support multiplexing of E-MBS and Unicast traffic

Kaiying Lv , Yanfeng Guan, Zhaohua Lu, Bo Sun

ZTE Corporation

- Introduction

In order to allow supporting FDM multiplexing of E-MBS and unicast traffic in the downlink frame, this contribution analyses the frequency partition schemes and proposes a simple frequency partition and corresponding indication method.

It is desirable to make simple signaling and relatively flexible resource allocation for E-MBS traffic when FDM multiplexing with unicast in the downlink frame. Two frequency partitioning schemes could be applied. The details of schemes and comparison are shown below.

Scheme 1: to divide the whole frequency resource of a carrier into several partitions. The whole reuse-1 frequency partition is allocated to E-MBS traffics only, and other frequency partitions are allocated to unicast traffics only. The following constraints should be obeyed.

- E-MBS traffic is only allowed to be carried on reuse-1 frequency partition #0, i.e.,FP0. All the resources in this partition are mapping to DRUs.

- Unicast traffic is only allowed to be carried on reuse-3 frequency partitions, i.e., FP1 ,FP2 ,FP3. All the resources in this partition are mapping to DRUs.

- The frequency partitioning of E-MBS subframes can be different from that of unicast subframes.

- The frequency partition configuration is transmitted in the SFH in a 3 or 2- bit field called the EMBS Downlink Frequency Partition Configuration (E-DFPC) depending on system bandwidth. The Frequency Partition Count (FPCT) defines the number of frequency partitions in the subframe carrying E-MBS traffic. The Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from DFPC as shown in Table XXX1 through Table XXX3.

Table xxx1 Mapping between E-DFPC and frequency partitioning for 20 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |
| 2 | 5 : 1 : 1 : 1 | 4 | NPRU *5/8 | NPRU *1/8 |
| 3 | 9 : 1 : 1 : 1 | 4 | NPRU *3/4 | NPRU *1/12 |
| 4 | 9 : 5 : 5 : 5 | 4 | NPRU *3/8 | NPRU *5/24 |
| 5~7 | RESERVED |  |  |  |

Table xxx2 Mapping between E-DFPC and frequency partitioning for 10 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |
| 2 | 5 : 1 : 1 : 1 | 4 | NPRU *5/8 | NPRU *1/8 |
| 3 | RESERVED |  |  |  |

Table xxx3 Mapping between E-DFPC and frequency partitioning for 5 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |

When E-DFPC =0, it means that all frequency resources are allocated to EMBS traffic. The unicast and EMBS traffic are TDM multiplexed.

Scheme 2: to divide the whole frequency resource of a carrier into several partitions. The whole reuse-1 frequency partition is allocated to E-MBS traffics only, and other frequency partitions are allocated to unicast traffics only. The following constraints should be obeyed.

- E-MBS traffic is only allowed to be carried on reuse-1 frequency partition #0, i.e.,FP0. All the resources in this partition are mapping to DRUs.

- Unicast traffic is only allowed to be carried on reuse-3 frequency partitions, i.e., FP1 ,FP2 ,FP3. All the resources in this partition are mapping to CRUs and DRUs.

- The frequency partitioning of E-MBS subframes can be different from that of unicast subframes.

- The frequency partition configuration is transmitted in the SFH in a 3 or 2- bit field called the EMBS Downlink Frequency Partition Configuration (E-DFPC) depending on system bandwidth. The Frequency Partition Count (FPCT) defines the number of frequency partitions in the subframe carrying E-MBS traffic. The Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from DFPC as shown in Table XXX1 through Table XXX3.

- A 3, 2, or 1-bit field called the Downlink Frequency Partition Subband Count (DFPSC) defines the number of subbands allocated to FPi, i > 0.

- CRU/DRU allocation defines the partition between CRUs and DRUs for frequency partition FPi, i>0, which are allocated to unicast data. The number of total allocated subband and miniband CRUs, in units of a subband (i.e. N1 PRUs), for FPi (i > 0) is given by the downlink CRU allocation size, DCASi. A 5, 4 or 3-bit field DCASi is sent in the SFH for partition FPi.

- Comparison of three schemes

Comparing the above three schemes, we can find:

a. In scheme 1, the frequency partition configuration parameters for E-MBS subframe are very simple. Only one 3 or 2-bit field E-DFPC is transmitted in the SFH. It provides lower overhead on signaling but only DRUs are allocated to both EMBS and unicast data.

b. In scheme 2, the frequency partition configuration parameters for E-MBS subframe are more than that of scheme 1. A 3 or 2-bit field E-DFPC , a 5,4 or 3-bit field DSAC and a 3,2 or 1-bit field DCASi are transmitted In the SFH. It provides relative flexible resource type to unicast data but the overhead on signaling is a little bit high.

- Conclusion

These two schemes have trade-offs between flexibility and signaling overhead. Scheme 1 is a simple and less-overhead frequency partition method for E-MBS subframe. Since scheme 1is good enough to provide resources to unicast data without incurring more complexity and overhead, we prefer to scheme 1.

- Text proposal for the 802.16m D1 draft

- Proposes the addition of a new Section 15.x.x.x.x, “frequency partitioning for E-MBS subframes”.

=====================Start of Proposed Text Section 15.x.x.x.x =================================

Both TDM and FDM multiplexing can be applied for E-MBS and unicast traffic. When TDM multiplexing is applied, one frequency partition is allocated in the E-MBS subframe. When FDM multiplexing is applied, four frequency partitions are allocated. E-MBS data are always in reuse-1 frequency partition #0, and unicast data can be scheduled in frequency partition FPi, i>0. All PRUs in the subframe are miniband PRUs. PPRUMB are allocated to one or four frequency partitions. The EMBS frequency partition configuration is transmitted in the SFH in a 3 or 2-bit field called EMBS downlink frequency partition configuration (E-DFPC) depending on the system bandwidth. The Frequency Partition Count (FPCT) defines the number of frequency partitions. The Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from EFPC as shown in Table XXX1 through Table XXX3.

Table xxx1 Mapping between E-DFPC and frequency partitioning for 20 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |
| 2 | 5 : 1 : 1 : 1 | 4 | NPRU *5/8 | NPRU *1/8 |
| 3 | 9 : 1 : 1 : 1 | 4 | NPRU *3/4 | NPRU *1/12 |
| 4 | 9 : 5 : 5 : 5 | 4 | NPRU *3/8 | NPRU *5/24 |
| 5~7 | RESERVED |  |  |  |

Table xxx2 Mapping between E-DFPC and frequency partitioning for 10 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |
| 2 | 5 : 1 : 1 : 1 | 4 | NPRU *5/8 | NPRU *1/8 |
| 3 | RESERVED |  |  |  |

Table xxx3 Mapping between E-DFPC and frequency partitioning for 5 MHz

| E-DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 3 : 1 : 1 : 1 | 4 | NPRU *1/2 | NPRU *1/6 |

When E-DFPC =0, it means that all frequency resources are allocated to EMBS traffic. The unicast and EMBS traffic are TDM multiplexed.

=====================end of Proposed Text Section  15.x.x.x.x =================================

- Proposes the addition of a new Section 15.x.x.x.x, “subcarrier permutation for E-MBS DRUs”.

=====================Start of Proposed Text section 15.x.x.x.x ==========================================

Both the E-MBS and unicast frequency partitions are mapped to DRUs. The subcarrier permutation defined for the DL distributed resource allocation within E-MBS frequency partition (i.e.FP0) spreads the subcarriers across the entire E-MBS frequency partition. The granularity of the sub-carrier permutation is equal to a pair of subcarriers. For multi-cell E-MBS transmission, a non-cell specific (or zone specific) permutation sequence is applied to subcarrier permutation.

The subcarrier permutation defined for the DL distributed resource allocation within unicast frequency partition(s)(i.e.FPi) spreads the subcarriers across the entire unicast frequency partition.

=====================end of Proposed Text Section 15.x.x.x.x ==========================================