| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on E-MBS MAC Header |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Xiaojuan Shi, Kaiying Lv , Bo Sun ZTE Corp. | lv.kaiying@zte.com.cn shi.xiaojuan1@zte.com.cn |
| Re: | IEEE 802.16m-09/0037, “Call for Contributions on Project 802.16m Amendment Content”. P802.16m/D1 E-MBS |  |
| Abstract | This contribution proposes text for E-MBS MAC Header section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on E-MBS MAC Header in 16m

Xiaojuan Shi, Kaiying Lv, Bo Sun

ZTE Corp.

# Introduction

This contribution proposes the text for -EMBS MAC Header format based on P802.16m/D1 [1] and SDD [2].

For all ABSs that belong to the same E-MBS Zone, one unique identifier (multicast STID ) is used for a common E-MBS service flow or E-MBS content. Since the identifier of E-MBS service flow can be indicated in the E-MBS control signaling, it is not necessary to include the flow identifier in the MAC header.

This contribution proposed the amendment text on E-MBS MAC PDU formats. The following sections are added to the text.

15.x.x E-MBS MAC PDU formats

15.x.x.1 E-MBS MAC header formats

15.x.x.1.1 E-MBS compact MAC header (E-CMH)

15.x.x.2 E-MBS extended header formats

15.x.x.2.1 E-MBS fragmentation and packing extended header(E-FPEH)

15.x.x.2.2 E-MBS multiplexing extended header(E-MEH)

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.x.x.x as follows:]

15.2.x.x E-MBS MAC PDU formats

E-MBS MAC PDUs use the same common form illustrated in Figure 385. Each PDU shall begin with an E-MBS MAC header. The header may be followed by one or more E-MBS extended headers. The E-MBS MAC PDU may also contain payload.

Multiple MAC SDUs and/or SDU fragments from one or more E-MBS service flows can be multiplexed into a single E-MBS MAC PDU. The multiplexed E-MBS service flows shall be associated with the same security association.

15.x.x.1 E-MBS MAC header formats

E-MBS compact MAC header is defined as E-MBS MAC header.

15.x.x.1.1 E-MBS compact MAC header(E-CMH)

The E-CMH format is defined in Table 654. Its fields are defined in 655.

15.x.x.2 E-MBS extended header formats

The existence of E-MBS Extended Header is indicated by EH bit in E-MBS MAC Header. The Extended header format is defined in Table 657 and will be used unless specified otherwise. And the type field is 2 bit in size. The E-MBS extended header types are defined in table x1.

Table x1—Description of E-MBS Extended Header Types

| E-MBS Extended Header Types | Names | Description |
| --- | --- | --- |
| 00 | E-MBS fragmentation and packing extended header | See 15.x.x.x2.1 |
| 01 | E-MBS multiplexing extended header | See 15.x.x.x2.2 |
| 10 | Reserved |  |
| 11 | Reserved |  |

15.x.x.2.1 E-MBS fragmentation and packing extended header(E-FPEH)

The E-FPEH shall be used when MAC PDU contains payload from single E-MBS service flow. The E-FPEH format is defined in Table x2 and its fields are defined in Table x3.

Table x2—E-FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| E-FPEH() { |  |  |
| SN | 10 | Payload sequence number. |
| FC | 2 | Fragmentation Control(see P802.16m/D1 Table 661). |
| Do{ |  |  |
| End | 1 |  |
| If(End==0){ |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } | 1 |  |
| } while(!End) |  |  |
| Reserved | variable |  |
| } |  |  |

Table x3—E-FPEH fields

| Name | Length (bits) | Description |
| --- | --- | --- |
| FC | 2 | Fragmentation Control bits (encoding shown in P802.16m/D1 Table 661) |
| SN | 10 | Sequence number for the payload identified in the E-MEHB |
| Length | 11 | Length of the correspondent SDU/SDU fragment |
| End | 1 | Indication of more information 0 = Indication another “Length” and “End” fields are followed 1 = Indication no more “Length” and “End” fields are followed |
| Reserved | variable | Reserved bits are added at the end of FPEH for byte alignment. |

15.x.x.2.2 E-MBS multiplexing extended header(E-MEH)

The format of E-MEH is defined in Table x4. It is used when SDUs/SDUs fragments from different E-MBS service flows are multiplexed in the same MPDU. Each E-MEH contains multiple E-MBS Multiplexing Extended Header Blocks (E-MEHBs). The SDUs or SDU fragments belonging to the same E-MBS service flow are packed together and the information related to these SDUs or SDU fragments is included in one E-MEHB. The fields of E-MEH are defined in Table x5. The M bit in E-MEHB indicates if there is more E-MEHB followed.

Table x4—E-MEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| E-MEH() { |  |  |
| Last | 1 | 0 = Another E-MBS extended header follows this E-MEH; 1 = This is the last E-MBS extended header. |
| Type | 4 | E-MEH type |
| Do{ |  |  |
| E-MEHB(M) |  | E-MBS multiplexing extended block |
| } while(!M) |  |  |
| Reserved | variable |  |
| } |  |  |

Table x5—E-MEH Fields

| Name | Length (bits) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended header indicator |
| Type | 2 | Extended header type |
| E-MEHB | variable | E-MBS multiplexing extended header block(format shown in table x6) |

The format of E-MEHB is shown in Table x6, except the first E-MEHB.

Table x6—E-MEHB Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| E-MEHB() { |  |  |
| M | 1 | 0=Another E-MEHB follows this E-MEHB; 1=This is the last E-MEHB in E-MEH. |
| FC | 2 | Fragmentation Control(see P802.16m/D1 Table 661). |
| SN | 10 | Payload sequence number. |
| Do{ |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| End | 1 |  |
| } while(!End) |  |  |
| } |  |  |

The format of the first E-MEHB is shown in Table x7. The first E-MEHB doesn't contain the length for the first SDU/SDU fragment associated with the E-MBS service flow. The Length fields in the E-MBS compact MAC header represent the length of the first SDU/SDU fragment associated with the first E-MEHB.

Table x7—First E-MEHB Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| First E-MEHB () { |  |  |
| M | 1 | 0=Another E-MEHB follows this E-MEHB; 1=This is the last E-MEHB in E-MEH. |
| FC | 2 | Fragmentation Control(see P802.16m/D1 Table 661). |
| SN | 10 | Payload sequence number. |
| Do{ |  |  |
| End | 1 |  |
| If(End==0){ |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } |  |  |
| }while(!End) |  |  |
| } |  |  |

The fields of E-MEHB are defined in Table x8 .

Table x8—E-MEHB Fields

| Name | Length (bits) | Description |
| --- | --- | --- |
| M | 1 | Indication of more E-MEHB 0 = no more E-MEHB follows the current E-MEHB; 1 = one or more MEHB follows the current MEHB. |
| FC | 2 | Fragmentation Control bits (encoding shown in P802.16m/D1 Table 661) |
| SN | 10 | Sequence number for the payload identified in the E-MEHB |
| Length | 11 | Length of the correspondent SDU/SDU fragment |
| End | 1 | Indication of more information 0 = Indication another “Length” and “End” fields are followed 1 = Indication no more “Length” and “End” fields are followed |

# Reference

[1] IEEE P802.16m/D1

[2] IEEE 802.16m-08_0034, “802.16m System Description Document (SDD)”