| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes to PFBCH Encoding Type 1 of UL Control in IEEE P802.16m/D1 |  |
| Date Submitted | 2009-08-27 |  |
| Source(s) | Sangheon Kim, David Mazzarese Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes editorial correction to the PFBCH encoding type 1 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial changes to PFBCH Encoding Type 1 of UL Control in IEEE P802.16m/D1

Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

Table 743 in the section 15.3.9.3.1.1 in [1] specifies the Encoding Type 1 in PFBCH. However, the 63th row for event-driven indicator for bandwidth request indicator (sequence 2) is missed out in the table. This contribution provides correction to this table.

Proposed text has been underlined in blue, text that has been deleted has been marked in red and struck through and existing text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Proposed Amendment Text Change

<Modify Table 743 in the section 15.3.9.3.1.1, on line 2, page 448 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 6 bits of information, providing wideband channel quality feedback, and MIMO feedback.

Table 741 PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI | 0, 1, 2, 3, 4, 5, 6, 7 | 1) Wideband CQI 2) Subband CQI for Best -1 subband |
| STC Rate Indicator | 0, 1, 2, 3 |  |
| Subband index | 2, 3, 5, 6 | Subband selection for best-1 subband |
| PMI | 3, 4, 6, 7 | - wideband PMI - subband PMI for best-1 subband |
| Event-driven Indicator (EDI) for request for switching MFM | N/A | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| Event-driven Indicator (EDI) for Bandwidth Request Indicator | N/A | This is used to request UL bandwidth. 2 sequences (two services) |
| Event-driven Indicator (EDI) for Frequency partition selection (FPS) | N/A | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| Event-driven Indicator (EDI) for Buffer management | N/A | Indicates occupancy status of HARQ soft buffer |

For PFBCH transmission, three encoding types are defined. Encoding type corresponding to MFM and feedback format in Feedback Allocation A-MAP IE is used.

# PFBCH Encoding Type 0

Table 742 - Contents Encoding Type 0 in PFBCH

| Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | STC rate = 1, MCS=0000 |  |
| 1 | STC rate = 1, MCS=0001 |  |
| 2 | STC rate =1, MCS=0010 |  |
| 3 | STC rate =1, MCS=0011 |  |
| 4 | STC rate =1, MCS=0100 |  |
| 5 | STC rate =1, MCS=0101 |  |
| 6 | STC rate =1, MCS=0110 |  |
| 7 | STC rate =1, MCS=0111 |  |
| 8 | STC rate =1, MCS=1000 |  |
| 9 | STC rate =1, MCS=1001 |  |
| 10 | STC rate =1, MCS=1010 |  |
| 11 | STC rate =1, MCS=1011 |  |
| 12 | STC rate =1, MCS=1100 |  |
| 13 | STC rate =1, MCS=1101 |  |
| 14 | STC rate =1, MCS=1110 |  |
| 15 | STC rate =1, MCS=1111 |  |
| 16 | STC rate =2, MCS=0000 |  |
| 17 | STC rate =2, MCS=0001 |  |
| 18 | STC rate =2, MCS=0010 |  |
| 19 | STC rate =2, MCS=0011 |  |
| 20 | STC rate =2, MCS=0100 |  |
| 21 | STC rate =2, MCS=0101 |  |
| 22 | STC rate =2, MCS=0110 |  |
| 23 | STC rate =2, MCS=0111 |  |
| 24 | STC rate =2, MCS=1000 |  |
| 25 | STC rate =2, MCS=1001 |  |
| 26 | STC rate =2, MCS=1010 |  |
| 27 | STC rate =2, MCS=1011 |  |
| 28 | STC rate =2, MCS=1100 |  |
| 29 | STC rate =2, MCS=1101 |  |
| 30 | STC rate =2, MCS=1110 |  |
| 31 | STC rate =2, MCS=1111 |  |
| 32 | STC rate =3, MCS=0100 |  |
| 33 | STC rate =3, MCS=0101 |  |
| 34 | STC rate =3, MCS=0110 |  |
| 35 | STC rate =3, MCS=0111 |  |
| 36 | STC rate =3, MCS=1000 |  |
| 37 | STC rate =3, MCS=1001 |  |
| 38 | STC rate =3, MCS=1010 |  |
| 39 | STC rate =3, MCS=1011 |  |
| 40 | STC rate =3, MCS=1100 |  |
| 41 | STC rate =3, MCS=1101 |  |
| 42 | STC rate =3, MCS=1110 |  |
| 43 | STC rate =3, MCS=1111 |  |
| 44 | STC rate =4, MCS=1000 |  |
| 45 | STC rate =4, MCS=1001 |  |
| 46 | STC rate =4, MCS=1010 |  |
| 47 | STC rate =4, MCS=1011 |  |
| 48 | STC rate =4, MCS=1100 |  |
| 49 | STC rate =4, MCS=1101 |  |
| 50 | STC rate =4, MCS=1110 |  |
| 51 | STC rate =4, MCS=1111 |  |
| 52 | Reserved |  |
| 53 | Reserved |  |
| 54 | Reserved |  |
| 55 | Event-driven Indicator (EDI) for Buffer management (80% full) | Event-driven for buffer management |
| 56 | Event-driven Indicator (EDI) for Buffer management (full) |  |
| 57 | Event-driven Indicator (EDI) for request for switching MFM | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| 58 | Event driven indicator (EDI) for frequency partition 0 indication (reuse-1) | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| 59 | Event driven indicator (EDI) for frequency partition 1 indication (reuse-3) |  |
| 60 | Event driven indicator (EDI) for frequency partition 2 indication (reuse-3) |  |
| 61 | Event driven indicator (EDI) for frequency partition 3 indication (reuse-3) |  |
| 62 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 1) | Event-driven Indicator for Bandwidth request |
| 63 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 2) |  |

# PFBCH Encoding Type 1

Table 743 - Contents Encoding Type 1 in PFBCH

| Sequence Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | Subband index 0 | Subband index for Best-1 subband (refer 15.3.9.3.1.4 feedback format) |
| 1 | Subband index 1 |  |
| 2 | Subband index 2 |  |
| 3 | Subband index 3 |  |
| 4 | Subband index 4 |  |
| 5 | Subband index 5 |  |
| 6 | Subband index 6 |  |
| 7 | Subband index 7 |  |
| 8 | Subband index 8 |  |
| 9 | Subband index 9 |  |
| 10 | Subband index 10 |  |
| 11 | Subband index 11 |  |
| 12 | Reserved | Reserved |
| … | … |  |
| 54 | Reserved |  |
| 55 | Event-driven Indicator (EDI) for Buffer management (80% full) | Event-driven for buffer management |
| 56 | Event-driven Indicator (EDI) for Buffer management (full) |  |
| 57 | Event-driven Indicator (EDI) for request for switching MFM | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| 58 | Event driven indicator (EDI) for frequency partition 0 indication (reuse-1) | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| 59 | Event driven indicator (EDI) for frequency partition 1 indication (reuse-3) |  |
| 60 | Event driven indicator (EDI) for frequency partition 2 indication (reuse-3) |  |
| 61 | Event driven indicator (EDI) for frequency partition 3 indication (reuse-3) |  |
| 62 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 1) | Event-driven Indicator for Bandwidth request |
| 63 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 2) |  |

------------------------------------------- End of the Proposed Text --------------------------------------------