| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes to the text on feedback formats of UL Control in IEEE P802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes editorial corrections to the feedback formats of UL control |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial changes to the text on feedback formats of UL Control in IEEE P802.16m/D1

Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

Feedback formats of UL control are specified in Section 15.3.9.3.1.4 in [1]. In the description, however, table number is incorrectly mentioned, and the title of table 749 and multiplication signs in some tables are missed out. This contribution provides corrections to text for feedback format.

Proposed text has been underlined in blue, text that has been deleted has been marked in red and struck through and existing text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Proposed Amendment Text Change

<Modify the section 15.3.9.3.1.4 on line 36, page 450 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table xx 745. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 745 - Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 1 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 4 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |
| 7 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |

# Feedback format for MFM 2

The detailed format is listed in Table 727 746. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 746 - Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | N/A | Joint encoding of CQI and STC rate with PFBCH PFBCH encoding Type 0 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Subband index Subband CQI STC rate PFBCH indicator |  | 3, 4, or 5 4 1~3 1 | Subband index for 5, 10, or 20MHz Support of STC rate 1 to 8 |
| 2 (M = 3) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 23 = 6 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 5, 8 or 11 1~3 1 |  |
| 3 (M = 5) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 25 = 10 |  |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 3, 10, or 16 1~3 1 |  |

# Feedback format for MFM 3

The detailed format is listed in Table 728 747. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. For feedback format 0 with PFBCH, subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 747 - Feedback formats for MIMO feedback mode 3

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 STC rate = 1 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 7 or 9 1~3 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 5, 8 or 11 1~3 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 9 or 14 1~3 4 1 |  |

Table 748 - Feedback formats for MIMO feedback mode 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2-4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband index STC rate Base PMI CQI PFBCH indicator | 3, 4 or 5 1 to 3 3-6 4 1 | Subband index for 5, 10, or 20MHz PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 1~3 4 2×3~2×6=6~12 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 8 or 10 1~3 4 3×3~3×6=9~18 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 1~3 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

# Feedback format for MFM 5

The detailed format is listed inTable 729 749. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 749 - Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Short | Subband index Subband CQI Stream Index PFBCH indicator | 3, 4, or 5 4 1~2 1 | Subband index for 5, 10, or 20MHz |
| 1 (M=2) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 22=4 (1~2) 2 |  |
|  |  |  | Long | Subband index PFBCH indicator | 4, 7, or 9 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 23=6 3 (1~2) |  |
|  |  |  | Long | Subband index PFBCH indicator | 5, 8, or 11 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =5) | SFBCH | 3 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 25=10 (1~2) 5 |  |
|  |  |  | Long | Subband index PFBCH indicator | 3, 10, or 16 1 | Subband index for 5, 10, or 20MHz |

# Feedback format for MFM 6

The detailed format is listed in Table xxx 750. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. For feedback format 0 with PFBCH, subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 750 - Feedback formats for MIMO feedback mode 6

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 7 or 9 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 5, 8 or 11 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 9 or 14 4 1 |  |

Table 751 - Feedback formats for MIMO feedback mode 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2~4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband selection Base PMI CQI PFBCH indicator | 3, 4 or 5 3-6 4 1 | PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 4 2×3~2×6=6~12 1 |  |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 5, 8 or 10 4 3×3~3×6=9~18 1 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------