| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose text for the SFH IE (P802.16M/D1 – 15.3.6.5.3.2) |  |
| Date Submitted | 2009-08-27 |  |
| Source(s) | Sassan Ahmadi, Shantidev Mohanty, Yuan Zhu Intel | Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | IEEE 802.16m-09/0028r1: “802.16m/D1 letter ballot document” Target topic: P802.16M/D1 - 15.3.6.5.3.2 |  |
| Abstract | This contribution proposes fix to erroneously incorporated comments during IEEE 802.16m meeting in July 2009. |  |
| Purpose | To be adopted by TGm for the P802.16m/D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Propose text for the SFH-IE (P802.16M/D1 – 15.3.6.5.3.2)

Sassan Ahmadi, Shantidev Mohanty

Intel

- Introduction

All the proposed text of the contributions IEEE C802.16m-09/1429r3 and IEEE C802.16m-09/1450r2 that are adopted during IEEE 802.16 meeting during July 2009 are not incorporated in to P802.16m/D1. This contribution provides text proposals to fix the missing parts.

- References

[1] IEEE P802.16m/D1, “IEEE 802.16m Amendment Letter Ballot Document” (P802.16m/D1)

- Text proposal

[Note to Editor: Suggested Remedy#1 Replace Table 674 in 15.3.6.5.3.2 in page 319 as follows]

Table 674 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit BS MAC ID | 12 | Specifies the 12 least bit of BS ID |
| UL Ack/Nack channel resource size | 2 |  |
| DL Ack/Nack channel resource size | 2 |  |
| Power control channel resource size indicator | 2 |  |
| Non-user specific AMAP location | 1 | Reuse 1 or reuse 3 |
| A-A-MAP MCS selection | 1 |  |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20MHz, DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) |
| Unsync ranging allocation interval channel information (ranging region periodicity) | 3 |  |
| Unsync ranging location in the frame (time and frequency) | 2 |  |
| RNG codes information | 7 |  |
| Ranging code subset/partition configuration | 3 |  |
| BS EIRP | 7 |  |
| Cell bar information | 1 |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| Reserved | TBD |  |
| } |  |  |

[Note to Editor: Suggested Remedy#2 Replace Table 675 in 15.3.6.5.3.2 in page 320 as follows]

Table 675 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Duplexing mode | 1 | 0b0: TDD 0b1: FDD |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 7 |  |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency offset | 6 |  |
| UL bandwidth | 3 |  |
| } |  |  |
| MSB bytes of BSID | 36 | Specifies 36 MSB of BSID |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | 12 | For 20MHz, DL_SAC(5), DL_FPSC(3), DL_FPC(4) For 5MHz, DL_SAC(3), DL_FPSC(1), DL_FPC(3) |
| FFR partitioning info for UL region | 12 | For 20MHz, UL_SAC(5), UL_FPSC(3), UL_FPC(4) For 5MHz, UL_SAC(3), UL_FPSC(1), UL_FPC(3) |
| MS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| (EIRPIRmin) | 5 |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| reserved |  |  |
| } |  |  |

[Note to Editor: Suggested Remedy#3 Replace Table 676 in 15.3.6.5.3.2 in page 322 as follows]

Table 676 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| UL Fast FB Size | 4 | Specifies the size of UL feedback channel per a UL subframe; (Need the decision from UL Ctrl section) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| Default RSSI and CINR averaging parameter | 3 | Default averaging parameter for physical CINR measurements and RSSI measurements, in multiple of 1/16 |
| SP scheduling periodicity information | TBD |  |
| HO Ranging backoff start | 4 |  |
| HO Ranging backoff end | 4 |  |
| Initial ranging backoff start | 4 |  |
| Initial ranging backoff end | 4 |  |
| UL BWREQ Channel information | 3 |  |
| Bandwidth request backoff start | 4 |  |
| Bandwidth request backoff end | 4 |  |
| Uplink subframe bitmap for sounding | 8 |  |
| Sounding Multiplexing Type (SMT) for sounding | 1 |  |
| Decimation value D/Max Cyclic Shift Index P for sounding | 3 |  |
| Start super-frame offset where new SP3 information is used | 2 |  |
| Rate of change of SP (1-3) info | 4 |  |
| SA-sequence soft partitioning information | 4 |  |
| FFR Partition resource Metrics | TBD |  |
| MIMO rank 1 OL region signaling | TBD |  |
| MIMO rank 1 OL region | TBD |  |
| NI information for UL power control | TBD |  |
| Reserved | TBD |  |
| } |  |  |

--------------------------------  Text End  ---------------------------------------------------