| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to DL Pilot Patterns Boosting Values (15.3.5.4.1) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009- |  |
| Source(s) | Alexei Davydov, Jong-Kae (JK) Fwu, Huaning Niu Intel Corporation | jong-kae.fwu@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.5 (DL Physical Structure) “Comments on AWD 15.3.5 (DL Physical Structure)” |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D1 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to DL Pilot Patterns (15.3.5.4.1) for the IEEE 802.16m/D1

Alexei Davydov, Jong-Kae (JK) Fwu, Huaning Niu

Intel Corporation

- Introduction

The pilot boosting value in the current 802.16m D1 draft text is missing and should be specified. We had proposed to set the following values in the past: (1) 1 stream of 2 dB boosting (2dB net boosting); (2) 2 stream-interlaced pilot boosting value to 5 dB (i.e. net 2dB boost); (3) 4 stream pilot to 6 dB (0 dB net boosting) based on our previous simulation results.  However, members requested some more time to evaluate the boosting value themselves. The following texts are proposed to provide the missing details associated with pilot boosting definitions and boosting values.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [  ------ Recommended AWD Text Proposal (Section 15.3.5.4.1, page 282, line 44) = insert the following text after Figure 422 ------]

The average power for data and pilot subcarriers shall be set according to the number of data streams Mt as indicated in Table X and in Table Y for downlink and uplink transmissions respectively. The per stream pilot boosting is defined as power of pilot subcarrier relative to average power of data subcarrier on corresponding data stream.

Table X: Downlink power settings for data and pilot subcarriers for different number of data streams

| number of data streams Mt | per stream power on data subcarriers | per stream pilot power | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1-2 | 1 / Mt | 1.5849 | 2 + 10*log10(Mt) |
| 3-4 |  | 1 | 0 + 10*log10(Mt) |
| 5-8 |  | 1 | 0 + 10*log10(Mt) |

Table Y: Uplink power settings for data and pilot subcarriers for different number of data streams

| number of data streams Mt | per stream power on data subcarriers | per stream pilot power | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1-2 | 1 / Mt | 1 | 0 + 10*log10(Mt) |
| 3-4 |  | 1 | 0 + 10*log10(Mt) |

For UL CSM (collaborative spatial multiplexing) mode, the pilot subcarriers shall be further amplified by TNSMt, where TNS is a total number of streams in CSM mode.

-------------------------------------- End of Text Proposal -------------------------------------------