| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on coverage loss (section 15.2.x and 15.2.6.x) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Chunmei Liu, Masoud Olfat Clearwire Kelvin Chou MediaTek Inc. Jin Lee, Kiseon Ryu, Ronny Kim, Jinsam Kwak LGE | Chunmei.liu@clearwire.com Masoud Olfat@clearwire.com Kelvin.Chou@mediatek.com {jins978, ksryu, ronnykim}@lge.com |
| Re: | Proposed text on coverage loss |  |
| Abstract | Proposed text on coverage loss in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on Coverage Loss

Chunmei Liu and Masoud Olfat

Clearwire

Kelvin Chou

MediaTek Inc.

Jin Lee, Kiseon Ryu, Ronny Kim, Jinsam Kwak

LGE

# Introduction

Coverage losses are common in practical network deployments. The device may lose signal temporarily due to various reasons, such as entering into an area without coverage, fading, etc.  According to the current spec, upon a coverage loss, the MS will lose PHY synchronization or DL/UL synchronization. In addition, if the periodic ranging procedure is conducted, the ranging procedure may fail.

However, there is a lack of sufficient specs for addressing this problem at MAC layer. First, the standard does not synchronize the network and the MS regarding each other’s state, including link status and MS’s context retained at the network. Actually there is no mechanism specified for the BS to detect an MS’s coverage loss.

In addition, the standard does not specify the network’s behavior after an MS’s coverage loss either.  Note that in practical deployments, it may be desirable for the network to release MS’s context in order to free resource occupied.

Furthermore, there is ambiguity in the previous standards, which are the base of 16m AWD, on coverage recovery procedure after the MS comes back into coverage, and this ambiguity may lead to significant application performance downgrade. For example, in 802-16 2009, after incurring a coverage loss, a MS will scan for a new channel, or reinitializes MAC.  However, the spec does not clearly define what follows channel scanning and what it means by “reinitializes MAC”. In C80216maint-09_0014r1.doc, after a coverage loss, a MS may perform initial network entry or network reentry.

The proposed text below addresses this coverage loss problem with the necessary clarity by providing solutions to maintain and synchronize state information between a MS and the network, and coverage loss recovery procedure.

# Proposed Amendment text

Black text: existing text in AWD (P802.16m/D1)

Red Strikethrough text: deleted text

Blue text: new text

[Insert the below new section]

15.2.x Coverage loss

An AMS may lose signal temporarily due to various reasons, such as entering into an area without coverage and fading. A coverage loss refers to such a situation.

Upon a coverage loss, the AMS and the ABS should be synchronized on each other’s state, including link status and AMS’s context retained at the network.

15.2.x.1 State synchronization and coverage loss detection at ABS

For state synchronization purpose, for each AMS, the ABS shall maintain a timer called active_ABS_timer.  The initial value of the active_ABS_timer is determined through AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry procedure.  The timer starts upon the completion of the AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry.

Upon sending out a DL HARQ ACK or receiving an UL HARQ ACK, for either a unicast MAC management message or a data packet, which indicates a correct reception, the ABS shall restart the active ABS timer.

Upon expiration of the active_ABS_timer, the ABS shall start the Resource Retain Time and send unsolicited AAI-RNG-RSP message to request the AMS to perform ranging using periodic ranging codes by setting the Ranging Request bit to one. The initial value of the Resource Retain Time is determined through AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry procedure, and should be larger than the typical duration required for a successful ranging and allow another ABS to retrieve this AMS’s context in case the AMS performs HO to another ABS.

When the ABS receives a periodic ranging code that requires no correction, the ABS shall send an AAI-RNG-ACK message with success status together with a CDMA Allocation A-MAP IE for the AMS. Upon receiving the CDMA Allocation A-MAP IE, the AMS shall send AAI-RNG-CFM message that includes AMS’s STID and the RA-ID carried by the CDMA Allocation A-MAP IE.

Upon receiving an AAI-RNG-CFM message with AMS’s STID, the ABS shall restart the active_ABS_timer and cancels the Resource Retain Time if the Resource Retain Time was started.

If the ABS receives backbone context request for the AMS after sending the unsolicited AAI-RNG-RSP message, the ABS considers that the AMS is performing HO.  Accordingly, the ABS cancels the retries for the unsolicited AAI-RNG-RSP message and restarts the Resource Retain Time.

Upon expiration of the Resource Retain Time, the ABS considers that the AMS is not in the network anymore, and releases AMS’s context.

15.2.x.2 State synchronization and coverage loss detection at AMS

The AMS can detect a coverage loss by losing PHY synchronization or DL synchronization or UL synchronization. In addition, if the periodic ranging procedure is conducted, the AMS can detect a loss by a failure ranging.

For state synchronization purpose, the AMS shall maintain a timer called active_AMS_timer. Its value is the sum of the active_ABS_timer and the Resource Retain Time.  The active_AMS_timer starts upon the completion of the AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry.

Upon sending out an UL HARQ ACK or receiving a DL HARQ ACK, for either a unicast MAC management message or a data packet, which indicates a correct reception, the AMS shall restart the active AMS timer.

Upon each successful periodic ranging and ranging initiated by an unsolicited AAI-RNG-RSP message, indicated by receiving an AAI-RNG-ACK message with a success status and corresponding periodic ranging code, the AMS shall send back a AAI-RNG-CFM message that includes its STID and corresponding RA-ID.  Upon sending out the AAI-RNG-CFM message, the AMS shall restart the active_AMS_timer.

If the AMS is in the network and intends to retain its context, the AMS shall send an AAI_RNG-REQ ahead of the expiration of the active_AMS_timer. Upon receiving the AAI_RNG-REQ, ABS shall respond with an AAI_RNG-RSP message. Upon receiving the AAI-RNG-RSP message, the AMS shall restart the active_AMS_timer.

Upon expiration of the active_AMS_timer, the AMS considers that the network has released its context.

15.2.x.3 Coverage loss recovery procedure

Upon detection of a coverage loss, the AMS scans for a new channel. After achieving PHY synchronization and DL synchronization with the discovered ABS, which could be its previous serving ABS before the coverage loss, if the active_AMS_timer has expired, the AMS considers that the network does not have its context anymore, and performs initial network entry as indicated in section 15.3.10. Otherwise, the AMS considers that the network still has its context, and performs network reentry as indicated below.

During the network re-entry, the AMS shall perform CDMA ranging using HO ranging codes. Upon the discovered ABS’s sending AAI-RNG-ACK with success status, the ABS shall provide UL BW allocation. When receiving an UL BW allocation, the AMS shall send the AAI_RNG-REQ message with its previous STID and CMAC, and set bit#5 in the Ranging Purpose Indication parameter, indicating that this is a network reentry after a coverage loss. If the discovered ABS is the AMS’s previous serving ABS, the AMS should not include the serving BSID parameter in the AAI-RNG-REQ message; otherwise, the AMS shall include the serving BSID parameter and set it to be the BSID of its previous serving ABS.

After receiving the AAI-RNG-REQ message, the discovered ABS identifies AMS’s reentry attempt after a coverage loss and checks its context availability.  If the discovered ABS is the AMS’s previous serving ABS, it has the AMS’s context information.  Otherwise, the discovered ABS may request relevant AMS’s context from its previous serving ABS, indicated in the AAI-RNG-REQ message by the serving BSID parameter.

Based on AMS’s relevant context retained at the network, there are three outcomes:

- If the network has all relevant context, the discovered ABS sends AAI-RNG-RSP with the Message Skip Response parameter set to be 0b10, indicating skipping all the rest of the reentry process management messages.

- If the network has partial context, the discovered ABS sends AAI-RNG-RSP with the Message Skip Response parameter set to be 0b01, indicating skipping some of the reentry process management messages, and corresponding HO Process Optimization parameter.

- If the network does not have the AMS’s context, the ABS sends AAI-RNG-RSP with the Message Skip Response parameter set to be 0b00, indicating no messages can be skipped.

Accordingly, after receiving the AAI-RNG-RSP message, the AMS resumes normal operation with the discovered ABS immediately, performs optimized network reentry, or performs full network reentry, respectively.

[Insert the below new section]

15.2.6.31 AAI-RNG-CFM

The AMS shall send the AAI-RNG-CFM message to the ABS upon each successful periodic ranging and ranging initiated by an unsolicited AAI-RANG-RSP message. It shall include the below parameters:

- STID of the AMS.

- RA-ID corresponding to the successful ranging.

[Change some entries of Table 682 in 15.2.6.1, as follows]

Table 682—parameters for AAI_ RNG-REQ

| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#5 is set to 1, it indicates that the AMS is currently attempting network reentry after experiencing a coverage loss. Bit#6-75-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| --- | --- | --- |
| Serving BSID | The BSID of the AMS’s previous serving ABS before incurring a coverage loss, or tThe BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry, or a network reentry after a coverage loss when the discovered ABS is not the AMS’s previous serving ABS before incurring the coverage loss. In the latter case, it shall be set to be the BSID of the AMS’s previous serving ABS before incurring the coverage loss. |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry or a reentry after incurring a coverage loss |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, or a reentry after incurring a coverage loss, if the AMS has a CMAC tuple necessary to expedite security authentication. |

[Change some entries of Table 683 in 15.2.6.2 and insert some new entries, as follows]

Table 683—parameters for AAI_ RNG-RSPEQ

| HO Process Optimization | Identifies reentry process management messages that may be omitted during the current HO attempt or network reentry attempt after a coverage loss due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC management message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process management messages that may be omitted during the current HO attempt. It also shall be included when the Message Skip Response parameter is set to be 0b01. |
| --- | --- | --- |
| Message Skip Response | The presence of this item in the message indicates the following AMS action: 0b00: none of the reentry process management messages can be skipped. 0b01: partial of the reentry process management messages can be skipped. The messages that can be skipped are indicated in the HO Process Optimization item. 0b10: all the reentry process management messages can be skipped. Other values: reserved. | It shall be included when the AMS is attempting to perform a network reentry after experiencing a coverage loss and the discovered ABS wishes to identify if some of the reentry process management messages can be omitted |