| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Action Code reuse of AAI_DREG-CMD (P802.16/D1-15.2.6.20) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Lei Zhang Nan Li Yang Liu ZTE Corporation Hyunjeong Kang Samsung Electronics Ming-Hung Tao Ying-Chuan Hsiao ITRI Shantidev Mohanty, Muthaiah Venkatachalam Intel Corporation | zhang.lei13@zte.com.cn hyunjeong.kang@samsung.com mhtao@itri.org.tw |
| Re: | Category: LB30 / Area: Chapter 15.2.6.20 (AAI_DREG-CMD message) |  |
| Abstract | This contribution proposes changes to the Section 15.2.6.20 of IEEE P802.16m/D1 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Action Code reuse of AAI_DREG-CMD (P802.16/D1-15.2.6.20)

Lei Zhang, Nan Li, Yang Liu

ZTE Corporation

Hyunjeong Kang

Samsung Electronics

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Shantidev Mohanty, Muthaiah Venkatachalam

Intel Corporation

- Introduction

In P802.16m/D1, action code 0x05 in AAI_DREG-CMD is used by ABS to allow AMS-initiated Idle Mode request or signal AMS to begin Idle Mode in unsolicited manner. However, if ABS-initiated request (i.e. Unsolicited DREG-CMD) and an AMS-initiated request for Idle Mode entry are handled concurrently, the AMS cannot distinguish the exact purpose of action code 0x05.

For example, when ABS sends an unsolicited AAI_DREG-CMD with 0x05 to initiate Idle Mode, at the same time, the AMS also sends AAI_DREG-REQ with 0x01 to request entering Idle Mode. In this case, when receiving the AAI_DREG-CMD, the AMS may not distinguish whether it is ABS's Idle Mode initiation request or just a response to AAI_DREG-REQ.

For the reason mentioned above, we propose to instead 0x05 with 0x07 to be the response to AMS's AAI_DREG-REQ message.

- Proposed text

======================== Start of Proposed Text ========================

[Modify the paragraph (in line 35 on page 72) of P802.16m/D1 as follows]

- 15.2.6.20 AAI_DREG-CMD message

0x05: AMS shall begin idle mode initiation: a) to allow AMS-initiated idle mode request or b) a) to signal AMS to begin idle mode in unsolicited manner or c)b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration

[Modify the sentence (in line 43 on page 72) of P802.16m/D1 as follows]

0x07: reserved to allow AMS-initiated idle mode request. This option is valid in response to a AAI_DREG-REQ message with De-Registration Code 0x01.

0x08-0x15: reserved

[Modify the sentence (in line 45 on page 72) of P802.16m/D1 as follows]

When the action code is set to 0x05 or 0x07, the following parameters shall be included in AAI_DREG-CMD message.

[Modify the sentence (in line 64 on page 72) of P802.16m/D1 as follows]

When the action code is set to 0x05, or 0x06 or 0x07, the following parameter may be included in the AAI_DREG-CMD message.

[Modify the paragraph (line 48 on page 202) of P802.16m/D1 as follows]

- 15.3.12.1.1 AMS initiated

When the ABS decides to allow AMS-initiated idle mode request, the ABS shall send a AAI_DREG-CMD with action code 0x05 0x07 in response to the AAI_DREG-REQ message.

[Replace Figure 442(on page 203) of P802.16m/D1 with the following figure]

======================== End of Proposed Text ========================