| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Legacy support for Idle Mode (P802.16/D1-15.3.12) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Lei Zhang Mary Chion Hongyun Qu Yang Liu ZTE Corporation Hyunjeong Kang Samsung Electronics Ming-Hung Tao Ying-Chuan Hsiao ITRI | zhang.lei13@zte.com.cn hyunjeong.kang@samsung.com mhtao@itri.org.tw |
| Re: | Category: LB30 / Area: Chapter 15.3.12 (Idle Mode) |  |
| Abstract | This contribution proposes changes to the Section 15.3.12 of IEEE P802.16m/D1 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Legacy support for Idle Mode (P802.16/D1-15.3.12)

Lei Zhang, Mary Chion, Hongyun Qu, Yang Liu

ZTE Corporation

Hyunjeong Kang

Samsung Electronics

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

As shown in Figure 1 below, a legacy supported network may compose of YBS, mixed mode ABS and 16m only BS. The Paging Group of networks is always divided based on BSs’ geographical position, not operation mode (Legacy/IEEE 802.16m). For this reason, LZone and MZone of a mixed mode ABS shall share the same PGID of the ABS. Accordingly, if YBS or 16m only BS is setup close to a mixed ABS, there must be a great chance that we divide them into the same Paging Group with same PGID.

Figure 1 Example of legacy supported network

An AMS in Idle Mode may move across the boundaries of BSs operating in different mode (Legacy/ IEEE 802.16m mode) and perform cell reselection of preferred BS, in which case the following scenarios may happen in a legacy supported network:

- AMS moves from MZone of a mixed mode ABS to a neighbor YBS.

- AMS switches from LZone to MZone of a mixed ABS.

- AMS moves from LZone of a mixed mode ABS to a neighbor 16m only BS.

- AMS moves between YBS and neighbor 16m only BS.

- Problem and solution

As defined in P802.16m/D1, an Idle Mode AMS shall perform Location Update to update paging information when switching from LZone of a mixed mode ABS to the MZone of the ABS.

However, in 16m, it is defined that Temp ID is used to identify an AMS during Location Update. But, when switching from LZone to MZone during Idle Mode, the AMS will not have Temp ID to be used in RNG-REQ. In addition, if an Idle Mode AMS changes its operation mode, the paging controller and AMS both need to update 16m paging information and AMS content, such as security information. Therefore the AMS may not work well by only performing secure Location Update.

Instead of secure Location Update in the scenarios mentioned before, we propose that the AMS shall perform network re-entry procedure from Idle Mode when moving between BSs operating in different mode or switching from LZone to MZone of a mixed mode ABS.

- Proposed text

[Modify the paragraph at lines 65 on page 201 through line 1 on page 202 of P802.16m/D1 as follows and insert the following text to lines 1 on page 202]

======================== Start of Proposed Text ========================

- 15.3.12 Idle mode

When AMS operating in legacy mode select the mixed ABS as a preferred ABS, AMS may stay in the Lzone and perform the legacy Idle Mode operation. If an idle mode AMS in the Lzone of a mixed mode ABS decides to move to the Mzone of the ABS, in which case the AMS shall perform location update. If an AMS in Idle Mode decides to change its operation mode (Legacy/IEEE 802.16m mode), the AMS shall perform network re-entry from Idle Mode in the new operation mode. The change includes the AMS moves between BSs operating in different mode or switches from LZone to MZone of a mixed mode ABS. And the decision may be based on the detection of a new operation mode.

======================== End of Proposed Text ========================