| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HO from Macro ABS to OSG/CSG-Open Femto ABS |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | chenyunghan@itri.org.tw whsheen@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text on HO from a macro ABS to an OSG/CSG-Open Femto ABS in Section 15.4.8.1 of P802.16m/D1. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HO from Macro ABS to OSG/CSG-Open Femto ABS

Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu

ITRI

Wern Ho Sheen
Chaoyang University of Technology / ITRI

1. Introduction

This contribution provides text proposal for the handover from a macro ABS to an OSG/CSG-Open Femto ABS. The text proposal is based on the 16m handover defined per 15.2.6 in [1] and SDD [2] considering the characteristics of Femto deployment and operations. To facilitate the trigger of neighbor scanning and handover, a snooping-based method to confirm the entrance of an AMS for the Femto ABS is also suggested. The following figure shows an example of using snooping for HO from a serving macro ABS to a Femto ABS if the detected AMS’s STID is known.

2. Text Proposal

{Blue text: new added}

--------------------------------------------- Text Start ----------------------------------------------------------------

15.4 Support for Femto ABS

15.4.8 Handover (HO)

15.4.8.1 HO from Macro ABS to Femto ABS

15.4.8.1.x Handover from macro ABS to OSG/CSG-Open Femto ABS

The handover procedures of an AMS from a macro ABS to an OSG Femto ABS shall follow the same procedures defined per 15.2.7 for Advanced WirelessMAN-OFDMA System. For a CSG-Open Femto ABS, it may allow the AMS that is not a member of the CSG to hand over into the Femto cell for critical connection maintenance or emergency call. In such a case, the handover procedures shall also follow the same procedure defined per 15.2.7.3.

A serving macro ABS shall broadcast or unicast information of neighbor OSG and CSG-Open Femto ABSs to AMSs by using AAI_NBR-ADV message. The AMS may also request the accessible neighbor OSG/CSG-Open Femto ABSs’ information from the overlaid macro ABS through NBR-REQ/RSP messages. An AMS may cache this information for future handover to the specific OSG/CSG-Open Femto ABSs.

The trigger TLV defined per Table 702 shall be adopted to define the trigger conditions listed in 15.2.7.2. An AMS may also initiate neighbor scanning, reporting, and handover procedures autonomously. In additions, when the UL signal of a macro AMS can be detected autonomously by a OSG/CSG-Open Femto ABS and the detectable signal strength exceeds a pre-defined threshold, the Femto ABS may request the macro ABS to send an unsolicited AAI_SCN-RSP for the AMS to scan the OSG/CSG-Open Femto ABS. To achieve this, the Femto ABS shall report to the macro ABS that either which AMS is detected or which RU is used by a detected unknown AMS. Then the macro ABS can look up the corresponding A-MAP to identify the unknown AMS detected by the Femto ABS. This helps the AMS to be aware of its possible proximity to an OSG/CSG-Open Femto ABS and provides a trigger condition to initiate handover. The handover procedure shall be based on 15.2.6.3.

---------------------------------------------------------------Text End ------------------------------------------------------------

Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface,” July, 2009.

[2] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”