| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | chenyunghan@itri.org.tw whsheen@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text on HO from a macro ABS to a CSG-Closed Femto ABS in Section 15.4.8.1 of P802.16m/D1. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS

Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu

ITRI

Wern Ho Sheen
Chaoyang University of Technology / ITRI

1. Introduction

This contribution provides text proposal for the handover from a macro ABS to a CSG-Closed Femto ABS. The text proposal is based on the 16m handover defined per 15.2.6 in [1] and SDD [2] considering the characteristics of Femto cell/type, deployment, and operations. To facilitate the trigger of neighbor scanning and handover, a snooping-based method to confirm the entrance of an AMS for the Femto ABS is also suggested. The following figure shows an example of using snooping for HO from a serving macro ABS to a Femto ABS if the detected AMS’s STID is known.

2. Text Proposal

{Blue text: new added}

--------------------------------------------- Text Start ----------------------------------------------------------------

15.4 Support for Femto ABS

15.4.8 Handover (HO)

15.4.8.1 HO from Macro ABS to Femto ABS

15.4.8.1.x Handover from macro ABS to CSG-Closed Femto ABS

The macro ABSs shall not broadcast the system information of the neighbor CSG-Closed Femto ABSs in its neighbor list. At the time of handover preparation, the system information of a target accessible Femto ABS may be sent to the AMS upon AMS request. The AMS may also monitor the neighbor Femto ABS to obtain neighbor information.

The trigger TLV defined per Table 702 shall be adopted to define the trigger conditions listed in 15.2.7.2. An AMS may also initiate neighbor scanning, reporting, and handover procedures autonomously. In additions, when an AMS enters a macro cell in which an AMS’s CSG is overlapped with the macro cell, the macro ABS should inform the CSG Femto ABS that it may start to detect the AMS’s UL signal. Once the AMS’s UL signal is detectable by the CSG-Closed Femto ABS, the Femto ABS should report to the macro ABS, and then the macro ABS sends an unsolicited AAI_SCN-RSP for the AMS to scan the CSG-Closed Femto ABS. The aid of a CSG-Closed Femto ABS to monitor potential member AMSs will provide a trigger condition to initiate handover. The handover procedure shall be based on 15.2.6.3.

---------------------------------------------------------------Text End ------------------------------------------------------------

Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface,” July, 2009.

[2] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”