| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI Yongseouk Choi, Raunak Maheshwari, Nak Wook Sung, Namsuk Lee, Jisoo Park, Sookjin Lee ETRI | chenyunghan@itri.org.tw whsheen@itri.org.tw choiys@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text on HO from a macro ABS to a CSG-Closed Femto ABS in Section 15.4.8.1 of P802.16m/D1. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS

Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

Yongseouk Choi, Raunak Maheshwari, Nak Wook Sung, Namsuk Lee, Jisoo Park, Sookjin Lee

ETRI

1. Introduction

This contribution provides text proposal for the handover from a macro ABS to a CSG-Closed Femto ABS. The text proposal is based on the 16m handover defined per 15.2.6 in [1] and SDD [2] considering the characteristics of Femto cell/type, deployment, and operations. To facilitate the trigger of neighbor scanning and handover, a snooping-based method to confirm the entrance of an AMS for the Femto ABS is also suggested. The following figure shows the snooping process.

The following figure shows an example of using snooping for HO from a serving macro ABS to a Femto ABS if the detected AMS’s STID is known.

2. Text Proposal

{Blue text: new added}

--------------------------------------------- Text Start ----------------------------------------------------------------

15.4 Support for Femto ABS

15.4.8 Handover (HO)

15.4.8.1 HO from Macro ABS to Femto ABS

15.4.8.1.x Handover from macro ABS to CSG-Closed Femto ABS

The macro ABSs shall not broadcast the system information of the neighbor CSG-Closed Femto ABSs in its neighbor list. At the time of handover preparation, the system information of a target accessible Femto ABS may be sent to the AMS upon AMS request. The AMS may also monitor the neighbor Femto ABS to obtain neighbor information.

The trigger TLV defined per Table 702 shall be adopted to define the trigger conditions listed in 15.2.7.2. An AMS may also initiate neighbor scanning, reporting, and handover procedures autonomously. In additions, when an AMS enters a macro cell which overlaps CSG-closed Femto ABS of which AMS is a member, the macro ABS should inform suchCSG-Closed Femto ABS that it may start to monitor interference caused by UL signals from the AMS using the procedure defined below

- CSG-Closed Femto ABS knows UL A-MAP of the overlaid macro ABS

Femto ABS knows AMSs’ STIDs and their RU allocations from the UL A-MAP. Using this information the Femto ABS can decide where to monitor the member AMS served by the macro ABS. If CSG-closed Femto ABS detects such UL RUs with unknown interference above a threshold, it informs the Macro BS about them.

- CSG-Closed Femto ABS does not know the UL A-MAP of the overlaid macro ABS

Femto ABS shall inform the overlaid macro ABS about RUs which are detected with unknown interference above a threshold. The macro ABS looks up the corresponding UL A-MAP to identify the AMSs which causedinterference to the Femto ABS. Macro ABS makes a list of such AMSs which are also member of CSG-Closed Femto group.

Macro ABS transmits unsolicited AAI_SCN_RSP to inform the AMS aware of its possible proximity to a CSG-Closed Femto ABS and provides a trigger condition to initiate handover to Femto ABS.

---------------------------------------------------------------Text End ------------------------------------------------------------

Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface,” July, 2009.

[2] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”