| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | chenyunghan@itri.org.tw whsheen@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text on HO from a macro ABS to a CSG-Closed Femto ABS in Section 15.4.8.1 of P802.16m/D1. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HO from Macro ABS to CSG-Closed Femto ABS

Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

1. Introduction

This contribution provides text proposal for the handover from a macro ABS to a CSG-Closed Femto ABS. The text proposal is based on the 16m handover defined per 15.2.6 in [1] and SDD [2] considering the characteristics of Femto cell/type, deployment, and operations. To facilitate the trigger of neighbor scanning and handover, a snooping-based method to confirm the entrance of an AMS for the Femto ABS is also suggested. The following figure shows the snooping process.

The following figure shows an example of using snooping for HO from a serving macro ABS to a Femto ABS if the detected AMS’s STID is known.

2. Text Proposal

{Blue text: new added}

--------------------------------------------- Text Start ----------------------------------------------------------------

15.4 Support for Femto ABS

15.4.8 Handover (HO)

15.4.8.1 HO from Macro ABS to Femto ABS

15.4.8.1.x Handover from macro ABS to CSG-Closed Femto ABS

When an AMS enters a macro cell which overlaps CSG-closed Femto ABS of which AMS is a member, the macro ABS should inform such CSG-Closed Femto ABS that it may start to monitor UL signals with predefined Tx power from the AMS with STID provided.

When the Femto ABS obtains AMSs’ RU allocations about the UL signals at the macro ABS, the Femto ABS can decide where to monitor the member AMS served by the macro ABS. If CSG-closed Femto ABS detects the UL signal from the member AMS, it reports the information to the network.

Macro ABS transmits unsolicited AAI_SCN_RSP to request the AMS to scan the CSG-Closed Femto ABS.

---------------------------------------------------------------Text End ------------------------------------------------------------

Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface,” July, 2009.

[2] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”