| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HO from Femto ABS to Macro ABS or other Femto ABS |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | chenyunghan@itri.org.tw whsheen@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text on HO from Femto ABS to Macro ABS or other Femto ABS in Section 15.4.8.2 of P802.16m/D1. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HO from Femto ABS to Macro ABS or other Femto ABS

Yung-Han Chen, Fang-Ching (Frank) Ren, Chie Ming Chou, Chun-Yuan Chiu

ITRI

Wern Ho Sheen
Chaoyang University of Technology / ITRI

1. Introduction

This contribution provides text proposal for the handover from a Femto ABS to a macro ABS or another Femto ABS. The text proposal is based on the 16m handover defined per 15.2.6 in [1] and SDD [2] with considering the characteristics of Femto characteristics.

To facilitate the ranging process to a target macro ABS during handover network re-entry, it is suggested that the AMS may perform the ranging process based on the timing and power information provided by the original serving Femto ABS. The timing and power information is the relationship of timing and power between the Femto ABS and the overlaid macro ABS. To obtain the timing and power information, the Femto ABS should perform ranging process with the overlaid macro ABS in advance, such as during Femto ABS initialization. The AMS can take the timing and power information as the reference to quickly complete ranging with the target macro ABS. The following figure is an example to facilitate the ranging process with a target macro ABS based on the ranging results (timing and power relationship) between the Femto ABS and the macro ABS.

2. Text Proposal

{Blue text: new added}

--------------------------------------------- Text Start ----------------------------------------------------------------

15.4 Support for Femto ABS

15.4.8 Handover (HO)

15.4.8.2 HO from Femto ABS to Macro ABS or other Femto ABS

The handover procedures from a serving Femto ABS to a target macro ABS shall be the same as defined per 15.2.7 with the exceptions as defined in this subsection. A serving Femto ABS shall broadcast or unicast AAI_NBR-ADV message, which contains the information of neighbor macro ABSs and OSG/CSG-Open Femto ABSs. An AMS may also request the information of neighbor CSG-Closed Femto ABSs in the whitelist.

The trigger TLV defined per Table 702 shall be adopted to define the trigger conditions listed in 15.2.7.2. An AMS may also initiate neighbor scanning, reporting, and handover procedures autonomously. The handover procedure shall be based on 15.2.6.3.

During initiation of a Femto ABS, it can perform ranging with overlaid Macro ABS to obtain timing and power relationship between the Femto ABS and the macro ABS. When an AMS in the Femto decides to hand over to the macro cell, the timing and power relationships between the Femto ABS and the macro ABS can be send to the AMS by using AAI_HO-CMD to facilitate AMS ranging with the macro ABS.

---------------------------------------------------------------Text End ------------------------------------------------------------

Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface,” July, 2009.

[2] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”