| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Method of Resource Reservation for Interference Mitigation in Femto Cells |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yung-Han Chen, Chie Ming Chou, Fang-Ching (Frank) Ren , Chun-Yen Wang ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | chenyunghan@itri.org.tw whsheen@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes a procedure of resource reservation for interference mitigation between a Femto ABS and a macro ABS. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Method of Resource Reservation for Interference Mitigation in Femto Cells

Yung-Han Chen, Chie Ming Chou, Fang-Ching (Frank) Ren, Chun-Yen Wang

ITRI

Wern Ho Shee

Chaoyang University of Technology / ITRI

1. Descriptions

A procedure of resource reservation by Femto ABS is suggested to achieve interference mitigation between Femto and Macro cells.

2. Suggested Remedy

--------------------------------------------------------  Text Start  ------------------------------------------------------------

15.4 Support for Femto ABS

15.4.11 Interference Avoidance and Interference Mitigation

{Remedy 1: Insert the following text at the end of line 47 in page 493}

An AMS connected to a macro ABS or Femto ABS may detect interfered resource from surrounding Femto ABSs and/or macro ABSs and report to the serving ABS, so that the serving ABS may select appropriate resources for its traffic.

{Remedy 2: Insert the following text in line 60, page 493}

When a Femto ABS finds that there is no clear enough channel for its serving AMSs or serving AMSs feed back the channel quality with interference level caused by the overlaid co-channel macro ABS, the Femto ABS requests the macro ABS to reserve its resource(s), called Femto zone(s), for the Femto ABS’s transmissions/receptions. Once the macro ABS grants the Femto zone(s), the macro ABS shall avoid scheduling its serving AMSs to use the Femto zone(s). The Femto ABS may schedule the transmissions/receptions in the granted Femto zone(s). The allocating information of Femto zone is provided by S-SFH. In the Femto zone, an A-MAP of the Femto ABS should be given to specify the data allocations for the serving AMSs. In addition, the Femto zone may be shared with multiple Femto ABSs to acquire the spatial multiplexing benefit. The Femto zone shall be released to macro ABS when the Femto ABS does not need it for interference mitigation. The frame structure of Femto zone reservation and the procedure are shown in Figure XX1 and Figure XX2.

Figure XX1. The frame structure of Femto zone reservation

Figure XX2. The procedure for Femto zone reservation

--------------------------------------------------------  Text End  ------------------------------------------------------------