| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of E-MBS Zone Specific Pilot Patterns for MBSFN |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu, Zirong Li, Dihao Ning ZTE Corporation | Voice: Email: {liang.ting12, sun.bo1}@zte.com.cn |
| Re: | IEEE P802.16m/D1: EMBS |  |
| Abstract | The contribution proposes the text of EMBS pilot pattern to be included in amendment document. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Zone Specific Pilot Pattern for MBSFN

# Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu

Zirong Li, Dihao Ning

# ZTE Corporation

# Introduction

In current 802.16m SDD and D1, it’s defined that E-MBS zone specific pilot is transmitted for MBSFN transmissions. The pilot structures of E-MBS should be supported up to two streams. In SFN transmission, the Delay spread is higher than typical unicast scenarios, and the unicast pilot patterns may be not sufficient. So a new pilot pattern designs is needed for E-MBS zone.

This contribution proposes one pilot pattern solution for E-MBS zone. And the performance of this solution is evaluated.

# The structure of MIMO measurement pilot

A physical structure of E-MBS pilot which supported up to two streams for MBSFN transmisstion is proposed.

Figure 1 Proposed E-MBS pilot patterns in a RU for 2 Stream

Figure1(a) shows the proposed E-MBS pilot pattern 1 for up to 2 streams which is to be used for small inter-site distance (e.g. ISD1500m). Figure1(b) shows the proposed E-MBS pilot pattern 2 for up to 2 streams which is to be used for large inter-site distance (e.g. ISD5000m). The blue squares stand for E-MBS pilot for stream1 and the red squares stand for E-MBS pilot for stream2.

# The Performance of Proposal E-MBS pilot

Figure 2  2Tx EMBS pilot performance in 2*2 SM ISD1500m

Figure 3  2Tx EMBS pilot performance in 2*2 SFBC ISD1500m

Figure 4  2Tx EMBS pilot performance in 2*2 SM ISD1500m

Figure 5  2Tx EMBS pilot performance in 2*2 SFBC ISD1500m

Figure 6  2Tx EMBS pilot performance in 2*2 SM ISD5000m

Figure 7  2Tx EMBS pilot performance in 2*2 SFBC ISD5000m

Figure 8  2Tx EMBS pilot performance in 2*2 SFBC ISD5000m

The simulation results in the SFN transmission scenarios of ISD1500m show that the proposed E-MBS pilot achieves better FER-SNR performance than that of Intel and Samsung while the pilot density keep the same which is shown in table1. Especially when SM is employed with high mobile speed(e.g.120km/h),our design has about 1-2dB gain.

The simulation results in the SFN transmission scenarios of ISD5000m show that the proposed E-MBS pilot achieves better FER-SNR performance than that of Intel while the pilot density keep the same. We could see that whether the MIMO mode is SM or SFBC, the FER-SNR performance has about 1～2dB gain than the pilot pattern 2 of Intel.

Table 1 Comparisons of E-MBS pilot density per antenna

| Scenarios | ISD 1500 | ISD 5000 |
| --- | --- | --- |
| ZTE | 5.56% | 11.1% |
| Intel | 5.56% | 11.1% |
| Samsung | 5.56% | 5.56% |

Above all, the E-MBS pilot pattern1 and pattern2 of us achieves relatively better performance than any other scheme. So we recommend to take it into the SDD text.

For the subframe consisting of 5 symbols, the last OFDM symbol in each EMBS pilot pattern is deleted. For the subframe consisting of 7 symbols, the first OFDM symbol in each pilot pattern is added as 7th symbol.

# Appendix1

The simulation parameters is bellow.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | RU (18x6) |
| Permutation | Tone-pair Logical DRU |
| Data Size | 4PRU |
| Channel condition | ISD 1500m/5000m, 3km/h /120km/h, |
| The number of antennas | 2Tx, 2Rx |
| Channel estimation for demodulation | 2D-MMSE, Window size = 3PRU |
| Receiver | Soft-MMSE receiver |
| CL-SU-MIMO schemes | Rate-2 SM / Rate-1 SFBC |
| Scenarios | Noise limited |
| Pilot Pattern | Intel, Samsung and ZTE |
| Pilot Boosting | 5dB |

# Appendix2

The following is EMBS pilot pattern for Samsung and Intel.

Figure 9  EMBS-Pilot for Samsung and Intel

# References

[1] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[2] “E-MBS Zone Specific Pilot Patterns for MBSFN”, IEEE C802.16m-09/0929

# Text proposal for inclusion in the P802.16m D1

-------------------------------  Text Start  ------------------------------------------------------------------------------------

[Insert the following text into the EMBS subclause.]

15.X.X.X E-MBS zone specific pilot for MBSFN

E-MBS zone specific pilot is transmitted for MBSFN transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilot, that’s, common inside one E-MBS zone but different between neighboring E-MBS zones, is configured. Synchronous transmissions of the same contents with common pilot from multiple ABS in one MBS zone would result in correct MBSFN channel estimation. The E-MBS zone specific pilot streams depend on the maximum number of streams within the E-MBS zone.

Pilot structures/patterns should be supported up to two streams.

The E-MBS pilot patterns for ISD 1500 and ISD 5000 is shown in Figure xxx.

Figure xxx Proposed E-MBS pilot patterns in a RU for 2 Stream

-------------------------------  Text End  ------------------------------------------------------------------------------------