| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of E-MBS Zone Specific Pilot Patterns for MBSFN |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu, Zirong Li, Dihao Ning ZTE Corporation | Voice: Email: {liang.ting12, sun.bo1}@zte.com.cn |
| Re: | IEEE P802.16m/D1: EMBS |  |
| Abstract | The contribution proposes the text of EMBS pilot pattern to be included in amendment document. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

E-MBS Zone Specific Pilot Pattern for MBSFN

Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu

Zirong Li, Dihao Ning

ZTE Corporation

# Introduction

In current SDD description, E-MBS zone specific pilot is transmitted for MBSFN transmissions. The pilot structures of E-MBS should be supported up to two streams. In SFN transmission , the Delay spread is higher than typical unicast scenarios, and the unicast pilot patterns may be not sufficient. So a new pilot pattern designs is needed for E-MBS zone.

This contribution proposes one pilot pattern for E-MBS zone. And the performance of this pilot patterns is evaluated.

# The structure of MIMO measurement pilot

A physical structure of E-MBS pilot which supported up to two streams for MBSFN transmisstion is proposed.

Figure 1 Proposed E-MBS pilot patterns in a RU for 2 Stream

Figure1(a) shows the proposed E-MBS pilot pattern 1 for up to 2 streams which is to be used for small inter-site distance (e.g. ISD1500m). Figure1(b) shows the proposed E-MBS pilot pattern 2 for up to 2 streams which is to be used for large inter-site distance (e.g. ISD5000m). The blue squares stand for E-MBS pilot for stream1 and the red squares stand for E-MBS pilot for stream2.

The BS decides the EMBS pilot pattern to be used according to the inter-site distance, and then inform MS in S-SFH IE with only one bit information. If this bit is set to 0, the E-MBS pilot pattern 1 is to be used by all user belong to this BS. If this bit is set to 1, the E-MBS pilot pattern 2 is to be used.

# The Performance of Proposal E-MBS pilot

- ISD1500，SFBC

Figure 2  2Tx EMBS pilot performance in 2*2 SFBC ISD1500m

Figure 3  2Tx EMBS pilot performance in 2*2 SFBC ISD1500m

- ISD1500 SM

Figure 4  2Tx EMBS pilot performance in 2*2 SM ISD1500m

Figure 5  2Tx EMBS pilot performance in 2*2 SM ISD1500m

Figure 6  2Tx EMBS pilot performance in 2*2 SM ISD1500m

- ISD5000 SFBC

Figure7  2Tx EMBS pilot performance in 2*2 SFBC ISD5000m

- ISD5000 SM

Figure 8  2Tx EMBS pilot performance in 2*2 SM ISD5000m

Figure 9  2Tx EMBS pilot performance in 2*2 SFBC ISD5000m

Figure 10  2Tx EMBS pilot performance comparison with ITRI in 2*2 SFBC ISD5000m

The simulation results in the SFN transmission scenarios of ISD1500m show that the proposed E-MBS pilot achieves better FER-SNR performance than that of Intel and Samsung while the pilot density keep the same which is shown in table1. Especially when SM is employed with high mobile speed(e.g.120km/h),our design has about 1-2dB gain.

The simulation results in the SFN transmission scenarios of ISD5000m show that the proposed E-MBS pilot achieves better FER-SNR performance than that of Intel while the pilot density keep the same. We could see that whether the MIMO mode is SM or SFBC, the FER-SNR performance has about 1～2dB gain than the pilot pattern 2 of Intel.

Figure 10 shows the FER and Throughput comparison of EMBS pilot pattern with ITRI. The FER-SNR performance of our pilot design is more than 1dB better than ITRI’s. The throughput is also largely better than ITRI.

Table 1 Comparisons of E-MBS pilot density per antenna

| Scenarios | ISD 1500 | ISD 5000 |
| --- | --- | --- |
| ZTE | 5.56% | 11.1% |
| Intel | 5.56% | 11.1% |
| Samsung | 5.56% | 5.56% |

Above all, the E-MBS pilot pattern1 and pattern2 of us achieves relatively better performance than any other scheme. So we recommend to take it into the SDD text.

For the subframe consisting of 5 symbols, the last OFDM symbol in each EMBS pilot pattern is deleted. For the subframe consisting of 7 symbols, the first OFDM symbol in each pilot pattern is added as 7th symbol.

# Appendix1

The simulation parameters is bellow.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | RU (18x6) |
| Permutation | Tone-pair Logical DRU |
| Data Size | 4PRU |
| Channel condition | ISD 1500m/5000m, 3km/h /120km/h, |
| The number of antennas | 2Tx, 2Rx |
| Channel estimation for demodulation | 2D-MMSE, Window size = 3PRU |
| Receiver | Soft-MMSE receiver |
| CL-SU-MIMO schemes | Rate-2 SM / Rate-1 SFBC |
| Scenarios | Noise limited |
| Pilot Pattern | Intel, Samsung and ZTE |
| Pilot Boosting | 5dB |

# Appendix2

The following is EMBS pilot pattern for Samsung and Intel.

Figure 11  EMBS-Pilot for Samsung and Intel

# References

[1] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[2] “E-MBS Zone Specific Pilot Patterns for MBSFN”, IEEE C802.16m-09/0929

# Text proposal for inclusion in the 802.16m SDD

-------------------------------  Text Start  ------------------------------------------------------------------------------------

11.5.3.2 E-MBS zone specific pilot for MBSFN

E-MBS zone specific pilot is transmitted for MBSFN transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilot, that’s, common inside one E-MBS zone but different between neighboring E-MBS zones, is configured. Synchronous transmissions of the same contents with common pilot from multiple ABS in one MBS zone would result in correct MBSFN channel estimation. The E-MBS zone specific pilot streams depends on the maximum number of streams within the E-MBS zone.

Pilot structures/patterns should be supported up to two streams.

The definitions of the E-MBS zone specific pilots are FFS.

The E-MBS pilot patterns for different inter-site distance is shown in Figure xxx. Figure xxx(a) shows the proposed E-MBS pilot pattern 1 for up to 2 streams which is to be used for small inter-site distance (e.g. ISD1500m). Figure xxx(b) shows the proposed E-MBS pilot pattern 2 for up to 2 streams which is to be used for large inter-site distance (e.g. ISD5000m).

The BS decides the EMBS pilot pattern to be used according to the inter-site distance, and then inform MS in S-SFH IE with only one bit information. If this bit is set to 0, the E-MBS pilot pattern 1 is to be used by all user belong to this BS. If this bit is set to 1, the E-MBS pilot pattern 2 is to be used.

Figure xxx Proposed E-MBS pilot patterns in a RU for 2 Stream

-------------------------------  Text End  ------------------------------------------------------------------------------------