| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on AAI_HO-CMD |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Yang Liu, Yuqin Chen, Nan Li, Hongyun Qu, Mary Chion ZTE Corporation | Liu.yang8@zte.com.cn chen.yuqin@zte.com.cn |
| Re: | Call for Letter Ballot 30 |  |
| Abstract | This contribution proposes new text on AAI-HO_CMD. |  |
| Purpose | [HO]To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on AAI_HO-CMD

Yang Liu, Yuqin Chen, Nan Li, Hongyun Qu, Mary Chion

ZTE Corporation

Proposed Text Changes

------------------------------------------------------------Start of the Text--------------------------------------------------------

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message shall include the following parameters:

- Mode:

  - 0b00: HO command

  - 0b01: Zone switch command from MZone to LZone

  - 0b10: HO to YBS/LZone of neighbor ABS

  - 0b11: AMS HO request rejected (ABS in list unavailable).

In case of Mode = 0b00/0b10, AAI_HO-CMD message should include one or more target ABS/YBS/LZone of neighbor ABS and the following parameters:

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS; otherwise the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS. The following parameters shall be included when HO_Reentry_Mode is set to 1:

  - HO_Reentry_Interleaving Interval: If HO_Reentry_Interleaving Interval is greater than zero, the AMS continues data transmission with the Serving ABS within the HO_Reentry_Interleaving Interval and performs network reentry to the target ABS in the remaining time. Otherwise if HO_Reentry_Interleaving Interval is set to 0, the AMS maintains communication with Serving ABS on one carrier while performing network reentry with Target ABS on another carrier.

    - For HO_Reentry_Interleaving_Interval >= 0, the following parameter is further included:

      - HO_Reentry Iteration: The requested number of iterating HO Reentry Intervals by an AMS.

- Disconnect time offset: Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time – Disconnect time offset); for HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset).

- Resource_Retain_Time

For Mode = 0b00 and for each target ABS, the following parameters shall be included:

- Target ABS ID

- Action Time: Action Time included in this message is the absolute frame number at the serving ABS.

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunity

- HO process optimization: For each Bit location, a value of 0 indicates the associated reentry management messages shall be required, a value of 1 indicates the reentry management message may be omitted.

  - Bit #0: Omit AAI_SBC-REQ/RSP management messages during reentry processing

  - Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing

  - Bit #2: Seamless HO

  - Bit #3: TBD.

- Ranging initiation deadline.

- Service level prediction, which indicates the level of service the AMS can expect from this ABS. The following encodings apply:

  - 0 = No service possible for this AMS

  - 1 = Some service is available for one or several service flows authorized for the AMS

  - 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

  - 3 = No service level prediction available.

For Mode = 0b00 for each target ABS which is multi-carrier ABS, the following parameters should be included:

- Physical_Carrier_Index: Physical carrier index of the Target ABS where AMS will perform network reentry procedure.

For Mode = 0b00 for an AMS with multi-carrier capability, the following parameters should be included:

- MCHO_Indication: If MCHO_Indication is 0, the AMS shall disconnect all its active secondary carriers and perform single-carrier handover procedure per 15.2.6.3; otherwise it performs multi-carrier handover procedure per 15.2.9.2.9.2.

In case of Mode = 0b01, the AAI_HO-CMD message is used as a zone switch instruction from MZone to LZone, and the following parameters shall be included:

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier; otherwise the AMS disconnects from the MZone before performing network reentry with the LZone.

- Action Time: This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number.

- Resource_Retain_Time

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunity

- LZone information: LZone information for the AMS to switch from MZone to LZone.The following parameters may be included

  - Carrier information of LZone (e.g. carrier physical index)

For Mode = 0b10 and for each target YBS//Lzone of target ABS, the following parameters shall be included:

- Target YBS ID/BS ID of LZone of neighbor ABS

- Preamble Index

- Service level prediction, which indicates the level of service the AMS can expect from this YBS. The following encodings apply:

  - 0 = No service possible for this AMS

  - 1 = Some service is available for one or several service flows authorized for the AMS

  - 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

  - 3 = No service level prediction available.

For Mode = 0b10 and for each target YBS/LZone of target ABS, the following parameters may be included:

- Pre allocated Basic CID, which is used by the AMS to derive its own primary CID and transport CID in target YBS.

In case of Mode = 0b11, AAI_HO-CMD message shall not include any target ABS.