| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Resource Mapping Signaling to Sections 15.3.5 (Downlink Physical Structure) and 15.3.8 (Uplink Physical Structure). |  |
| Date Submitted | 2009-08-24 |  |
| Source(s) | Yanfeng Guan, Ding Ning, Xiangyu Liu, Ying Liu, Huiying Fang ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D1” 15.3.5 Downlink Physical Structure & 15.3.8 Uplink Physical Structure |  |
| Abstract | This contribution proposes text specifying the some resource mapping signaling. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Resource Mapping Signaling to Section 15.3.5 (Downlink Physical Structure) and 15.3.8 (Uplink Physical Structure)

Yanfeng Guan, Ding Ning, Xiangyu Liu, Ying Liu, Huiying Fang

ZTE Corporation

1 Introduction

This contribution provides modified text for the DL/UL physical structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Draft Amendment [1] and accepted contribution [2] []3]. The proposed update is developed so that it can be readily combined with P802.16m/D1 [1], it is compliant to the 802.16m SRD [4] and the 802.16m SDD [5], and it follows the style and format guidelines in [6].

2 Problems

After a continual round of discussion on the physical resource structure, that section has become better than before. However, there are still several problems.

1) For evaluating the real performance of the 16m system, the peak rate should be tested. Regarding the design of the resource structure, the peak rate should be tested under these three cases:

a) All LRUs are DLRUs,

b) All LRUs are CLRUs,

c) Mixed LRUs including (DLRUs and CLRUs)

The physical resource structure should support this requirement. But, DSAC table can't support all LRUs are CLRUs because several possible values close to the maximum are reserved. Therefore, Table 649, 650 and 651 for downlink should be modified. Because the uplink is similar to the downlink, table 720, 721 and 722 for uplink should be modified.

2) Some signaling for DL/UL resource mapping should be sent depending on FPCT in order to reduce the overhead. So, some formulas of DL/UL resource mapping can be optimized depending on the Number of Frequency Partition, for example, Formula  (183) in section 15.3.5 and (228) in section 15.3.8.

3 References

[1] IEEE P802.16m/D1 July 2009, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” July. 2009.

[2] IEEE 802.16m-09/1205r7. “Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.5 (Downlink Physical Structure) of IEEE 802.16m AWD -- AWD-IMDG”. July. 2009.

[3] IEEE 802.16m-09/1206r7. “Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.8 (Uplink Physical Structure) of IEEE 802.16m AWD -- AWD-IMDG”. July. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document,” Jan. 2009.

[3] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Proposed Text

Proposed Text for Remedy 1:

Modify Table 649,650 and 651 in Section 15.3.5 and Table 720,721 and 722 in Section 15.3.8.

--------------------------------------------- Proposed Text Starts ----------------------------------------------------

Table 649—Mapping between DSAC and KSB for 20MHz

| DSAC | # of subbands allocated (KSB) | DSAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 |
| 1 | 1 | 17 | 17 |
| 2 | 2 | 18 | 18 |
| 3 | 3 | 19 | 19 |
| 4 | 4 | 20 | 20 |
| 5 | 5 | 21 | 21 |
| 6 | 6 | 22 | N. A. 22 |
| 7 | 7 | 23 | N. A. 23 |
| 8 | 8 | 24 | N. A. 24 |
| 9 | 9 | 25 | N. A. |
| 10 | 10 | 26 | N. A. |
| 11 | 11 | 27 | N. A. |
| 12 | 12 | 28 | N. A. |
| 13 | 13 | 29 | N. A. |
| 14 | 14 | 30 | N. A. |
| 15 | 15 | 31 | N. A. |

Table 650—Mapping between DSAC and KSB for 10MHz

| DSAC | # of subbands allocated (KSB) | DSAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 |
| 1 | 1 | 9 | 9 |
| 2 | 2 | 10 | N. A. 10 |
| 3 | 3 | 11 | N. A. 11 |
| 4 | 4 | 12 | N. A. 12 |
| 5 | 5 | 13 | N. A. |
| 6 | 6 | 14 | N. A. |
| 7 | 7 | 15 | N. A. |

Table 651—Mapping between DSAC and KSB for 5MHz

| DSAC | # of subbands allocated (KSB) | DSAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | N. A. 4 |
| 1 | 1 | 5 | N. A. 5 |
| 2 | 2 | 6 | N. A. 6 |
| 3 | 3 | 7 | N. A. |

Table 720—Mapping between USAC and KSB for 20MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 |
| 1 | 1 | 17 | 17 |
| 2 | 2 | 18 | 18 |
| 3 | 3 | 19 | 19 |
| 4 | 4 | 20 | 20 |
| 5 | 5 | 21 | 21 |
| 6 | 6 | 22 | N. A. 22 |
| 7 | 7 | 23 | N. A. 23 |
| 8 | 8 | 24 | N. A. 24 |
| 9 | 9 | 25 | N. A. |
| 10 | 10 | 26 | N. A. |
| 11 | 11 | 27 | N. A. |
| 12 | 12 | 28 | N. A. |
| 13 | 13 | 29 | N. A. |
| 14 | 14 | 30 | N. A. |
| 15 | 15 | 31 | N. A. |

Table 721—Mapping between USAC and KSB for 10MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 |
| 1 | 1 | 9 | 9 |
| 2 | 2 | 10 | N. A. 10 |
| 3 | 3 | 11 | N. A. 11 |
| 4 | 4 | 12 | N. A. 12 |
| 5 | 5 | 13 | N. A. |
| 6 | 6 | 14 | N. A. |
| 7 | 7 | 15 | N. A. |

Table 716—Mapping between USAC and KSB for 5MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | N. A. 4 |
| 1 | 1 | 5 | N. A. 5 |
| 2 | 2 | 6 | N. A. 6 |
| 3 | 3 | 7 | N. A. |

------------------------------------------------- Proposed Text Ends -------------------------------------------------

Proposed Text for Remedy 2:

Modify the text from line 20 to 28 on page 268 in section 15.3.5

-------------------------------------------------- Proposed Text Starts -----------------------------------------------

The number of subbands for each frequency partition when FPCT = 1 or FPCT = 4, is given by equation (183).

(183)

(183)

When DFPC = 1 and FPCT = 3, the number of subbands in FPi (for i>0) is given by KSB,FPi = DFPSC.

-------------------------------------------------- Proposed Text Ends ------------------------------------------------

Modify the text from line 23 to 29 on page 405 in section 15.3.8

-------------------------------------------------- Proposed Text Starts -----------------------------------------------

The number of subbands in the ith frequency partition is denoted by KSB,FPi, as shown in equation (228) for the cases when FPCT = 1 or FPCT = 4,

(228)

(228)

When UFPC = 1 and FPCT = 3, the number of subbands in FPi (for i>0) is given by KSB,FPi = UFPSC.

-------------------------------------------------- Proposed Text Ends ------------------------------------------------