| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed “Resource Allocation” field in Assignment A-MAP IEs |  |
| Date Submitted | 2009-08-24 |  |
| Source(s) | Yanfeng Guan Xiangyu Liu, Ying Liu, Huiying Fang, Bo Sun ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D1” 15.3.6 Downlink Control Structure & 15.3.8 Uplink Control Structure |  |
| Abstract | This contribution proposes text specifying the structure and interpretation of the “Resource Allocation” filed in the A-AMAP IEs for the 5, 10 & 20 MHz in some special cases. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed “Resource Allocation” field in Assignment A-MAP IEs

Yanfeng Guan, Xiangyu Liu, Ying Liu, Huiying Fang, Bo Sun

ZTE Corporation

- Introduction

In current 802.16m draft amendment [1], four types of Assignment A-MAP IEs are defined and used for dynamic scheduling. They are DL/UL Basic Assignment A-MAP IEs for an allocation of contiguous LRUs and DL/UL Sub-band Assignment A-MAP IEs for an allocation of subband-based LRUs in DL/UL subframes. For these four IEs, the main difference is the interpretation for the “Resource Index” field.

- Problem

RI (Resource Index) field indicates the location and size of one instance of a resource allocation for an AMS. Although some good design considerations in [2] were taken into account, but there are still some other issues needed to be considered.

For RA field in DL/UL Basic Assignment A-MAP IEs, the pre-processing is derivation of the mapping between LRU index and physical PRU index. Mapping CRU/DRU to LRU is defined as:

where and

Obviously, the derivate LRU index is according Frequency Partition Number, not the DRUs and CRUs, as a result of that, all the CRUs or DRUs are non-contiguous in logical index, and then many allocation granularities can’t be implemented when FPCT is more than 1. For example in 10MHz, if DSAC=7, DFPC=2, DFPSC=2, DCASSB,0=1, DCASMB,0=0 and DCASSB,i=2 according to the DL Resource Mapping Signaling, that means the maximum logical contiguous DRU is 8, so any allocation size more than 8 for DRU allocation is impossible. At the same time, any allocation more than 3 subbands is also impossible. This problem is not only a special case, actually, it will be present for many resource mapping configurations in DL/UL Resource Mapping Configurations.

For RA field in DL/UL Subband Assignment A-MAP IEs, the indexing method means that the ABScan use two different resource allocation granularities (a subband and a half subband) for the different bandwidth or different number of subband. When a half subband is used as the allocation granularity, the ABS can only allocate those resources with odd logical 2-CRU index or even logical 2-CRU index regarding Table 668 [Y = 6, ITF = 00/01] and Table 669, but there are some problems:

1) When ITF is equals 01 or 10, it's impossible the scheduler always use the half subband from those with odd (2j+1) or even (2j) index.

2) There is no much evidence to testify the partition for the value Y is reasonable. The interpretation of the RA Field in 5/10/20 MHz systems present a specific resource indication technique, but in some cases, a lot of combination of different CLRU resource can't be indicated.

3) For 20MHz, the method is too complicated. In addition, Table for triplets of non-contiguous sub-band indices is TBD.

In this contribution, we propose the corresponding text for the “Resource Allocation” field in DL/UL Subband-Assignment A-MAP IE for some special case.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D1 July 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/1327r1, “Proposed Changes to the DL and UL A-MAP Information Element Types in AWD ”

[6] IEEE 802.16m-09/1331r2, “Proposed AWD text specifying the “Resource Allocation” field in the A-MAP IEs AWD ”

[7] IEEE 802.16m-09/1334r1, “AWD Proposal for DL/UL Resource Indexing in IEEE 802.16m ”

[8] IEEE 802.16m-09/1516r “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

- Proposed Text

Proposed Text for Remedy 1:

Modify “Derivation of the mapping between LRU index and physical PRU index” part on page 326 in the 15.3.6.5.4.2 DL basic assignment A-MAP IE as followed:

----------------------------------------------------- Proposed Text Starts --------------------------------------------------

Derivation of the mapping between LRU index and physical PRU index:

For each frequency partition i, tThe total number of CLRUs &and DLRUs up-to and including that partition i is calculated as

(200)

(200)

(200a)

(200b)

(200c)

(201)

(201)

with

with

The mapping from CRUFPi and DRUFPi to the physical PRU indices (and vice-versa) is as specified in Section 15.3.5 and Section 15.3.8.

The LRU index, L obtained from the RI field refers to the LRU[ ] defined above.

------------------------------------------------------ Proposed Text Ends --------------------------------------------------

Proposed Text for Remedy 2:

Modify indexing method part on page 311 in the 15.3.6.5.1 DL Sub-band Assignment A-MAP IE as followed:

------------------------------------------------------ Proposed Text Starts --------------------------------------------------

15.3.6.5.1. DL Sub-band Assignment A-MAP IE

……

15.3.6.5.1.1 Pre-Processing

The mapping between the LRU indices and the physical PRU indices (and vice-versa) may be derived as described in Section 15.3.6.5.2.2 (“DL Basic Assignment A-MAP IE”).

Derivation of the mapping between the sub-band-CRU index and the physical PRU index: shall be done as follows.

The total number of sub-bands over all partitions may be is calculated as

(Eq. 001)

For each frequency partition i, tThe total number of sub-band-CRUs up-to and including that partition may be i is calculated as

(Eq. 002)

Then, the logical sub-band CRUs are indexed as

(Eq. 003)

with

with

The mapping from the  to the physical PRU indices (and vice-versa) is as specified in Section 15.3.5 and Section 15.3.8.

The LRUSBCRU & SBCRU indices indicated by the RA field refer to the LRUSBCRU [] and the SBCRU[] defined above.

15.3.6.5.1.2 Nominal Channel Bandwidth = 5 MHz

For a 5 MHz system:

- A single instance of a resource allocation shall be made using a single IE.

- The AMS shall interpret the RA field as defined by Table 668.

Table 668: Interpretation of the RA Field in a 5 MHz system

| Total # of sub-bands over all frequency partitions, YSSB | RA Field Interpretation |  |
| --- | --- | --- |
| YSSB = 6 | The 2 MSB bits of the RA field are denoted as the “Indication Type field” (ITF). The 6 9 LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < 6 9, with j = 0 being the LSB. |  |
|  | ITF == 00 | Each of the RIF[j], 0 <= j < 6, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band. = j have been allocated. = j have been not allocated. |
|  | ITF == 01 | Each of the RIF[j], 0 <= j < 69, indicates the allocation or non-allocation of the 1st or last 2 CRUs within a particular sub-band. RIF[j] = 1 The 2 CRUs with indices k such that=2j have been allocated. RIF[j] = 0 The 2 CRUs with indices k such that =2j have been not allocated. |
|  | ITF == 10 | Each of the RIF[j], 0 <= j < 69, indicates the allocation or non-allocation of the last 2 CRUs within a particular sub-band. RIF[j] = 1 The 2 CRUs with indices k such that=2j+1 have been allocated. RIF[j] = 0 The 2 CRUs with indices k such that=2j+1 have been not allocated. |
|  | ITF == 11 | Reserved |
| Y SSB <= 5 | Each of the j bit-positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of 2 CRUs within a particular sub-band. Position j in RA corresponds to the 2 CRUs with indices k such that =j. Depending on the value of Y, not all j may be relevant. RA[j] = 1 The 2 CRUs with indices k such that= j have been allocated. RA[j] = 0 The 2 CRUs with indices k such that= j have been not allocated. |  |

15.3.6.5.1.3 Nominal Channel Bandwidth = 10 MHz

For a 10 MHz system a single instance of a resource allocation shall be made using a single IE. Furthermore when Y SSB (the total number of sub-bands over all frequency partitions) <= 11, the AMS shall interpret the bits in the Resource Allocation field as indicated by Table 669 (the row corresponding to the particular value of Y SSB in that table). In the case that Y SSB = 12, the RA field shall be interpreted as in the first row of Table 669.

Table 669: Interpretation of the RA Field in a 10 MHz or a 20 MHz system, when Y SSB <= 11 (Y is the total number of sub-bands over all frequency partition)

| Total # of sub-bands over all frequency partitions, Y SSB | RA Field Interpretation |  |
| --- | --- | --- |
| Y SSB = 10 or 11 | Each of the j bit-positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band. Denote RA[j] as the jth bit position in the RA Field, 0 <= j < 11, with j = 0 being the LSB. RA[j] corresponds to the subband, i.e., the 4 CRUs with indices k such that = j. Depending on the value of YSSB , not all j may be relevant. RIF[j] = 1The 4 CRUs with indices k such that =j have been allocated. RIF[j] = 0The 4 CRUs with indices k such that =j have been not allocated. |  |
| Y SSB = 6,7,8 or 9 | The 2 MSB bits of the RA field are denoted as the “Indication Type field” (ITF). The Y SSB LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < Y SSB, with j = 0 being the LSB. |  |
|  | ITF == 00 | Each of the RIF[j], 0 <= j < Y SSB, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band (out of the Y SSB sub-bands). RIF[j] = 1The 4 CRUs with indices k such that =j have been allocated. RIF[j] = 0The 4 CRUs with indices k such that =j have been not allocated |
|  | ITF == 01 | Each of the RIF[j], 0 <= j < Y SSB, indicates the allocation or non-allocation of the 1st 2 CRUs within a particular sub-band. RIF[j] = 1The 2 CRUs with indices k such that = 2j have been allocated. RIF[j] = 0The 2 CRUs with indices k such that = 2j have been not allocated. |
|  | ITF == 10 | Each of the RIF[j], 0 <= j < Y SSB, indicates the allocation or non-allocation of the last 2 CRUs within a particular sub-band. RIF[j] = 1The 2 CRUs with indices k such that = 2j +1 have been allocated. RIF[j] = 0The 2 CRUs with indices k such that = 2j +1 have been not allocated. |
| SSB = =6 | The 2 MSB bits of the RA field are denoted as the “Indication Type field” (ITF). The 9 LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < 9, with j = 0 being the LSB. |  |
|  | ITF == 00 | Each of the RIF[j], 0 <= j < 6, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band. RIF[j] = 1The 4 CRUs with indices k such that =j have been allocated. RIF[j] = 0The 4 CRUs with indices k such that =j have been not allocated |
|  | ITF == 01 | Each of the RIF[j], 0 <= j < 9, indicates the allocation or non-allocation of the 1st or last 2 CRUs within a particular sub-band. RIF[j] = 1 The 2 CRUs with indices k such that= j have been allocated. RIF[j] = 0 The 2 CRUs with indices k such that = j have been not allocated. |
|  | ITF == 10 | Each of the RIF[j], 0 <= j < 9, indicates the allocation or non-allocation of the 1st or last 2 CRUs within a particular sub-band. RIF[j] = 1 The 2 CRUs with indices k such that= j have been allocated. RIF[j] = 0 The 2 CRUs with indices k such that = j have been not allocated. |
|  | ITF == 11 | Reserved |
| Y SSB <= 5 | Each of the j bit-positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of 2 CRUs within a particular sub-band. Position j in RA corresponds to the 2 CRUs with indices k such that =j. Depending on the value of Y, not all j may be relevant. RA[j] = 1 The 2 CRUs with indices k such that= j have been allocated. RA[j] = 0 The 2 CRUs with indices k such that= j have been not allocated. |  |

15.3.6.5.1.3 Nominal Channel Bandwidth = 20 MHz

For a 20 MHz system the following procedure shall be followed:

1) When Y SSB (total number of sub-bands over all frequency partitions) <= 11, a single instance of a resource allocation shall be made using a single IE. In this case, thes AMS shall interpret the bits in the Resource Allocation field as indicated by Table 669 (the row corresponding to the particular value of Y SSB in that table).

2) When Y SSB (total number of sub-bands over all frequency partitions) > 11, a single instance of a resource allocation may be made using a single or two IEs.

a) When an allocation is made using 2 IEs, the RA fields of the two IEs shall be concatenated to form a 22-bit field, referred to as the Concatenated-RA field (C-RA field). The LSB of the RA field of the IE occurring last in the A-AMAP region shall be interpreted as the LSB of the C-RA field. The AMS shall interpret the bits in the C-RA field as indicated by Table 670 (the row corresponding to the particular value of Y SSB in that table). The AMS may infer that two IEs refer to the same instance of a resource allocation from the values of the ACID and SPID fields.

b) When an allocation is made using a single IE, the RA field shall be interpreted as in Table 671 & Table_TBD. These tables map the value of the RA field to the indices of 2 non-contiguous sub-bands, and 3 non-contiguous sub-bands, respectively.

Table 670— Interpretation of the C-RA Field in a 20 MHz system

| Total # of sub-bands over all frequency partitions, Y SSB | C-RA Field Interpretation |  |
| --- | --- | --- |
| Y SSB = 21,22,23 or 24 | Each of the j bit-positions in the C-RA field, 0 <= j < 22, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band. Denote RA[j] as the jth bit position in the C-RA Field, 0 <= j < 22, with j = 0 being the LSB. RA[j] corresponds to the subband, i.e., the 4 CRUs with indices k such that = j. Depending on the value of YSSB , not all j may be relevant. RA[j] = 1The 4 CRUs with indices k such that =j have been allocated. RA[j] = 0The 4 CRUs with indices k such that =j have been not allocated. |  |
| 1220 <= Y SSB <= 1220 | The 2 MSB bits of the C-RA field are denoted as the “Indication Type field” (ITF). The Y SSB LSB bits of the C-RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < Y SSB, with j = 0 being the LSB. |  |
|  | ITF == 00 | Each of the RIF[j], 0 <= j < Y SSB, indicates the allocation or non-allocation of all N1 (= 4) CRUs within a particular sub-band (out of the Y SSB sub-bands). RIF[j] = 1The 4 CRUs with indices k such that = j have been allocated. RIF[j] = 0The 4 CRUs with indices k such that = j have been not allocated |
|  | ITF == 01 | Each of the RIF[j], 0 <= j < Y SSB,, indicates the allocation or non-allocation of the 1st 2 CRUs within a particular sub-band. RIF[j] = 1The 2 CRUs with indices k such that = 2j have been allocated. RIF[j] = 0The 2 CRUs with indices k such that = 2j have been not allocated. |
|  | ITF == 10 | Each of the RIF[j], 0 <= j < Y SSB, indicates the allocation or non-allocation of the last 2 CRUs within a particular sub-band. RIF[j] = 1The 2 CRUs with indices k such that = 2j+1 have been allocated. RIF[j] = 0The 2 CRUs with indices k such that = 2j+1 have been |
|  | ITF == 11 | Reserved |

Table 671 maps the value of the 11-bit RA to a pair of non-contiguous sub-band indices.

Table 671—Mapping of decimal values of the 11-bit RA (for 0 <= RA value < 254) to a pair of non-contiguous sub-band indices, for a 10 MHz system

| 1st SBI 2nd SBI | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  | 21 |
| 1 |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42 |
| 2 |  |  | 43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 |
| 3 |  |  |  | 63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 81 |
| 4 |  |  |  |  | 82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 99 |
| 5 |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 116 |
| 6 |  |  |  |  |  |  | 117 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 132 |
| 7 |  |  |  |  |  |  |  | 133 |  |  |  |  |  |  |  |  |  |  |  |  |  | 147 |
| 8 |  |  |  |  |  |  |  |  | 148 |  |  |  |  |  |  |  |  |  |  |  |  | 161 |
| 9 |  |  |  |  |  |  |  |  |  | 162 |  |  |  |  |  |  |  |  |  |  |  | 174 |
| 10 |  |  |  |  |  |  |  |  |  |  | 175 |  |  |  |  |  |  |  |  |  |  | 186 |
| 11 |  |  |  |  |  |  |  |  |  |  |  | 187 |  |  |  |  |  |  |  |  |  | 197 |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  | 198 |  |  |  |  |  |  |  |  | 207 |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  | 208 |  |  |  |  |  |  |  | 216 |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 217 |  |  |  |  |  |  | 224 |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 225 |  |  |  |  |  | 231 |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 232 |  |  |  |  | 237 |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 238 |  |  |  | 242 |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 243 |  |  | 246 |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 247 |  | 249 |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 | 251 |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 252 |

In Table 671, the decimal value of the RA field increase from left to right, and then from top to bottom; values at the beginning and end of each row are shown in the table. The shaded cells are not used to make interpretations. For a given value of the RA, the mapping using that value is given by the indices of the two sub-bands as indicated in the “1st sub-band index” row and the “2nd sub-band index” column. As an example, RA == 00001001 (Decimal value 9) maps to the pair of sub-bands indices {0, 11}. Then, the allocation is the set of LRUs with index k such that  = 0 or 11.

Table Table_RA_5 translates values of the 11-bit RA from 253 to 2047 to triplets of non-contiguous sub-band indices. This table is TBD.

------------------------------------------------------ Proposed Text Ends --------------------------------------------------