| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS Operation in IEEE P802.16m (LBS) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 LBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | LBS operation on IEEE 802.16m |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# LBS Operation in IEEE P802.16m (LBS)

Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes the support for Located Based Services (LBS) and Section 12 in SDD describes some primitive concept of LBS, which is not enough to cover LBS operation on the next version of IEEE 802.16m Amendment Draft Standard [2] . Therefore, in this contribution, we describe the detail description of LBS operation and some AAI MAC management messages (i.e., AAI_LBS-RSP, AAI_LBS-REP), which support LBS operation in IEEE 802.16m.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

Proposed Text for Support for LBS (15.2 in the 802.16m Amendment Draft Standard)

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[add the following new section]

15.2.x Support for LBS

15.2.x.a General Concept of LBS

IEEE 802.16m supports basic MAC and PHY features to support both use cases, with or without use of GPS or equivalent satellite based location solution.

IEEE 802.16m system entities will support LBS applications by providing them with:

• Relevant measurements, periodic or event driven

• Resources (time and frequency slots) to perform the relevant measurements

• Communication channels (unicast and broadcast), as allocated to higher layer applications of any type.

It should be emphasized that the actual implementation of the LBS application or method of location determination is out of the scope of IEEE 802.16m.

In order to enhance location based service, AMS should send report location-related information which includes the location information or the measurement for determining location in response to the request of ABS . In addition, LBS is supported for AMS in connected state as well as idle state. For the connected state, AMS can report location information when it is needed. For the idle state, AMS should perform network re-entry to report location information when it is needed.

The AMS positioning is performed by using measurement methods, such as TDOA, TOA, AOA, and etc., whose relevant location-related parameters may include cell-ID, RSSI, CINR, RD, RTD, angle, and Spatial Channel Information. These parameters are exchanged between the AMS and its serving/attached or/and neighboring ABSs/ARSs. The measurements of these parameters are extracted by processing DL and/or UL signals at the AMS and ABSs, respectively. Positioning algorithms that depend on such measurements have certain performance tradeoffs in terms of positioning accuracy, latency, and signaling overhead. Two or more measurements can be utilized to provide higher accuracy estimate of the AMS position.

15.2.x.b Reporting Methods for LBS

15.2.x.b.1 Reporting Types

According to the measurement methods of LBS, some location information or some LBS measurement parameters such as CINR/RSSI/RD/RTD/Angle are transmitted to the ABS to measure the location.

15.2.x.b.2 Reporting Mode and Condition

An AMS supported LBS reports location information if any of following location information reporting condition is met.

- Timer based location information reporting

-Threshold based location information reporting

An LBS-capable AMS should support the following reporting modes: per-request, periodic, and event-triggered reporting modes. The event-triggered reporting mode is a variation of the periodic reporting mode with reporting criteria, such as a moving distance threshold and updated timer expiration. For example, the AMS will report the location when the distance between the current location and the last reported location beyond the “moving distance threshold”.

15.2.x.c LBS Operation

15.2.x.c.1 LBS Operation in Connected State

For connected state, LBS can be initiated by the ABS or the AMS.

In order to initiate the LBS report, The ABS sends an AAI_LBS-RSP MAC management message to the AMS or the AMS sends an AAI_LBS-REP MAC management message to the ABS. In the response to the AAI_LBS-REP MAC management message, the ABS shall send AAI_LBS-RSP.

The AAI_LBS-RSP MAC management message contains some LBS information, which may include identifier of the AMS, mode of LBS, and method of LBS measurement, metric to measure the location according to the measurement method. Mode of LBS is used to instruct the BS and/or the MS to perform LBS measurement and report location information.

15.2.x.c.2 LBS Operation in Idle State

Beside to the LBS operation in connected state, LBS should be supported for MS in idle state. AAI_PAG-ADV MAC management message with some parameter (i.e., action code, LBS method, LBS measurement metric, and etc.) is transmitted by ABS to make AMS start/end LBS and report location information.

For idle state AMS, the AMS may report the LBS measurement parameters (i.e., requested measurement metric, updated timer) during on paging. The idle state AMS, on being paged, should perform LBS ranging with the code set and region which are transmitted in the AAI_PAG-ADV MAC management message by ABS. An AAI_ RNG-REQ MAC management message with the ranging purpose indication (LBS measurement) is transmitted to an ABS when it receives an AAI_PAG-ADV with Action Code = 0b10 (Perform LBS measurement) or when any location information reporting condition is met. The AAI_RNG-REQ MAC management message includes the metric according to the method and the required metric from the ABS and preferred update timer to perform the next LBS measurement.

[modify and add the following rows into Table 729 – parameters for AAI_PAG-ADV message format]

| Name | Value | Usage |
| --- | --- | --- |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement(initiate/change) 0b11 = Reserved Stop performing LBS measurement | Paging action instruction to each paged AMS. |
| Dedicated ranging code and opportunity | [TBD] | Ranging to perform LBS measurement It may be included when Action Code = 0b10 |

[Adopt the following rows to the Table 682 – parameters for AAI_RNG-REQ]

| Name | Value | Usage |
| --- | --- | --- |
| … |  |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#5 is set to 1, it indicates that the AMS is attempting to report the LBS related metric to perform LBS measurement. Bit#56-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO, or location update, or LBS measurement |
| … |  |  |
| LBS GPS-based measurement metric | In case of GPS based method, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimated its location and the following parameters are included: Longitude Latitude Altitude | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it is included in the AAI_RNG-REQ LBS measurement metric transmitted by the AMS which is used to perform GPS based LBS. measurement |
| LBS Non-GPS-based measurement metric | In case of Non-GPS based method, LBS measurement may be supported in the DL and UL and according to the measurement metric, the following parameters are included: CINR RSSI Relative Delay (RD) Round Trip Delay (RTD) Time of arrival (ToA) Angle | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it is included in the AAI_RNG-REQ LBS measurement metric transmitted by the AMS which is used to perform Non-GPS based LBS. measurement |
| Preferred update timer | It indicates the next preferred time to perform LBS measurement | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it may be included in the AAI_RNG-REQ. It may help the ABS to transmit the next AAI_PAG-ADV |

[Adopt the following rows to the Table 683 – parameters for AAI_RNG-RSP]

| Name | Value | Usage |
| --- | --- | --- |
| LBS measurement indication | 0b0 = No LBS measurement 0b1 = Needs LBS measurement report | Indicates whether the LBS measurement report is needed or not. |
| LBS Method | 0b0 = GPS-based Method 0b1 = Non-GPS-based Method | Indicates a LBS measurement method. It shall be included in the AAI_RNG-RSP when the LBS measurement indication is set (0b1). It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| LBS measurement metric | 0b000 = CINR 0b001 = RSSI 0b010 = Relative Delay (RD) 0b011 = Round Trip Delay (RTD) 0b100 = Time of arrival (ToA) 0b101 = Angle 0b110-0b111 = Reserved | Indicates a measurement parameter when Non-GPS based method is used. It shall be included when LBS Method = 0b1 |
| Preferred timer | Preferred time to report the next LBS-REP | It indicates the next preferred time to perform LBS measurement. It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| Threshold | Threshold to compare the previous and current LBS measurement metric | It indicates the threshold to compare the previous and current LBS measurement metric. It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| Resource info. to report AAI_LBS-REP | [TBD] | UL resource info.(region, MCS) to report AAI_LBS-REP MAC management message It shall be included in the AAI_RNG-RSP when the LBS measurement indication is set (0b1). |

[Add the following rows to Table 675 – MAC Control Messages]

| Message name | Message description | Connection |
| --- | --- | --- |
| AAI_LBS-RSP | LBS Response | Basic |
| AAI_LBS-REP | LBS Report | Basic |

[Add the following new section]

15.2.6.x AAI_LBS-RSP

The ABS shall send AAI_LBS-RSP to initiate the LBS report, or to acknowledge the AAI_LBS-REP sent by the AMS. The AAI_LBS-RSP message shall include the following parameters:

Mode: indicates the start of LBS report or end of report. In case of Mode = 0b0, the AMS shall start the LBS report or change the LBS report method until the Mode = 0b1.

0b0: LBS on

0b1: LBS off

Method:

0b0: GPS-Based Method

0b1: Non-GPS-Based Method

In case of Method = 0b0, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimate its location.

In case of Method = 0b1, Non-GPS-Based Methods rely on the role of the serving and neighboring ABSs/ARSs. LBS related measurements may be supported in the DL and UL and according to the measurement metric, the following parameters are included:

CINR

RSSI

Relative Delay (RD)

Round Trip Delay (RTD)

Time of arrival (ToA)

Angle

Reporting Mode

0b0: Timer based location information reporting

0b1: Threshold based location information reporting

Report update value: next preferred update timer due to crossing the boundary of the cell or threshold to measure for moving distance threshold

Updated Timer (t)

Threshold

15.2.6.y AAI_LBS-REP

The AMS shall send AAI_LBS-REP to report the measure result according to the measurement metric requested by the ABS, or to request measure the location of the AMS itself.

Method:

0b0: GPS-Based Method

0b1: Non-GPS-Based Method

In case of Method = 0b0, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimated its location and the following parameters are included:

Longitude

Latitude

Altitude

In case of Method = 0b1, LBS measurement may be supported in the DL and UL and according to the measurement metric, the following parameters are included:

CINR

RSSI

Relative Delay (RD)

Round Trip Delay (RTD)

Time of arrival (ToA)

Angle

Report Timer: next preferred update timer due to crossing the boundary of the cell

Updated Timer (t)

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document,” July 2009

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 200