| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS Mobility Management in IEEE P802.16m (E-MBS) |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 E-MBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | E-MBS Support in detail of E-MBS mobility management on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Mobility Management in IEEE 802.16m (E-MBS)

Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes the support for Enhanced Multicast Broadcast Service(E-MBS) and Section 13 in SDD describes that an AMS can continue to receive E-MBS data from the ABS in connected state or idle state. 15.3.12.16 in IEEE 802.16m Amendment Draft Standard [2] also describes the operation of idle mode support MBS.

# 2 E-MBS Mobility Management

The AMS may continue to receive E-MBS transmissions from any ABS that is part of the E-MBS Zone, regardless of the AMS operating state—Connected State, Idle State.

When an AMS moves across the E-MBS zone boundaries and an ABS in another E-MBS zone is transmitting the same E-MBS flows, it can continue to receive E-MBS data from the ABS in Connected State or Idle State.

According to the current SDD[1], when an AMS moves across the E-MBS boundaries, it can continue to receive E-MBS data from the ABS regardless of its operating state. In Connected State, the AMS performs handover procedure for E-MBS. In Idle State, the AMS may transit to Connected State to perform handover or it may initiate E-MBS location update process for the purpose of E-MBS zone transition unless the AMS already has the MSTID mappings in the target E-MBS zone.

However, the current HO procedure and location update procedure are not sufficient for seamless transition from one E-MBS zone to another without any interruption of E-MBS data service and operation.

Therefore, in this contribution, some MAC management messages (i.e., AAI_NBR-ADV, AAI_HO-CMD, AAI_RNG-REQ, and AAI_RNG-RSP) are modified to support the E-MBS HO and E-MBS location update.

# 3. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard or C802.16m-09/2166

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text or C802.16m-09/2166

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text or C802.16m-09/2166

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following rows into Table xxx-parameters for AAI_HO-CMD in 15.2.6.9 AAI_HO-CMD of 802.16m/D1]

| Name | Value | Usage |
| --- | --- | --- |
| Mode | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS Ho request rejectes (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| N_New MBS Zones | Number of target ABS supports E-MBS Zones which is different from those supported by serving ABS | It may be included only if the Mode = 0b00 for each target ABS which supports E-MBS each target ABS which supports E-MBS |
| MBS Zone Identifier(s) | The new MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. |  |
| [Multicast STID] | [The MSTID which the AMS shall maintain in any state on being served E-MBS for each E-MBS Zone.] |  |
| Service Flow Encoding(FID, service flow related parameter(s)) | mapping of the flow IDs used in E-MBS Zone supported by the serving ABS to flow IDs to be used in E-MBS Zone supported by the target BS. New Flow IDs are included in the increasing order of the corresponding old Flow IDs. |  |

[Adopt the following row into Table 682 – parameters for AAI_RNG-REQ of 802.16m/D1]

| Name | Value | Usage |
| --- | --- | --- |
| E-MBS Zone Change Indicator | Indicates the AMS is currently attempting to perform location update due to E-MBS Zone change. | It shall be included only if the #3(location update) of Ranging purpose indication is set. |
| E-MBS related information | Current [MSTID, FID][MFID], MBS Zone ID | If the E-MBS Zone Change Indicator is set, it shall be include in the AAI_RNG-REQ. |

[Adopt the following rows into Table 683-parameters for AAI_RNG-RSP of 802.16m/D1]

| Name | Value | Usage |
| --- | --- | --- |
| [Multicast STID] | [The MSTID which the AMS shall maintain in any state on being served MBS.] | It may be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to update. |
| E-MBS Zone Identifier | The new E-MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. |  |
| Service Flow Encoding(FID, service flow related parameter(s)) | An identifier of a service flow. It pairs with [MSTID][E-MBS Zone ID] in the E-MBS MAP message and it is allocated to each E-MBS connections. As a result, an AMS can receive multiple MBS traffics for an E-MBS connection with different E-MBS contents distinguished by the FID belonging to a [MSTID][E-MBS Zone]. |  |

[Modify 15.2.6.10 on page 57 of 802.16m/D1 as follows]

15.2.6.10 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

1) ABS type (macro, micro, femto, relay, TBD)

2) carrier frequency

3) MAC version

4) TDD/FDD and related definitions (expected to be the same given carrier frequency)

5) BW, CP info: may not be required if carried in P/S-SCH channel

6) Multi-carrier capability and configuration

7) E-MBS zone related information (E-MBS Zone ID)

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

[Modify 15.8.2.1 on page 5 of C802.16m-09/2166 as follows]

15.8.2.1 E-MBS Operation in Connected State

When the AMS transits to a new E-MBS Zone while in Active Mode or Sleep Mode, and the E-MBS service flow management encodings of the AMS have not been updated, the ABS may include MSTID and FID Update in REG-RSP encoding TLV in the RNG-RSP to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

Prior to the HO preparation phase, the AMS continues to monitor AAI_NBR-ADV MAC Management message to determine whether the target preferred BS belongs to the same E-MBS Zone. An ABS that supports E-MBS shall include the E-MBS zone related information in the AAI_NBR-ADV MAC management message.

If the target ABS is in the same E-MBS Zone (i.e., intra MBS Zone), the AMS should refer to part of or all the information E-MBS MAP to continue to receive the E-MBS DL transmission. During the MBS HO within E-MBS Zone, E-MBS related context information (e.g., MSTID, E-MBS Zone ID) is preserved and accessible by the serving HO function.

If the target ABS is not the same E-MBS Zone (i.e., inter MBS Zone), the AMS is required to retrieve the new target E-MBS Zone parameters such as MSTID, E-MBS Zone ID, etc. To support E-MBS HO to retrieve the new target E-MBS Zone parameter during HO, new target E-MBS Zone parameters included in the HO related MAC management message (e.g., AAI_HO-CMD). If the AMS does not acquire the new target E-MBS Zone parameters, it acquires them from the target AMS during network re-entry using ranging procedure. In this case, the AMS shall transmit the Ranging Purpose Indication using the AAI_RNG-REQ MAC management message. In the response to the AAI_RNG-REQ MAC management message, the target ABS shall allocate the MBS information using AAI_RNG-RSP MAC management message including MBS zone identifier, MSTID, FID, and etc.

[Modify 15.8.2.2 on page 5 of C802.16m-09/2166 as follows]

15.8.2.2 E-MBS Operation in Idle State

When an AMS in Idle mode moves to an ABS which does not belongs to AMS’ previous E-MBS Zone, the AMS is expected to update the E-MBS service flow management encodings at that ABS to provide for continuous  reception of E-MBS content. The AMS may obtain the E-MBS information in the target E-MBS zone through broadcast messages in the serving EMBS-Zone.. If the such idle AMS has not received such information from the serving E-MBS Zone, the AMS Shall use location update procedure to acquire updated E-MBS service flow management encodings. In order to perform the MBS location update process, the AMS shall transmit AAI_RNG-REQ MAC management message including E-MBS Zone Change Indicator. In the response to the MBS location update, the ABS shall transmit AAI_RNG-RSP MAC management message which may include the E-MBS zone identifier, MSTID, FID, Packet Classification Rule parameters, and E-MBS Zone Identifier Assignment parameter and etc. to provide update service flow management encodings for any affected E-MBS flow(s).

The AMS  may also conduct re-entry from Idle mode if required by quality of service of corresponding EMBS flow..

The ABS may include STID_Update in REG-RSP encoding TLV in the RNG-RSP to provide updated service flow management encodings for any affected E-MBS flow.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document,” July 2009

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 2009