| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Control Structure for E-MBS Support in IEEE 802.16m (E-MBS) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 E-MBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | E-MBS Support in detail of Downlink Control Structure on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Downlink Control Structure for E-MBS Support in IEEE 802.16m (E-MBS)

Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes the support for Enhanced Multicast Broadcast Service(E-MBS) and Section 13 in SDD describes some primitive concept of E-MBS, which is not enough to cover general concept of support E-MBS on the next version of IEEE 802.16m Amendment Draft Standard [2] . Therefore, in this contribution, we describe the detail description of Downlink Control Structure on E-MBS operation in IEEE 802.16m for the 802.16m Amendment Draft Standard.

# 2. E-MBS operation and frame and control structure on the 802.16m Amendment Draft Standard

In unicast/multicast mixed carrier, E-MBS uses the same frame structure used for unicast carrier. The E-MBS data is multiplexed with Unicast traffic. The S-SFH indicates E-MBS region which may span over multiple subframes for each E-MBS zone. E-MBS subframes are allocated with fixed pattern for each FID until the next E-MBS MAP message. Each E-MBS MAP message may support one or more E-MBS services within an E-MBS zone. E-MBS MAP is transmitted in the pre-defined location (i.e., the beginning of the E-MBS region). E-MBS MAP indicates the next E-MBS MAP message and describes the burst information of the corresponding MSTID. The pattern may vary between E-MBS MAP messages. Figure xx illustrates the frame structure when E-MBS subframes are present in superframes.

Figure xx: Illustration for E-MBS support in Mixed Broadcast/Unicast Carrier

For unicast/multicast mixed carrier, the control channel design to support E-MBS is as follows.

S-SFH

- Indicates whether the E-MBS Region in included in the superframe (up to N superframe)

- If the E-MBS region is included in the superframe

  - Region Information for multicast/unicast

  - Indicates whether the E-MBS MAP related information is included in the superframe

    - If the E-MBS MAP related information is included in the superframe

    - Provides pointers to help AMS find the location of the E-MBS MAP message

E-MBS MAP message

- Provides location and region information to help AMS find the location of the next E-MBS message

- For each MSTID

  - Number of FID which is supported in the E-MBS Zone

  - E-MBS burst location and region information for each FID (describes each region of the corresponding FID)

In the each E-MBS burst includes the following information in the each burst

- Location and region information of the Next E-MBS burst information (may be located in the pre-defined location(beginning of the each E-MBS burst)

# 3. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16m Amendment Draft Standard]

15. Advanced Air Interface

15.3 Physical Layer

15.3.6 Downlink Control Structure

15.3.6.5 DL Control Information Elements

15.3.6.5.3 Broadcast Control Information Elements

15.3.6.5.3.2 S-SFH IE

Table 674 – S-SFH SP1 IE format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| SFH SP1 IE () { |  |  |
| … |  |  |
| Region Length | [TBD] | 0: no MBS region in the superframe >0: length of the superframe bitmap |
| if (Region Length > 0) { |  |  |
| Superframe Bitmap | [TBD] | Size is the same as region length 1: multicast is included in the superframe 0: unicast only in the superframe |
| Frame Bitmap | 4*( number of bit set in the superframe bitmap) | Maximum length = (number of frame in an superframe) * (length of superframe); Length = (number of frame in an superframe) * (number of set bit in the superframe bitmap) 1: multicast is included in the frame 0: unicast only in the frame |
| Subframe Bitmap | (number of subframe in an frame) *(number of bit set in the frame bitmap) | Maximum length = (number of subframe in an frame) * (Max. length of frame bitmap) Length = (number of subframe in an frame) * (length of frame bitmap) 1: multicast is included in the subframe 0: unicast only in the frame |
| E-MBS MAP Indication | 1 | indicates whether MBS MAP is addressed or not in this SF |
| if(E-MBS MAP indication is set) |  |  |
| Frame Index | 2 | Frame location of the first E-MBS MAP message 0b00: 1st frame 0b01: 2nd frame 0b10: 3rd frame 0b11: 4th frame |
| Subframe Index | 3 | Subframe location in the frame of the first E-MBS MAP message 0b000: 1st subframe of the frame … 0b111: 8th subframe of the frame |
| MCS | [4] | MCS for the first MBS MAP message Depends on supported modes, 16modes |
| Resource Information | [TBD] | Region information of the first MBS MAP message |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

Region Length: Indicates the number of superframe which may includes multicast region

0: no E-MBS region is included in the superframe

Region Length(N) > 0: N superframe may includes multicast region

Superframe Bitmap: Indicates the superframe whether it includes multicast region

0: no E-MBS region in the superframe (only unicast region in the superframe)

1: E-MBS region in the superframe

Frame Bitmap: Indicates the frame whether it includes multicast region

0: no E-MBS region in the frame (only unicast region in the frame)

1: E-MBS region in the frame

Subframe Bitmap: Indicates the subframe whether it includes multicast region

0: no E-MBS region in the subframe (only unicast region in the subframe)

1: E-MBS region in the subframe

E-MBS MAP Indication: indicates whether MBS MAP is addressed or not in this SF

Frame Index: indicates frame location of the first E-MBS MAP message

0b00: 1st frame
0b01: 2nd frame
0b10: 3rd frame
0b11: 4th frame

Subframe Index: indicates subframe location in the frame of the first E-MBS MAP message

0b000: 1st subframe of the frame
…
0b111: 8th subframe of the frame

MCS: describes MCS for the first MBS MAP message
Resource Information: Region information of the first MBS MAP message

15.3.6.5.4 Unicast Control information Elements

15.3.6.5.4.x E-MBS MAP message

E-MBS MAP message is a control channel to support E-MBS and its location information is pointed by SFH. Each E-MBS MAP may support one or more E-MBS services within an MBS zone. E-MBS MAP message is transmitted in the pre-defined location (i.e., the beginning of the E-MBS region). E-MBS MAP may indicate the next E-MBS MAP message and describes the burst information of the corresponding FID as follows.

- Next E-MBS MAP message information

  - Location of the Next E-MBS MAP message

    - Frame index ~ the frame offset of the first E-MBS MAP

    - Subframe index ~ Subframe number in the frame of the first E-MBS MAP

  - MCS

  - Region information ~ Region of the next E-MBS MAP message

- Number of the FID (describes each region of the corresponding FID)

- E-MBS Burst Information for each FID (describes each region of the corresponding FID)

  - E-MBS Burst position

    - Frame offset ~ the frame offset of the first E-MBS Burst

    - Subframe offset ~ Subframe number in the frame of the first E-MBS Burst

  - MCS ~ all E-MBS burst of each FID is the same MCS

  - Region information ~ Region of the first transmitted E-MBS Burst

In the each E-MBS burst includes the following information (may be located in the pre-defined location(beginning of the each E-MBS burst) in the each burst

- Location of the Next E-MBS burst information

  - Frame offset ~ the frame offset of the E-MBS Burst

  - Subframe offset ~ Subframe number in the frame of the E-MBS Burst

- Region Information ~ Region of the next transmitted E-MBS Burst

Within a subframe where multicast data and E-MBS MAPs are carried, E-MBS MAPs and data channels are multiplexed using FDM.

Table xxx describes the E-MBS MAP message format and Table xxy describes the E-MBS Region Message format. The E-MBS Region Message shall be in the beginning of each E-MBS burst.

Table xxx – E-MBS MAP Message format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| E-MBS MAP Message format () { |  |  |
| # of MSTID | [TBD] | # of E-MBS Zone ID included in the E-MBS MAP IE |
| for(i=0; i<# of MSTID; i++){ |  |  |
| MSTID | 12 | Multicast STID |
| Next E-MBS MAP superframe offset | [TBD] | 0: current superframe 1: superframe number (current + 1) … n: superframe number (current + n) |
| Next E-MBS MAP frame offset | 2 | for the MSTID Frame location of the next E-MBS MAP 0b00: 1st frame 0b01: 2nd frame 0b10: 3rd frame 0b11: 4th frame |
| Next E-MBS MAP subframe offset | 3 | for the MSTID Subframe location in the frame of the next E-MBS MAP 0b000: 1st subframe of the frame … 0b111: (7+1)th subframe of the frame |
| MCS | [4] | MCS for MBS MAP message Depends on supported modes, 16modes |
| Resource Information | [TBD] | Region Information for the next MBS MAP message |
| # of FID | [TBD] | # of FIDs |
| for(k=0;k<#of FID;k++){ |  |  |
| FID | 4 | Multicast FID for MSTID |
| Frame offset for MBS Region | 2 | for the FID Frame location of the next E-MBS Region message 1: current frame 2: (current frame + 1)th frame … n: (current frame + n)th frame |
| Subframe offset for MBS Region | 3 | for the FID Subframe location in the frame of the next E-MBS Region message 0b000: 1st subframe of the frame … 0b111: (7+1)th subframe of the frame |
| MCS | [4] | MCS for the E-MBS region of FID Depends on supported modes, 16modes |
| Resource Information | [TBD] | Region Information of E-MBS region for the FID |
| } |  |  |
| } |  |  |

Table xxy – E-MBS Region Message format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| E-MBS Region Message format() { |  |  |
| Frame offset for the next MBS region | 2 | Frame location of the next E-MBS Region message 1: current frame 2: (current frame + 1)th frame … n: (current frame + n)th frame |
| Subframe offset for the next MBS region | 3 | Subframe location of the frame of the next E-MBS Region message 0b000: 1st subframe of the frame … 0b111: (7+1)th subframe of the frame |
| Resource Information | [TBD] | Region Information for the next E-MBS Region |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document,” July 2009

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 2009