| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Emergency Alert Indication in AAI_PAG-ADV (15.3.12.2.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI Hyunjeong Kang Samsung Electronics Chun-Yen Hsu, Chiu-Wen Chen Institute for Information Industry Lei Zhang ZTE Corporation Giwon Park, Kiseon Ryu LG Electronics | Email: mhtao@itri.org.tw hyunjeong.kang@samsung.com hcy@iii.org.tw zhang.lei13@zte.com.cn gwpark@lge.com |
| Re: | Category: LB30 / Area: Chapter 15.3.12 (Idle mode) |  |
| Abstract | This contribution proposes changes to the Section 15.3.12.2.1 and 15.3.12.2.3 of IEEE P802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Emergency Alert Indication in AAI_PAG-ADV (15.3.12.2.1)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Hyunjeong Kang

Samsung Electronics

Chun-Yen Hsu, Chiu-Wen Chen

Institute for Information Industry

Lei Zhang

ZTE Corporation

Giwon Park, Kiseon Ryu

LG Electronics

- Introduction

In current D1 [1], there are some sentences describing that the AAI_PAG-ADV message shall include the emergency alert indication, thus the AMS in idle mode will know when it has to decode the A-A-MAP to obtain the emergency information. Since there is no such indication in AAI_PAG-ADV in current D1, we propose text modifications to Table 729 (AAI_PAG-ADV message) so that the alter indication is included. In addition, we gather all the sentences related to emergency alert indication within one paragraph for better reading.

- Proposed Text for IEEE P802.16m/D1

[Adopt the following modifications marked in blue]

============================ Start of Proposed Text ==================================

#### 15.3.12 Idle mode

#### 15.3.12.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement. In addition, an emergency alert indicator is included in the paging message to notify the idle AMSs receoption reception of the emergency information. Upon reception of the emergency alert indicator that is set to ‘1’, the AMS shall decode the A-A-MAP to obtain the emergency information.

Table 729—Parameters for AAI_PAG-ADV Message Format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |
| Emergency Alert Indication | 0 = There is no emergency information 1 = There is emergency information | If the emergency alert indicator is set to ‘1’, the AMS shall decode the A-A-MAP to obtain the emergency information. |

#### 15.3.12.2.3 Operation during paging listening interval

The AMS derives the start of the paging listening interval based on the paging cycle and paging offset. The paging listening interval shall comprise of the superframe whose superframe number Nsuperframe meets the condition.

Nsuperframe modulo PAGING_CYCLE == PAGING_OFFSET

The length of the paging listening interval is one superframe.

At the beginning of the paging listening interval, the AMS shall scan and synchronize on the A-PREAMBLE

of its preferred ABS and decode the P-SFH of the ABS. Upon reception of the emergency alert indicator

that is set to ‘1’, the AMS shall decode the A-A-MAP to obtain the emergency information.

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D1. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, July 2009.

- IEEE 802.16m-08/003r9. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r8. IEEE 802.16m System Requirements Document, Jan 2009.