| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARS Initial Ranging |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Ting Lin, Jiun-Je Jian, Tsung-Yu Tsai, Chun-Yen Hsu Institute for Information Industry (III) | Voice: +886-2-66000100 E-mail: loa@iii.org.tw |
| Re: | P802.16m/D1 Relay |  |
| Abstract | This contribution proposes contention based ARS initial ranging |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARS Initial Ranging

Kanchei(Ken) Loa, Yi-Ting Lin, Jiun-Je Jian, Tsung-Yu Tsai, Chun-Yen Hsu

Institute for Information Industry (III)

# Introduction

In this contribution, we propose the procedure of ARS initial ranging.

# Proposed Changes

[Insert the following section 15.x “Contention-based initial ranging and automatic adjustments” :]

15.x Contention-based initial ranging and automatic adjustments

The initial ranging procedure for ARS is the same as that for AMS unless otherwise specified in this section:

- After receiving an initial ranging preamble from an ARS, the ABS may send an AAI_RNG-ACK containing status = abort with cell IDs of candidate neighbor access stations.

- Upon receiving an AAI_RNG-ACK containing status abort with cell IDs, the ranging ARS shall scan for a DL channel of candidate neighbor access stations and performs initial ranging.