| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SON Support in AMS Idle Mode (SON) |  |
| Date Submitted | 2009-08-31 |  |
| Source(s) | Chun-Yen Hsu, Yung-Lan Tseng, Chiu-Wen Chen, Kanchei (Ken) Loa, Yi-Ting Lin, Tsung-Yu Tsai, Youn-Tai Lee Institute for Information Industry (III) Shiann-Tsong Sheu, Chih-Cheng Yang National Central University (NCU) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) Yih-Guang Jan, Yang-Han Lee, Ming-Hsueh Chuang, Hsien-Wei Tseng Tamkang University (TKU) | Voice: +886-2-66000100 E-mail: hcy@iii.org.tw |
| Re: | IEEE 802.16m/D1 SON |  |
| Abstract | This contribution gives comments on Idle Mode Support for in SON |  |
| Purpose | To be discussed and adopted in IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SON support in AMS Idle Mode

Chun-Yen Hsu, Yung-Lan Tseng, Chiu-Wen Chen, Kanchei (Ken) Loa, Yi-Ting Lin, Tsung-Yu Tsai, Youn-Tai Lee

Institute for Information Industry

Shiann-Tsong Sheu, Chih-Cheng Yang
National Central University

Whai-En Chen, Shih-Yuan Cheng
National Ilan University

Yih-Guang Jan, Yang-Han Lee, Ming-Hsueh Chuang, Hsien-Wei Tseng
Tamkang University

# Introduction

In D1 [3], an idle AMS may be paged to perform LBS measurement. An idle AMS should be able to perform SON measurements as well.

# Reference

[1] IEEE 802.16m-08/0034,” IEEE 802.16m System Description Document [Draft]” July 27, 2009.

[2] IEEE 802.16m-09/0010r2,” Draft Amendment to IEEE Standard for Local and metropolitan area networks”. June 1,2009.

[3] IEEE P802.16m/D1,” DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”. July 2009

----------------------------------------------------------------------------------------------------------------------------------------

# Proposed text

[Insert the following text in P80216m /D1 page 207 line 52]

15.3.12.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement or SON measurement.

[Insert the following text in P80216m /D1 page 207 line 63]

The Paging message also includes an action code directing each AMS notified via the inclusion of its identifier as appropriate:

•0b00: Perform network re-entry

•0b01: Perform ranging to establish location

•0b10: Perform LBS measurement

•0b11: Perform SON measurement

[Modify Table 729 in P80216m /D1 as indicated]

Table 729—Parameters for AAI_PAG-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Perform SON measurement | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a pre-determined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |

[Insert the following text to P80216m /D1 15.3.12.8 as indicated]

15.3.12.8 Idle Mode Support for SON/Femto

Idle AMS may support SON measurement. The ABS signals an idle AMS that is requested to perform SON measurement through setting the Action Code to ‘11’ in the AAI_PAG-ADV message. After receiving the paging message with the Action Code of ’11’, the paged AMS performs SON measurement and reports the measurement results.