| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS Capability Negotiation (LBS) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Chun-Yen Hsu, Yung-Lan Tseng, Kanchei (Ken) Loa, Youn-Tai Lee Institute for Information Industry | Voice: +886-2-66000100 E-mail: hcy@iii.org.tw |
| Re: | IEEE 802.16m/D1 LBS |  |
| Abstract | This contribution gives comments on the LBS capability negotiation. |  |
| Purpose | To be discussed and adopted in IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LBS Capability Negotiation

Chun-Yen Hsu, YungLan Tseng, Kanchei (Ken) Loa, Youn-Tai Lee

Institute for Information Industry (III)

# Introduction

In this contribution we give comments for the LBS capability negotiation. ABS & AMS negotiates the capabilities of LBS through AAI_REG-REQ/AAI_REG-RSP message.

# Reference

[1] IEEE 802.16m-08/0034,” IEEE 802.16m System Description Document [Draft]” July 27, 2009.

[2] IEEE 802.16m-09/0010r2,” Draft Amendment to IEEE Standard for Local and metropolitan area networks”. June 1,2009.

[3] IEEE P802.16m/D1,” DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”. July 2009.

----------------------------------------------------------------------------------------------------------------------------------------

# Proposed text

[Replace the following table 684 in P80216m /D1 page 52 as indicated]

Table 684—parameters for AAI_ REG-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capabilities Negotiation Process | ARQ parameters | ARQ_SN_MODULUS | The number of unique SN value;default=2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks. |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the trans- mitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEO UT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | LBS capabilities | GPS capability | 0: AMS is GPS capable. 1: AMS is GPS incapable. |
|  |  | Reserved | TBD |
|  | Co-located coexistence capability supported |  | Indicates if colocated coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capa- bilities | Classification/PHS options and SDU encapsu- lation support |  | Indicates which classification/PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classi- fication rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |

[Replace the following table 685 in P80216m /D1 page 54 as indicated]

Table 685—parameters for AAI_ REG-RSP message

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capabilities Negotiation Process | ARQ parameters | ARQ_SN_MODULUS | The number of unique SN value;default=2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks. |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with Rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the trans- mitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEO UT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | LBS capabilities | LBS measurement required | 0: AMS does not need to perform LBS measurement 1: AMS may need to perform LBS measurement |
|  |  | Parameters of timer based Reporting mode | The length of timer threshold. |
|  |  | Parameters of distance based Reporting mode | The length of distance threshold. |
|  | Co-located coexistence capability supported |  | Indicates if colocated coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capa- bilities | Classification/PHS options and SDU encapsu- lation support |  | Indicates which classification/PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classi- fication rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |