| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Design of the BR ACK A-MAP IE (D1 - 15.2.11.1.1) |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim LG Electronics Kelvin Chou, Yih-Shen Chen MediaTek Inc. Yanling Lu, Kevin Power, Masato Okuda Fujitsu Lei Wang InterDigital Communications Jaehyuk Jang Samsung Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com E-mail : kelvin.chou@mediatek.com yihshen.chen@mediatek.com E-mail: Yanling.lu@cn.fujitsu.com Kevin.Power@uk.fujitsu.com okuda@jp.fujitsu.com E-mail: leiw@billeigean.com E-mail: jack.jang@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | The contribution proposes details for BR ACK A-MAP IE included in the 802.16m DRAFT amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Design of the BR ACK A-MAP IE (D1 - 15.2.11.1.1)

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim

LG Electronics

Kelvin Chou, Yih-Shen Chen

MediaTek Inc.

Yanling Lu, Kevin Power, Masato Okuda

Fujitsu

Lei Wang

InterDigital Communications

Jaehyuk Jang

Samsung Electronics

- Introduction

This contribution includes the proposed text for the BR ACK A-MAP IE in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

In the IEEE 802.16m DRAFT Amendment, UL resource allocation based on BR ACK has some missing or ambiguous explanation. Therefore, we propose some details and optimized BR ACK for UL resource allocation.

- Modifications and key-descriptions

- The main concepts of the BR ACK from “15.2.11.1.1 Contention-based random access bandwidth request” in the IEEE P802.16m DRAFT Amendment Working Document [6] are maintained in this text.

- The clarification of the operation for UL resource allocation

    - Allocation Order / Size for UL resource

      - There is no description of the order and size of allocating UL resource to the BR code index whose grant indicator is ‘1’.

      - Therefore, we propose that the UL resource should be allocated to the code index whose grant indicator is ‘1’ in ascending order based on the code index.

      - Also, we propose that BR ACK A-MAP IE should include an allocation size field.

Resource for code index 2

Resource start offset ‘1’

Resource for code index 7

Allocation size ‘2’

LRU 0

1

2

3

4

5

    - HFA (HARQ Feedback Allocation) for HARQ

      - To notify AMS of decoding status of the BW-REQ message, HARQ needs to be enabled.

        - For supporting HARQ, HFA (HARQ Feedback Allocation) and ACID (HARQ channel identifier) are required.

        - ACID can be omitted.

      - Therefore, we propose that each HFA should be allocated by using HFA start offset as using resource start offset for UL resource allocation.

- Optimization using 2bit-MSB of the resource start offset

    - The maximum number of LRUs is 96 for 20 MHz.

    - Because 95 are 0b1011111, there is no possibility that 2bit-MSB of the resource start offset in BR ACK A-MAP IE becomes 0b11.

    - If we use this as an indicator, we can increase the capacity of BR ACK-A-MAP IE.

      - If 2bit-MSB !=0b11, allocation information will be transmitted in the BR ACK-A-MAP IE.

      - If 2bit-MSB == 0b11, no allocation information will be included in this BR ACK-A-MAP IE.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[7] IEEE S802.16m-09/1830, “Design of the BR ACK A-MAP IE”

- Text proposal for the 802.16m DRAFT amendment

Remedy 1

On the IEEE P802.16m/D1, page 154, line 1, make the following changes to subsection 15.2.11.1.1 and,

On the IEEE P802.16m/D1, page 357, line 23, add a new subsection (15.3.6.5.4.13 BR-ACK A-MAP IE) for BR-ACK A-MAP IE and move the following paragraph and Table 715 from the section 15.2.11.1.1 to the new subsection)

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.11.1.1 Contention-based random access bandwidth request

The BR Acknowledgement A-MAP IE indicates the decoding status of the BR opportunities in the previous UL frame. All the successfully received preamble sequences, if present, shall be included in an ascending order. It In addition, the BR-ACK A-MAP IE also includes the allocation information only for the fixed-sized of bandwidth such as BW-REQ message BR header. The UL resource and HFA shall be allocated to the preamble sequences whose grant indicator is ‘1’. The allocations shall be ordered based on the index of preamble sequences arranged in an ascending order. The maximum number of the HARQ retransmissions is [N_MAX_ReTx_RA].

Table 715—BR-ACK A-MAP IE Format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| ACK A-MAP IE() { | - | - |
| A-MAP Type | 4 | BR ACK A-MAP IE |
| BR-ACK Bitmap | N_BR_Slots Opportunities | Each bit indicates the decoding status of BR code preamble sequence in the corresponding BR opportunity. 0b0: No BR code preamble sequence is detected, 0b1: At least one code preamble sequence is detected. |
| MSB of resource start offset | 2 | 0b00, 0b01, 0b10 :2bit-MSB of the start offset of the resource allocation (LRU) 0b11: No grant exist in this BR ACK A-MAP IE. |
| If (MSB of resource start offset !=0b11) { |  |  |
| LSB of resource start offset | TBD 5 | This field is LSB of the start offset of the Resource allocation (LRU) for BR Header. |
| HFA start offset | 3 | This field is start offset of HARQ Feedback Allocation. |
| Allocation size | 1 | Resource size in a subframe which is allocated to a BR preamble sequence index 0b0: 1 LRU, 0b1: 2 LRUs |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D, is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| } |  |  |
| For (i=0; N_BR_ SlotsOpportunities; i++) { |  |  |
| If (BR-ACK Bitmap[i] == 1) { |  |  |
| Number of Received code preamble sequences (L) | TBD | The number of BR code preamble sequence indices included in this ACK A-MAP IE. |
| For (j=0; j<L; j++) { |  |  |
| Code Preamble sequence index | 5 | Code Preamble sequence index received in the BR opportunity |
| MSG decoding indicator | 1 | To indicate the decoding status of quick access message |
| If (MSB of resource start offset !=0b11) { |  |  |
| Grant indicator | 1 | To indicate whether grant of BR Header for the code preamble sequence index is included or not. If this bit is set, the UL resource is allocated with fixed size and MCS. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| MCRC | 16 | CRC masked by the reserved STID for BR ACK A-MAP |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 2

On the IEEE P802.16m/D1, page 328, line 31, make the following changes to subsection 15.3.6.5.4.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.4.2 DL basic assignment A-MAP IE

Table 669 describes Assignment A-MAP IE Types.

Table 669—A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Feedback Allocation A-MAP IE |
| 0b0011 | Reserved |
| 0b0100 | CDMA Allocation A-MAP IE |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved BR-ACK A-MAP IE |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-----------------------------------------------------------End of the Text---------------------------------------------------------