| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Design of the S-SFH IE for Updating NBR-ADV Message (D1 - 15.3.6.5.3.2) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | The contribution proposes design of the S-SFH IE so that the idle mode AMS can know in advance when NBR-ADV message is transmitted. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Design of the S-SFH IE for Updating NBR-ADV Message (D1 - 15.3.6.5.3.2)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim

LG Electronics

- Introduction

This contribution includes the proposed text for the S-SFH SP3 in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

According to the RPD [7] of WiMAX Forum, it is recommended that the MS always scan the neighbors defined in the MOB_NBR-ADV message. Since it is beneficial for the idle MS to use the MOB_NBR-ADV for scanning purposes, it is recommended that MS obtain the new MOB_NBR-ADV as soon as possible after it reselects into a new BS. The BS may assist the MS to receive updated MOB_NBR-ADV by sending MOB_NBR-ADV inside a Paging Listening Interval or shortly after a paging listening interval.

However, it is a burden to BS because of the followings:

- Since Paging Cycles can be different from each other, BS may frequently transmit MOB_NBR-ADV.

- The size of MOB_NBR-ADV message is relatively big.

The IEEE 802.16m systems need to lighten the burden. Therefore, we propose an efficient scheduling and updating method to minimize the effort of the idle mode AMS.

- Modifications and key-descriptions

- The main concepts from “15.3.6.5.3.2 S-SFH IE” in the IEEE P802.16m DRAFT Amendment Working Document [6] are maintained in this text.

- ABS needs to inform AMSs of superframes where NBR-ADV message is transmitted.

- We propose that the scheduling information should be sent through S-SFH SP3.

- Period (p) is used for calculating transmission interval of NBR-ADV message

  - The first superframe number in an interval meets the following condition :

Nsuperframe modulo 2p == 0

- Offset (o) is superframe offset between the first superframe in the interval and the superframe where NBR-ADV message is transmitted.

- In case that period is 3 and offset is 2, NBR-ADV message is sent as shown in the following figure :

Superframe #0

Interval = 8

Offset = 2

NBR-ADV transmission

Interval = 8

Superframe #1

Superframe #2

Superframe #3

Superframe #4

Superframe #5

Superframe #6

Superframe #7

Superframe #8

Superframe #9

Superframe #10

Superframe #11

Offset = 2

Frame #0

Frame #1

Frame #2

Frame #3

DL subframes

UL subframes

NBR-ADV transmission

DL subframes

UL subframes

Frame #0

….

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[7] TWG_Motorola_04r03_Idle Mode, “RPD for Idle Mode clarification”

- Text proposal for the 802.16m DRAFT amendment

Remedy 1

On the IEEE P802.16m/D1, page 322, line 63, make the following changes to subsection 15.3.6.5.3.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.3.2 S-SFH IE

Table 676—S-SFH SP3 IE format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| SP scheduling periodicity information | TBD |  |
| NBR-ADV scheduling periodicity information | 6 | It indicates transmission interval and offset of the AAI_NBR-ADV message. Bit 0-3 : it indicates period (p). Value of range 0 ~ 15. 2p is transmission interval of AAI_NBR-ADV message. The first superframe number in an interval meets the following condition: Nsuperframe modulo 2p == 0. Bit 4-5 : it indicates superframe offset (o) between the first superframe in an interval and the superframe at which NBR-ADV message is transmitted. Value of range 0 ~ 3 |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 2

On the IEEE P802.16m/D1, page 57, line 52, make the following changes to subsection 15.2.6.10:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.6.10 AAI_NBR-ADV

An AAI_NBR-ADV message is transmitted by ABS to identify the network and define the characteristics of neighbor ABS. ABS shall transmit the AAI_NBR-ADV message at superframes decided by the NBR-ADV scheduling periodicity information (period and offset) of S-SFH SP3.

As an example, Figure xxx illustrates transmission of the AAI_NBR-ADV message.

Superframe #n

Transmission Interval (2p)

Offset

Superframe at which AAI_NBR-ADV is transmitted

Transmission Interval

Superframe #n+1

Superframe #n+2

Superframe #n+3

Superframe #m-1

Superframe #m

Superframe #m+1

Superframe

# m+2

Superframe

# m+3

Superframe #

m+4

Offset

……

Figure xxx - Example of transmission of AAI_NBR-ADV message

AMS may try to receive the latest AAI_NBR-ADV message by using the NBR-ADV scheduling periodicity information in S-SFH SP3.

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

-----------------------------------------------------------End of the Text---------------------------------------------------------