| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Design of the Extended Non-user-specific A-MAP for Updating SCD and NBR-ADV Message (D1 - 15.3.6.5.4.1) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | The contribution proposes design of the extended NUS A-MAP so that AMS can know in advance whether any values of the SCD and NBR-ADV message change or not. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Design of the Extended Non-user-specific A-MAP for Updating SCD and NBR-ADV Message (D1 - 15.3.6.5.4.1)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim

LG Electronics

- Introduction

This contribution includes the proposed text for the extended NUS A-MAP in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

According to the RPD [7] of WiMAX Forum, it is recommended that the MS always scan the neighbors defined in the MOB_NBR-ADV message. Since it is beneficial for the idle MS to use the MOB_NBR-ADV for scanning purposes, it is recommended that MS obtain the new MOB_NBR-ADV as soon as possible after it reselects into a new BS. The BS may assist the MS to receive updated MOB_NBR-ADV by sending MOB_NBR-ADV inside a Paging Listening Interval or shortly after a paging listening interval.

However, it is a burden to BS because of the followings:

- Since Paging Cycles can be different from each other, BS may frequently transmit MOB_NBR-ADV.

- The size of MOB_NBR-ADV message is relatively big.

The IEEE 802.16m systems need to lighten the burden. Therefore, we propose an efficient scheduling and updating method to minimize the effort of the idle mode AMS.

- Modifications and key-descriptions

- The main concepts from “15.3.6.5.4.1 Non-user-specific A-MAP IE” in the IEEE P802.16m DRAFT Amendment Working Document [6] are maintained in this text.

- ABS needs to inform AMSs of version information of the SCD and NBR-ADV message.

- We propose that the information should be sent through the extended NUS A-MAP.

- An extended NUS part should include 1-bit type field.

  - According to type, the extended NUS part includes different information.

    - If type is ‘0’, it includes resource allocation information for additional broadcast messages.

    - If type is ‘1’, it includes version information for SCD and NBR-ADV message.

  - By receiving version information, AMS can know in advance whether any values of SCD and NBR-ADV message are changed or not.

- In case that only type ‘0’ is transmitted, an extended NUS part should follow right after the NUS A-MAP part.

| Part | Syntax | Size |
| --- | --- | --- |
| NUS A-MAP part | Assignment A-MAP size | TBD |
|  | HF-A-MAP Index Parameter | 1 |
|  | HFBCH Index Parameter | 1 |
|  | Reserved | TBD |
|  | Non-user specific AMAP extension flag = 1 | 1 |
| Extended NUS A-MAP part | If (Non-user specific AMAP extension flag == 1) { |  |
|  | Type = 0 | 1 |
|  | Resource allocation information | TBD |
|  | Reserved | TBD |
|  | } |  |

- In case that only type ‘1’ is transmitted, an extended NUS part should follow right after the NUS A-MAP part.

| Part | Syntax | Size |
| --- | --- | --- |
| NUS A-MAP part | Assignment A-MAP size | TBD |
|  | HF-A-MAP Index Parameter | 1 |
|  | HFBCH Index Parameter | 1 |
|  | Reserved | TBD |
|  | Non-user specific AMAP extension flag = 1 | 1 |
| Extended NUS A-MAP part | If (Non-user specific AMAP extension flag == 1) { |  |
|  | Type = 1 | 1 |
|  | Configuration Change Counts for SCD | 5 |
|  | Configuration Change Counts for NBR-ADV | 5 |
|  | Extension flag = 0 | 1 |
|  | } |  |

- In case that both type ‘0’ and ‘1’ are transmitted, type ‘1’ of the extended NUS part should follow right after the NUS A-MAP part and type ‘0’ of the extended NUS part should follow right after type ‘1’ of the extended NUS part.

| Part | Syntax | Size |
| --- | --- | --- |
| NUS A-MAP part | Assignment A-MAP size | TBD |
|  | HF-A-MAP Index Parameter | 1 |
|  | HFBCH Index Parameter | 1 |
|  | Reserved | TBD |
|  | Non-user specific AMAP extension flag = 1 | 1 |
| Extended NUS A-MAP part 1 | If (Non-user specific AMAP extension flag == 1) { |  |
|  | Type = 1 | 1 |
|  | Configuration Change Counts for SCD | 5 |
|  | Configuration Change Counts for NBR-ADV | 5 |
|  | Extension flag = 1 | 1 |
|  | } |  |
| Extended NUS A-MAP part 2 | If (Extension flag == 1) { |  |
|  | Type = 0 | 1 |
|  | Resource allocation information | TBD |
|  | Reserved | TBD |
|  | } |  |

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[7] TWG_Motorola_04r03_Idle Mode, “RPD for Idle Mode clarification”

- Text proposal for the 802.16m DRAFT amendment

Remedy 1

On the IEEE P802.16m/D1, page 323, line 59, make the following changes to subsection 15.3.6.5.4.1:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.4.1 Non-user-specific A-MAP IE

ABS shall transmit at least one extended non-user specific part inside a paging listening interval in order to notify AMSs of version information for AAI_SCD and AAI_NBR-ADV message.

Table xxx — Extended non-user specific part format

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Type | 1 | If type is 0b0, it indicates that this extended non-user specific part includes resource allocation information for additional broadcast message. If type is 0b1, it indicates that this extended non-user specific part includes Configuration Change Count for AAI_SCD, Configuration Change Count for AAI_NBR-ADV and extension flag. |
| If ( Type == 0b0) { |  |  |
| Resource allocation information | TBD |  |
| Reserved | TBD |  |
| } |  |  |
| Else if (Type == 0b1) { |  |  |
| Configuration Change Count for AAI_SCD | 5 | This Configuration Change Count is incremented by one (modulo 32) by the ABS whenever any of the values of the AAI_SCD message change. |
| Configuration Change Count for AAI_NBR-ADV | 5 | This Configuration Change Count is incremented by one (modulo 32) by the ABS whenever any of the values of the AAI_NBR-ADV message change. |
| Extension flag | 1 | If extension flag is set, it indicates that an extended non-user specific part is present following this part. |
| } |  |  |

If only one of type 0 and 1 is sent at a non-user specific A-MAP region, ABS shall transmit information corresponding to the type through the extended non-user specific part extended by non-user specific A-MAP extension flag.

If both type 0 and type 1 are sent at the same non-user specific A-MAP region, ABS shall transmit information corresponding to the type 1 through one extended non-user specific part extended by non-user specific A-MAP extension flag. Also, ABS shall transmit information corresponding to the type 0 through another extended non-user specific part extended by extension flag in type 1 of the extended non-user specific part.

The resource allocation for Broadcast messages (e.g., PGID Info, AAI-TRF-IND, AAI-PAG-ADV, and other broadcast) is based on A-MAP_IE or non-user specific A-MAP extension.

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 2

On the IEEE P802.16m/D1, page 57, line 52, make the following changes to subsection 15.2.6.10:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.6.10 AAI_NBR-ADV

AMS shall compare the last received Configuration Change Count with the last stored Configuration Change Count whenever it receives type ‘1’ of the extended non-user specific part. If two counts have different values each other, AMS may try to receive the latest AAI_NBR-ADV message.

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

-----------------------------------------------------------End of the Text---------------------------------------------------------