| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reliability for Femto ABS(15.4) |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI Whai-En Chen NIU Chiu Wen Chen Institute for Information Industy(III) | E-mail: sjjung@etri.re.kr E-mail:wechen@niu.edu.tw E-mail:cwchen@nmi.iii.org.tw |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | Comments on Femto ABS reliability |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reliability for Femto ABS(15.4)

Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

Whai-En Chen

NIU

Chiu Wen Chen

III

# 1. Instruction

According to the current SDD[1], when a Femto ABS needs to disable air interface, it should send out an indication, which contains available out-of-service information such as out-of-service reasons and expected downtime and/or expected uptime, to prevent AMS entry or reentry from other cells. Upon reception of the indication, the subordinated AMSs should perform handover to neighboring cells. In this contribution, we provide the detailed description of the Femto ABS operation in case of out-of-service and propose to indicate   the Femto ABS’s disability to neighbor ABSs with the AMS’s handover. Upon reception of the indication, the neighbor ABSs can update the neighbor list and prevent the subordinate AMSs from scanning the disabled Femto ABS and performing handover to it.

# 2. Femto ABS reliability

When a Femto ABS needs to disable air interface, it should send out an indication, which contains available out-of-service information such as out-of-service reasons and expected downtime and/or expected uptime, to prevent AMS entry or reentry from other cells. Upon reception of the indication, the subordinated AMSs should perform handover to neighboring cells.

When a Femto ABS needs to disable air interface due to reset, power down and other reasons, it should broadcast AAI_DISABLE-IND message repeatedly until shutdown and set cell bar bit to 1 in order to prevent AMS entry or reentry from other cells. The AAI_DISBALE-IND message may contain the following information,

- Out-of-service reason

- Expected down time

- Expected uptime

- Recommended BSID

- HO_ coordination_ flag

If the HO_ coordination_ flag is set to 1, the subordinated AMSs may wait the unicast AAI_HO-CMD from the serving Femto ABS.

Otherwise, detailed procedure is as follows. Upon reception of the AAI_DISABLE-IND, all the subordinated AMSs in the Femto ABS shall immediately perform handover to neighboring cell. The Femto ABS may transfer user information of all the subordinated AMS to specific neighbor BS(s) for optimized network reentry of the AMS(s).

In this case, the AMS may consider recommended BS as its preferred target ABS. And the AMS may select other ABS, which is not included in AAI_DISABLE_IND message, as its preferred target ABS.

When an AMS is performing HO network reentry procedure due to previously serving Femto ABS’s disability, the AMS sends an AAI_RNG-REQ message including disability indication and related information to the target ABS. Upon reception of the AAI_RNG-REQ message, the target ABS may get information such as the AMS’s previously serving Femto ABS’s status (i.e. out-of-service) and its reason. With the status, the target ABS deletes the disabled Femto ABS from the neighbor list in order to prevent AMSs from scanning the disabled ABS. In addition, with received ‘out-of-service reason’ in disability indication and ‘Reference Entity ID’ in disability information, the target ABS can determine the subsequent procedure after ranging for AMS network reentry. That is, if ‘out-of-service reason’ is set to ‘Femto ABS shutdown’ and ‘Reference Entity ID’ is set to the same value as the target ABS’ BSID, the target ABS may search AMS’s information, which has been transferred to the target ABS through the backbone network for optimized network reentry during HO. If the Reference Entity ID is not the same as itself, it means that AMS’s information has been transferred to the other entity. Therefore, the target ABS can obtain AMS’s information from the reference entity through backbone network. Figure 1 shows an example of the Femto ABS shutdown and the subordinate AMS’s handover procedure. But, in case that the serving ABS is disabled due to the backhaul link down, the AMS should follow the normal network reentry during HO and target ABS can’t obtain AMS’s information to perform optimized network reentry. Figure 2 shows an example of the Femto ABS backhaul link down and subordinate AMS’s network reentry procedure

Figure 1. Example flow of Femto ABS shutdown and subordinate AMS’s network reentry

Figure 2. Example flow of the Femto ABS backhaul link down and subordinate AMS’s network reentry procedure

Out proposal has an advantage that it helps to speed up the target ABS’s neighbor list update and handover network reentry.

# 3. Text Proposal for the 802.16m AWD

------------------------------------------------------------Start of the Text--------------------------------------------------------

[add the following rows to Table 682]

| Name | Value | Usage |
| --- | --- | --- |
| disability indication | Out-of-service reason 0b00:shut-down 0b01:backhaul link down 0b10~0b11:other reason | It shall be included when the AMS is attempting HO network reentry due to Femto ABS’s out-of-service |
| disability information | Recommended BSID ABS which preserves AMS’s information. It may be the same as recommend BSID in out-of-service indication message. | It should be included when the AMS is attempting HO network reentry due to Femto ABS’s out-of-service and disability indication is 0b00 or 0b01. |

[add the following new section in 15.4 ]

15.4.x Femto ABS reliability

When Femto ABS is going to disable the air interface by instruction or by accident, it shall broadcast an indication of the air interface down through AAI_SON-ADV with Action type 4 for Reason bit 0, specified in section 15.2.6.x, repeatedly until it disables air interface and set cell bar bit to 1 in order to prevent MS entry or reentry from other cells. If handover is to be performed, the indicator may also inform whether the handover will be coordinated or not for AMS to decide what handover procedure it will perform (e.g. uncontrolled handover, coordinated handover etc). The AMS may initiate HO to a BS based on the recommended list given by AAI_SON-ADV message or any previously cached neighbor BS list of its preference.

In this case the Femto ABS shall store MAC context information of the serving AMSs (e.g. Basic capabilities, Security capabilities, etc.). Such context information allows AMS to perform optimized network reentry when returning back to the femtocell BS upon its recovery.

When an AMS is performing HO network reentry for uncoordinated HO due to previously serving Femto ABS’s disability, the AMS may inform target ABS of the previously serving ABS’s status using disability indication in AAI_RNG-REQ message. Upon reception of the AAI_RNG-REQ message, the target ABS may delete the disabled Femto ABS from the neighbor list based on “serving BSID” and “disability indication” in AAI_RNG-REQ message in order to prevent AMSs from scanning the disabled ABS. In addition, the AMS may inform target ABS of the ‘recommended BSID’ in disability information such that the target ABS may retrieve AMS’s information from recommended ABS or network entity if it doesn’t have AMS information.

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE C802.16m-09/0034,” IEEE 802.16m System Description Document(SDD),”July 2009

- IEEE C802.16m-09/1428, “Proposed IEEE 802.16mAmendment Text on Femtocell BS Reliability,” July 2009