| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation in Femto ABS (15.4.11) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, HanGyu Cho, Jin Sam Kwak LG Electronics, Inc. Luciano Sarperi, Yanling Lu Fujitsu Zhaohua Lu，Feng Xie，Yang Liu， Kaiying Lv，Ying Liu，Hongyun Qu，Mary Chion ZTE Corporation | +82-31-450-2067 [jins978, cooper, ksryu, ronnykim, hgcho, samji] @lge.com luciano.sarperi@uk.fujitsu.com yanling.lu@cn.fujitsu.com {lu.zhaohua, xie.feng2, liu.yang8, lv.kaiying, liu.ying, qu.hongyun}@zte.com.cn mchion@zteusa.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m Session #63 in Jeju island, Korea |  |
| Abstract | This contribution is to propose detail description on resource reservation/release for IM |  |
| Purpose | Update P802.16m/D1 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation in Femto ABS (15.4.11)

Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, HanGyu Cho, Jin Sam Kwak

LG Electronics, Inc.

Luciano Sarperi, Yanling Lu

Fujitsu

Zhaohua Lu，Feng Xie，Yang Liu，Kaiying Lv，Ying Liu，Hongyun Qu, Mary Chion

ZTE Corporation

- Introduction

- The current draft defines semi-static resource reservation for interference mitigation. However, scanning trigger condition to identify interfering Femto ABS is not considered. Also, the mechanism for resource release is not defined. Therefore, this contribution proposes the aforementioned detail descriptions for semi-static resource blocking for interference mitigation in Femto ABS.

- Rationale of modifications

- Scanning trigger condition in Femto ABS can be different with one in macro ABS. For example, when the AMS connected to overlay macro ABS moves around inaccessible CSG Femto ABS, AMS should be triggered scanning for IM even if received RSSI from serving ABS is greater than the defined absolute value for HO. In this case, the interference mitigation scanning RSSI threshold value may not be lower than the handover scanning RSSI threshold value.

- Text (line 32~43, page 493)in regard to scanning report from inaccessible Femto ABS is shortened as replaced with suggested texts due to its duplicity

- Reserved resource blocks for IM shall be released when the AMS moves out of the inaccessible Femto ABS area (e.g., RSSI from Femto ABS of which resource block is reserved is falling below a certain threshold value). By scanning the Femto ABS periodically, AMS can notify serving macro ABS of its relived interference. In order to enable the aforementioned resource release, description needs to be added.

  - Trigger condition for scan report is created to capture the above mechanism (refer to Remedy2)

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

Remedy 1 :

[Modify texts as suggested below: Page 493, Line 32 to 43]

15.4.11 Interference Avoidance and Interference Mitigation

An MS may be requested by its serving macro BS or Femto ABS to report the signal strength measurement of neighbor BSs, including macro and/or Femto ABSs. The reported information can be used by the serving ABS to coordinate with its neighbor BSs to mitigate the interference at the AMSs.

Large interference from an inaccessible Femto ABS may trigger a nearby AMS to report the interference to the serving BS, and the report information should include system information of the inaccessible Femto ABS (e.g., BS_ID of the Femto ABS).

A separate scanning threshold value from handover shall be used to trigger AMS to scan neighbor ABSs in subject to reduce interference.

The AMS may report the scanning result which contains BS_ID(s) of the scanned interfering Femto ABS(s) to the serving macro ABS to mitigate interference at the AMS. Upon receiving its scanning result, the serving macro ABS should identify the interfering Femto ABS(s) and request those Femto ABS(s) to mitigate the interference by reducing its transmission power, and/or blocking the interference related resource region.

The serving BS and/or the network can request the interfering Femtocell BS to mitigate the interference by reducing transmission power, and/or blocking some resource region.

The macro ABS may send the interfering BS_ID(s) for which the resource reservation is coordinated via AAI_SCN-RSP to the AMS. If the AMS receives the interfering BS_ID(s), the AMS may scan the signals from the Femto ABS(s) periodically and report its scanning result to the serving macro ABS via AAI_SCN-REP when the trigger condition shown in table xx is met. Upon receiving the scanning report, the serving macro ABS should request the Femto ABS to terminate the interference mitigation.

Remedy 2 :

[Create a new section 15.4.11.1 for trigger condition and Add texts as suggested below: Page 494, Line 39]

15.4.11.1 Trigger condition definitions

The trigger TLV in table yy is encoded using the description in Table xx.

Table xx – Trigger TLV Description

| Name | Type | Length (Bytes) | Value |
| --- | --- | --- | --- |
| Type/Function/Action |  | 1 | See Table yy-for description |
| Trigger Value |  | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition |

The Type/function/action byte field of the trigger description in Table xx - is described in Table yy

Table yy – Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 1 | Trigger metric type : 0x0: Reserved 0x1: RSSI |
| Function | 1 | 0x0: Reserved 0x1: Metric of neighbor ABS is less than absolute value |
| Action | 1 | 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP to release reserved resource blocks terminate interference mitigation. *Option 0x1 is only applicable after the interference mitigation in Femto ABS is activated. |

---------------------------------------------------------------Text End -----------------------------------------------------------