| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to MIMO midamble (15.3.5.4.2) |  |
| Date Submitted | 2009-08-31 |  |
| Source(s) | Seunghyun Kang, Jinyoung Chun, Bin-Chul Ihm and Wookbong Lee LG Electronics | sh_kang@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 P802.16m/D1 : MIMO midamble (15.3.5.4.2) |  |
| Abstract | This contribution provides the proposed text for MIMO Midamble. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to MIMO midamble (15.3.5.4.2)

Seunghyun Kang, Jinyoung Chun, Bin-Chul Ihm and Wookbong Lee

LG Electronics

- Introduction

In session #62, the details of MIMO midamble such as MIMO midamble sequence, subcarrier mapping rule and MIMO midamble symbol location in the DL frame has been proposed and adopted in [1]. In this contribution, we provide analysis of the current MIMO midamble and propose modified MIMO midamble design.

- Analysis of the current AWD MIMO midamble

According to the equation (197) in [1], the current MIMO midamble subcarrier mapping rule has its periodicity depending on a number of ABS transmit antennas Nt. Furthermore, the midamble subcarriers are shifted by considering frequency reuse of 3 in order to avoid midamble collision between neighboring cells. Therefore, the period of the midamble subcarriers is 3×Nt and a midamble subcarrier for each antenna is regularly mapped every 3×Nt subcarriers.

When there exist ABSs having different cell deployment or exist both Femtocell BS and Macro BS simultaneously, the neighboring cells may be assumed to have different number of transmit antennas. In these cases, there is no way to avoid midamble collision even though the current MIMO midamble subcarrier mapping rule has considered frequency reuse of 3. Figure 1 shows an example of midamble collision between two cells assuming two transmit antennas and four transmit antennas for each. In the figure, there exist midamble collisions in the subcarrier index 6 and 7. Moreover, there exist midamble collisions every 12 subcarriers because of their periodicity.

Figure 1. Midamble collision between neighboring cells

In the figure 2, the average Mean Square Error (MSE) of channel estimation are derived for MIMO midamble employing 4 cases of transmit antenna configurations. The transmit antenna configurations are as follows.

Transmit antenna configurations:

Case-1: # transmit antennas of Serving cell = 2 & # transmit antennas of two neighboring cells = 2

Case-2: # transmit antennas of Serving cell = 2 & # transmit antennas of two neighboring cells = 4

Case-3: # transmit antennas of Serving cell = 4 & # transmit antennas of two neighboring cells = 4

Case-4: # transmit antennas of Serving cell = 4 & # transmit antennas of two neighboring cells = 2

Also, the average MSE has been performed by considering SIR of 0, 3 and 6 dB where SIR is a term for midamble power ratio between a serving cell midamble power and a sum of two neighboring cells midamble powers which are interference to the serving cell. In the figure, the average MSE of Case-1 and Case-3 configurations is getting smaller while increasing SNR. Also, there is no impact of the interference from the neighboring cells since there is no midamble collision. However, for Case-2 and Case-4 configurations, the average MSE are converging to some values depending on SIR. Also, for all the SNR ranges, their average MSE is always larger than that of Case-1 and Case-3 configurations due to the midamble collision.

Figure 2. Average MSE performance of the current AWD midamble design (SIR=0, 3 and 6[dB])

- Modified MIMO midamble

The subcarrier mapping rule of the current MIMO midamble design causes midamble collisions when the neighboring cells are assumed to have different number of transmit antennas. In order to avoid midamble collision with minimal change, we have modified the current MIMO midamble equation as follows.

bk=24⋅100.2Nt⋅1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod 24=g+k−sN1∗NscmodNt+8∗(BSIDmod3)

First of all, the period of the subcarrier mapping rule is fixed to the period of the eight transmit antennas regardless a number of transmit antennas. By using this modification on the subcarrier mapping rule, a midamble subcarrier for each antenna is regularly mapped every 24 subcarriers. Also, the shift values for frequency reuse of 3 is fixed to the period of the eight transmit antennas. As a result of this modification, the number of midamble subcarriers is proportionally reduced. In order to compensate the midamble subcarrier density, we have increased the midamble signal power depending on Nt. Since the period of the subcarrier mapping rule and the shift values for frequency reuse have been fixed, there is no midamble collision between neighboring cells having a different number of transmit antennas.

In the figure 3, the average MSE performance of the modified midamble design has been evaluated and compared with that of the current AWD midamble design. The modified midamble design shows almost same performance for the Case-1 and Case-3 configurations and shows good enhancement for the Case-2 and Case-4 configurations.

Figure 3. Average MSE comparison (SIR= 0[dB])

- References

[1] IEEE P802.16m/D1, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems”

[2] IEEE 80216m-09/0037, “Call for Contributions on Project 802.16m Amendment Content”

[3] IEEE C802.16m-09/1313r3, “MIMO midamble sequence proposal”

[4] IEEE C802.16m-09/1700, “Proposed Changes to MIMO Midamble (15.3.5.4.2) for the IEEE 802.16m/D1”

- Text proposal for inclusion in the 802.16m AWD

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing AWD text is shown in black.

-----------------------------------------------------------------   Text Start  -----------------------------------------------------------------

<Modify subsection 15.3.5.4.2 as follows :>

15.3.5.4.2 MIMO midamble

MIMO midamble is used for PMI selection in closed loop MIMO. For OL MIMO, midamble can be used to calculate CQI. MIMO midamble shall be transmitted every frame on the second last DL subframe. The midamble signal occupies the first OFDMA symbol in a DL type-1 or type-2 sub-frame. For the type-1 6 symbol DL subframe case, the remaining 5 consecutive symbols form a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols form a type-1 subframe.

The MIMO midamble signal transmitted by the ABS antenna is defined by Equation (196):

k≠Nused−12k=Nused−1				(196)

where bk is a complex coefficients modulating subcarriers in the midamble symbol defined from Equation (197)

bk=1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod(3∗Nt)=g+k−sN1∗NscmodNt+Nt∗(BSIDmod3)

bk=24⋅100.2Nt⋅1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod 24=g+k−sN1∗NscmodNt+8∗(BSIDmod3)

where k is the subcarrier index (0 ≤k ≤ Nused -1), Nused is the number of used subcarriers, Nt is a number of transmit antennas, G(x) is the Golay sequence defined in Table 658 (0 ≤ x ≤ 2047), fft is the FFT size used, u is a shift value (0 ≤ u ≤ 127), where the actual value of u is derived from u = mod(BSID, 128), offsetD(fft) is an FFT size specific offset as defined in Table 659, g is a ABS transmit antenna index, Nt is a number of ABS transmit antennas, parameter s = 0, for k≤(Nused - 1)/2 and s = 1, for k>(Nused - 1)/2.

Example of physical structure of MIMO midamble is shown in Figure 447 423 for 4TX antenna and BSID = 0.

Figure 423. MIMO midamble physical structure for 4TX antennas and BSID = 0

MIMO midamble shall be transmitted with boosting of 2dB.

| Table 658 – Golay sequence of length 2048 bits |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |  |  |  |  |

| Table 659 – Offsets in the Golay sequence |  |
| --- | --- |
| FFT size | Offset |
| 2048 | 30 |
| 1024 | 60 |
| 512 | 40 |

-------------------------------------------------------------------   Text End  -------------------------------------------------------------------