| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Femtocell BS detection (15.4.7.1 Femto ABS detection) for the P802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Jinyoung Chun, Inuk Jung, Kiseon Ryu, Bin-Chul Ihm LG Electronics | jychun03@lge.com |
| Re: | P802.16m/D1 : Femto (15.4 Support for Femto ABS) |  |
| Abstract | This contribution proposes the method of Femtocell BS detection. |  |
| Purpose | For member’s review and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for Femtocell ABS detection for P802.16m/D1

Jinyoung Chun, Inuk Jung, Bin-Chul Ihm, and Kiseon Ryu

LG Electronics

- Introduction

There is no detail method about Femtocell ABS detection. Therefore we clarify the procedure and define control signals for Femtocell operation.

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

[2] C802.16m_09/1844, “Femto ABS detection,” August 2009

- Text proposal for inclusion in the 802.16m draft

[Add the texts and figures marked in blue and remove the texts and figures marked in red in the P802.16m/D1.]

[Insert the below text to ‘15.4.7.1 Femto ABS detection, identification and selection’ in line 45, page 492. It is based on SDD.]

--------------------------------------  Text Start  --------------------------------------------

15.4.7.1 Femto ABS detection, identification and selection

The AMS detects the Femto ABS using the PHY identifier (see <<section x>>).

The AMS identifies the Femto ABS using the MAC identifier (ex. BSID, CSGID).

The AMS shall use scanning procedures specified in 15.3.9.2. When a CSG white list is maintained at an AMS, it should select the Femto ABSs according to the white list.

The AMS selects the target Femto ABS for network entry based on the subscription types of the Femto ABS and the membership to the relevant subscribers groups or accessibility to the service providers network (ex: Cell bar).

A Femtocell BS monitors periodic UL control signals (fast feedback channel, Feedback header, sounding channel, etc) with associated with an MS which is served by overlay macro BS. The monitoring is initiated by overlay macro BS and informed to Femtocell BS to detect the existence of the MS in its coverage. Then the Femtocell BS informs the macro BS over the backhaul that the MS is in its coverage and subsequently handover to Femtocell BS can be accomplished.

The overall monitoring procedure is as below.

- Macro ABS commands AMS to report the distance information (UL Tx Power report) if the trigger condition in Table xx is met.

- AMS transmits BR and UL Tx Power Report header, and Macro ABS makes monitoring lists of AMSs belonged to each CSG Femto ABSs based on the information.

- Macro ABS informs each CSG Femto ABSs about the allocation information of AMS’s periodic signals (e.g. Fast feedback channel, Feedback header and Sounding channel) and the distance information over backhaul.

- Femto ABS monitors the AMSs and informs Macro ABS about AMSs to handover over backhaul.

The trigger TLV is encoded using the description in Table xx.

Table xx – Trigger TLV Description

| Name | Type | Length (Bytes) | Value |
| --- | --- | --- | --- |
| Type/Function/Action |  | 1 | See Table yy-for description |
| Trigger Value |  | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition |

The Type/function/action byte field of the trigger description in Table xx is described in Table yy

Table yy – Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 1 | Trigger metric type : 0x0: Reserved 0x1: Tx power |
| Function | 1 | 0x0: Metric of serving ABS is more than absolute value 0x1: Metric of serving ABS is less than absolute value |
| Action | 1 | 0x0: Reserved 0x1: Respond on trigger with the Tx power report header to initiate monitoring of AMS by Femto ABS. |

15.3.6.5.4.13 Feedback Polling A-MAP IE2

Macro BS commands AMS to report the Tx power report header with long term period.

Table xxx. Feedback Polling A-MAP IE2

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | Feedback Polling IE2 |
| MCS | 2 | MCS level for transmitting feedback header |
| Resource Allocation | TBD | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | 2 | Transmit feedback header every 2p superframe |
| TRU | 3 | Target RU indicating which RUs or which type of RU to work on for feedback |
| Feedback Type | 2 | 0b00: Tx power report 0b01 – 0b11: Reserved |
| Padding | Variable |  |
| MCRC | 16 | 16 bit CRC masked by STID |

15.2.4.1.4. BR and UL Tx Power Report Header

BR Header with UL Tx Power Report is sent to report UL transmission power of the AMS. UL Tx power level in dBm for the burst that carries this header.

Table 20. BR and UL Tx Power Report Header field

| Name | Length (bit) | Description |
| --- | --- | --- |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header. The value is 0b0011. |
| Type | 4 | MAC signaling header type. |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: incremental 1: aggregate |
| BR Size | 11 | Burst size index of UL bandwidth requested by the AMS. The BR is for the FID. The request shall be independent of the physical layer modulation and coding. |
| UL Tx Power | 8 | UL Tx power level in dBm for the burst that carries this header. The value shall be estimated and reported for the burst. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| STID | 12 | STID of which AMS sends this header. |
| Reserved | 4 | Reserved. This field shall be filled by 0. |

-----------------------------------------  Text End  ------------------------------------------