| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Femtocell BS detection (15.4.8.1 HO from Macro ABS to Femto ABS) for the P802.16m/D1 |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Jinyoung Chun, Inuk Jung, Kiseon Ryu, Bin-Chul Ihm LG Electronics Kelvin Chou MediaTek | jychun03@lge.com kelvin.chou@mediatek.com |
| Re: | P802.16m/D1 : Femto (15.4 Support for Femto ABS) |  |
| Abstract | This contribution proposes the method of Femtocell BS detection. |  |
| Purpose | For member’s review and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for Femtocell ABS detection for P802.16m/D1

Jinyoung Chun, Inuk Jung, Bin-Chul Ihm, and Kiseon Ryu

LG Electronics

- Introduction

There is no detail method about Femtocell ABS detection. Therefore we clarify the procedure and define control signals for Femtocell operation.

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

[2] C802.16m_09/1844, “Femto ABS detection,” August 2009

- Text proposal for inclusion in the 802.16m draft

[Add the texts and figures marked in blue and remove the texts and figures marked in red in the P802.16m/D1.]

[Insert the below text in line 5, page 493.]

--------------------------------------  Text Start  --------------------------------------------

15.4.8.1 HO from Macro ABS to Femto ABS

The Femto ABS monitors the UL signal from the AMSs served by macro ABS from the pre-defined Tx power and radio resource region. The radio resource region is negotiated between Femto ABS and macro ABS where the macro ABS commands some of its UL transmission over the pre-defined region. Femto ABS can report the measurement results for each pre-defined zone to the macro ABS for identifying the handover opportunities from macrocell to Femtocell over the backhaul.

-----------------------------------------  Text End  ------------------------------------------