| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on PKMv3 state machines |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Gene Beck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronics Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | E-mail: gbhahn@lge.com Phone : +82-31-450-7188 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: |  |  |
| Abstract | This contribution proposes the PKMv3 authentication/TEK state machines. |  |
| Purpose | Accept the proposed specification changes on IEEE 802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on PKMv3 state machines

GeneBeck Hahn, Kiseon Ryu, Ronny Yongho Kim

LG Electronics

# Introduction

In IEEE 802.16m/D1 [1], the authentication and TEK state machines are missing. This contribution proposes the PKMv3 authentication and TEK state machines.

# References

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems(Advanced Air Interface)”, July. 2009.

[2] IEEE Std 802.16-2009, “Part 16: Air Interface for Broadband Wireless Access Systems”, May. 2009

# Proposed Text

Add the proposed text to the section 15.2.5.3.x as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

- 2.5. 3.x Authentication state machine

As in legacy system, the PKMv3 authentication state machine for single EAP authentication consists of six states and sixteen events (including receipt of messages and events from other FSMs) that may trigger state transitions. However, some states, messages and events are modified as newly defined in [1]. The authentication state machine is presented in both a state flow diagram (Figure 15.2.5.3.x.1) and a state transition matrix (Table 15.2.5.3.x.1). The state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

In PKMv3 [1], the authentication process has 2 phases: EAP phase and key agreement phase. The Auth_FSM is responsible for all PKM phase but the actual EAP exchange and communicates with other FSMs in the system using events.

Through operation of an Authentication state machine, the AMS attempts to get authenticated with the NW, maintain this authentication and support Authentication context switching for HO and Idle situations. The state machine takes care of getting authenticated with the NW, ensuring re-authentication will occur before authentication expires and supports key derivations according to support optimized re-entry for HO and idle.

The optimized re-entry support is done in a special state in which the NW connection is suspended and therefore re-authentication can’t occur, the triggers for re-authentication continue to work in this state but the initiation is done only after returning to an authenticated state.

Figure 15.2.5.3.x.1 Authentication State Machine for PKMv3 single EAP

Table 15.2.5.3.x.1. Authentication FSM state transition matrix for PKMv3

| State Event or receive message | (A) Stopped | (B) Not Authenticated | (C) Key Agreement MSG #3 Wait | (D) Authenticated | (E) Reauth Key Agreement MSG #3 Wait | (F) Reentry Auth Wait |
| --- | --- | --- | --- | --- | --- | --- |
| (1) Start Auth | Not Authenticated |  |  |  |  | Not Authenticated |
| (2) PKMv3 Key Agreement MSG #1 |  | Key Agreement MSG #3 Wait | Key Agreement MSG #3 Wait | Reauth Key Agreement MSG #3 Wait | Reauth Key Agreement MSG #3 Wait |  |
| (3) PKMv3 Key Agreement MSG #3 |  |  | Authenticated |  | Authenticated |  |
| (4) EAP Success |  | Not Authenticated |  | Authenticated |  |  |
| (5) Key Agreement Timeout |  |  | Key Agreement MSG #3 Wait |  | Reauth Key Agreement MSG #3 Wait |  |
| (6) Key Agreement MSG #2 max resend elapsed |  |  | Stopped |  | Stopped |  |
| (7) ReAuth needed |  |  |  | Authenticated |  |  |
| (8) Start Reentry |  |  |  | Reentry Auth Wait | Reentry Auth Wait |  |
| (9) EAPStart timeout |  |  |  | Authenticated |  |  |
| (10) HO cancelled |  |  |  |  |  | Authenticated |
| (11) TBS change |  |  |  |  |  | Reentry Auth Wait |
| (12) Reentry Completed |  |  |  |  |  | Authenticated |
| (13) Auth Expired |  |  |  | Stopped | Stopped | Stopped |
| (14) EAP Fail |  | Stopped |  | Stopped |  |  |
| (15) External Stop |  | Stopped | Stopped | Stopped | Stopped | Stopped |

15.2.5.3.x.1 States

Compared to the legacy system, the following states are newly defined.

- Key Agreement MSG #3 Wait (SA-TEK-Response Wait in legacy system) : The Authentication FSM has sent a PKMv3 Key Agreement MSG #2 and waits for a PKMv3 Key Agreement MSG #3 in this state.

- Reauth Key Agreement MSG #3 Wait (Reauth SA-TEK-Response Wait in legacy system) : The Authorization FSM has sent a PKMv3 Key Agreement MSG #2 for re-authentication and waits for a PKMv3 Key Agreement MSG#3.

15.2.5.3.x.2 Messages

Compared to the legacy system, the following messages are newly defined.

- PKMv3 Key Agreement MSG #1 (PKMv2 SA-TEK-Challenge in legacy system) : The first message of key agreement. It is sent from ABS to AMS after EAP-based authentication has finished.

- PKMv3 Key Agreement MSG #2 (PKMv2 SA-TEK-Request in legacy system) : The second message of key agreement. It is sent from AMS to ABS as a response to a PKMv3 key agreement MSG#1.

- PKMv3 Key Agreement MSG #3 (PKMv2 SA-TEK-Response in legacy system) : The last message of key agreement. It is sent from ABS to AMS as a response to a PKMv3 key agreement MSG#2.

15.2.5.3.x.3 Events

Compared to the legacy system, the following events are newly defined.

- Key Agreement Timeout (SATEK Timeout in legacy system) : This event is generated when the AMS does not receive PKMv3 Key Agreement MSG #3 from ABS within Key Agreement Timer after transmitting PKMv3 Key Agreement MSG #2.

- Key Agreement MSG #2 max resends elapsed (SATEK request max resends elapsed). The Authentication state machine generates this event when AMS has transmitted PKMv3 Key Agreement MSG #2 up to KeyAgreementMSG#2MaxResends times and Key Agreement Timer expires.

- 2.5. 3.y TEK state machine

Contrary to the TEK state machine of legacy system, the TEK state machine of 16m consists of six states and nine events (including receipt of messages) that may trigger state transitions. Like the Authorization state machine, the TEK state machine is presented in both a state flow diagram (Figure 15.2.5.3.y.1) and a state transition matrix (Table 15.2.5.3.y.1). However, as defined in [1], all M(ulticast) & B(roadcast) related states, messages and events are not specified in 16m TEK state machine. As was the case for the Authorization state machine, the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

Shaded states in Figure 2 (Operational, Rekey Wait, Rekey Reauthorize Wait) have valid keying material and encrypted traffic may be sent.

The Authentication state machine starts an independent TEK state machine for each of its authorized SAIDs.

Through operation of a TEK state machine, the AMS attempts to keep its copies of an SAID’s TEKs synchronized with those of its ABS. A TEK state periodically refresh copies of its SAID’s keying material soon after the schedules expiration time of the older of its two TEKs and before the expiration of its newer TEK.

15.2.5.3.y.1 States

Compared to the legacy system, no states are newly defined.

15.2.5.3.y.2 Messages

Compared to the legacy system, the following messages are no longer defined in 16m TEK state machine due to the 16m feature of 16m local TEK derivation.

- Key Request

- Key Reply

15.2.5.3.y.3 Events

Compared to the legacy system, the following events are newly defined.

- Timeout & TEK Local Update Initiation : A retry timer timeout. Generally, the local TEK Update is initiated.

- TEK Refresh Timeout & TEK Local Update Initiation : The TEK refresh timer times out. This timer event initiates the local TEK update.

.

Figure 15.2.5.3.y.1 TEK State Machine for PKMv3 single EAP

Table 15.2.5.3.y.1. TEK FSM state transition matrix for PKMv3

| State Event or Rcvd Message | (A) Start | (B) Op Wait | (C) Op Reauth Wait | (D) Op | (E) Rekey Wait | (F) Rekey Reauth Wait |
| --- | --- | --- | --- | --- | --- | --- |
| (1) Stop |  | Start | Start | Start | Start | Start |
| (2) Authorized | Op Wait |  |  |  |  |  |
| (3) Auth Pend |  | Op Reauth Wait |  |  | Rekey Reauth Wait |  |
| (4) Auth Comp |  |  | Op Wait |  |  | Rekey Wait |
| (5) TEK Invalid |  |  |  | Rekey Wait |  |  |
| (6) Timeout & TEK Local Update Init |  | Op Wait |  |  | Rekey Wait |  |
| (7) TEK Refresh Timeout & TEK Local Update Init |  |  |  | Rekey Wait |  |  |

--------------------------------------------------- End of Proposed Text------------------------------------------------------