| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Distributed Security Control of 16m Relay |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Gene Beck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronics Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | E-mail: gbhahn@lge.com Phone : +82-31-450-7188 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: |  |  |
| Abstract | This contribution proposes the Distributed Security Control of 16m Relay. |  |
| Purpose | Accept the proposed specification changes on IEEE 802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Distributed Security Control of 16m Relay

GeneBeck Hahn, Kiseon Ryu, Ronny Yongho Kim

LG Electronics

# Introduction

In this contribution, we propose texts for Security Control of 16m Relay. The proposed method of Relay Security Control basically inherits the feature of Distributed Security control of 16j and reflects the enhanced security feature defined in [1]. In particular, this contribution contains the following main technical proposals.

- Key agreement procedure that shall be deployed in 16m relay

- TEK update during RS to RS HO

- Selective confidentiality protection for control plane signaling

# References

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems(Advanced Air Interface)”, July. 2009.

[2] IEEE Std 802.16-2009, “Part 16: Air Interface for Broadband Wireless Access Systems”, May. 2009

# Text Proposal for Distributed Relay Security Control

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

- 5. Multi-hop Relay

- 5.1 Distributed Relay Security Control

When an access RS is operating is distributed security mode, the MSK established between MS and MR-BS is distributed to this RS.

An access RS relays initial PKM messages between the MR-BS and MS. When the MSK for MS is established, the MR-BS shall securely transfer the MSK of MS to its access RS. The access RS starts key agreement with the MS and drives all necessary keys if PKMv3 is used.

The access RS runs a secure encapsulation protocol with the MR-BS and MS. TEKs are different for relay and access links and locally derive either between MR-BS and access RS or between access RS and MS.

During authentication, the Security Association that contains the security pertaining to an MS will be created either between an MS and an access RS or between an access RS and MR-BS.

RS instantiates and sets the TEK Counter for each MS to zero for TEK derivation (access link) after it receives MSK for MS from MR-BS (whenever AK is setup). Whenever key agreement between MS and access RS is successfully completed, TEK is derived locally by MS and access RS respectively. TEK derivation follows the method defined in [1].

Figure 1 shows the key agreement for 16m relay.

Figure 1. Key Agreement Procedure of 16m Relay

- 5.2 MSK Transfer

Upon successful authorization of the MS, MR-BS shall send PKMv3 MSK Transfer message conveying MSK to the relevant access RS. Access RS shall send PKMv3 MSK Transfer ACK message to MR-BS in order to acknowledge successful reception of PKMv3 MSK Transfer message. The MSK in MSK Transfer message in AES encrypted using a 128 bit key derived for the access RS AK and a 128-bit block size.

- 5.3 TEK Update during HO

For RS-RS HO scenario, the MR-BS may process key pre-distribution to T-RS before MS handoff occurs. If target RS does not have MS’s active PMK, MR-BS may deliver an active PMK to the RS when receiving an entry/reentry request from MS. MS and target RS performs MSID*, AK derivation and locally update TEK based on the (newly derived) AK and TEK Count that is reset to 0. Figure 2 presents the procedure of TEK update during RS-RS HO.

Figure 2. TEK Update during HO in 16m Relay

When RS handover triggered, the RS may issue re-authentication to MSs that it serves to update the security material of the MSs.

- 5.4 SA Identification

Each MS shall effectively establish an exclusive primary SA with the RS, interactive with the RS as if it were a BS from the MS’s perspective. Each RS shall establish an exclusive primary SA with MR-BS. Table 1 shows the scheme for identifying the primary SAIDs

| Location | SA Identification |
| --- | --- |
| SS | SAID (SS Basic CID) |
| Access RS | SAID (Access RS Basic CID), SAID (SS Basic CID) |
| MR-BS | SAID (Access RS Basic CID) |

Table 1. SA identification at Relay System with Distributed Security Control

- 5.5 Integrity Protection for MAC Management Messages

After completing authentication and establishing a SA between MS and access RS, MAC management messages on the access link shall contain CMAC calculated by a key shared between MS and its access RS. Besides, the management messages on the relay link shall contain CMAC calculated by a key shared between RS and MR-BS. Old CMAC value shall be replaced with new CMAC value when access RS relays a management message.

If tunneling is used, the MT-CID in the relay MAC header assigned by MR-BS shall be used in the calculation, otherwise the FID in the MAC header shall be used.

Multiple MAC management messages from the connections belonging to the same management tunnel can be transported by the ingress station in a relay MAC PDU. Instead of carrying individual CMAC tuple, if present, for each individual management message, the ingress station may send an MT_Transfer message carrying multiple MAC management messages with only one CMAC tuple to authenticate the entire relay MAC PDU. The AC bit in the relay security extended header shall be set to 1 and FID in the relay MAC header shall belong to MT-FID.

- 5.5.1 Calculation of CMAC value for Relay

PKM protocol messages shall be authenticated using CMAC keys shared between MS/RS and MR-BS only. Intermediate RS shall not verity the integrity of PKM messages.

In the case when one CMAC value is shared by the concatenated MAC management messages in a relay MAC PDU, the CMAC_KEY_* used in the CMAC value calculation are the CMAC_KEY_* shared between the access RS and MR-BS. The MAC management messages used in the calculation of CMAC value covers the entire MT_Transfer message, i.e., the entire relay MAC PDU.

- 5.6 Confidentiality Protection for MAC Management Messages

After completing authentication and establishing a SA between MS and access RS, the management messages can be protected selectively based on the use of AES-CCM. The management messages on the access link shall be encrypted by TEK shared between MS and access RS. The management messages on relay link shall be encrypted using TEK shared between access RS and MR-BS.

The TEK for access link shall be replaced with TEK for relay link when access RS relays a management message. If tunneling is used and when an access RS is either an ingress or egress station of a tunnel, multiple MAC management messages from the connections belonging to the same management tunnel can be transported in a relay MAC PDU. The ingress station (access RS) may send MT_Transfer message carrying multiple MAC management messages by encrypting it with access RS’s TEK (EKS field identifies which RS’s TEK is used). Only a single set of PN and integrity check values are added to encrypted MT_Transfer message.

The AES-CCM encryption rules are applied to the tunnel data unit of relay Mac PDU, i.e., including only concatenated MAC PDUs encapsulated in the relay MAC PDU but not extended sub-headers and sub-headers. Extended sub-headers and sub-headers of a relay MAC PDU are not encrypted. A PN is placed after the sub-headers of the MT_Transfer message.

--------------------------------------------------- End of Proposed Text------------------------------------------------------